| VESTEL ELEKTRONİK A.Ş. |  |  | CHANGE BULLETIN (TCN) |  |  | CHANGE NO: 62 REF NO :- |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AFFECTED MODELS: 11AK19 TV MODELS WITHOUT TUNER APPLICATION |  |  |  |  |  |  |  |
| # START DATE : 15.09.1998 REASONS OF CHANGE: NOTE1: THIS RESISTOR IS BEEN CANCELED, THE CABLE (WHICH WAS CONNECTED TO PIN15 OF IC050) WILL BE CONNECTED FROM PIN19 AND PIN21 (GND) OF SCART SC051 TO PIN 17 AND GND PIN OF IC050 THE CABLE WILL BE FIXED AT THE SCART SIDE BY MEANS OF SILICON. NOTE2:THE JUMPER POSITION ON 11AK19-8A DOESN’T EXIST ON 11AK19-B CHASSIS. A JUMPER SHOULD BE INSERTED AT THE POSITION NR. R588 ON 11AK19-B CHASSIS. # OUT |  |  |  |  |  |  |  |
| NO | ## PCB CODE | ## VESTEL CODE |  | # COMPONENT NAME | ## POSITION NUMBER |  | ## QUANTITY |
| 1 | 11AK19-8A | 30000767 |  | RES CF 1/4W 68R | NOTE1 |  | 1 |
| 2 | 11AK19-8A | 30001734 |  | JUMPER SMD | S069 |  | 1 |
| 3 | 11AK19-8A | 30002584 |  | JUMPER WİRE | J127 |  | NOTE2 |

IN

| NO | PCB CODE | VESTEL CODE | COMPONENT NAME | POSITION NUMBER | QUANTITY |
| --- | --- | --- | --- | --- | --- |
| 2 | 11AK19-B | 30006879 | RES SMD 1/10W 62R | S069 | 1 |
| 3 | 11AK19-B | 30002584 | JUMPER WİRE | R588 | NOTE2 |