| VESTEL ELEKTRONİK A.Ş. |  |  | CHANGE BULLETIN (TCN) |  |  | # CHANGE NO: 74 REF NO :UMB-176 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AFFECTED MODELS: ALL MONO 90º MODELS WITH 11AK19E-3 CHASSIS. |  |  |  |  |  |  |  |
| START DATE : 15.03.2000 |  |  |  |  |  |  |  |
| RES CF 1/4W 1K J DIODE IN4007 ISOLATION COVER REASONS OF CHANGE: TO REMOVE THE “CLICK NOISE” THROUGH SPEAKERS WHEN THE TV SET IN ST-BY MODE. NOTE 1: THE DIODE AND THE RESISTOR MUST BE CONNECTED IN SERIES AS SHOWN BELOW. THE CATHODE OF THE DIODE MUST BE CONNECTED TO JUMPER WIRE R802(JUMPER). THE OTHER END OF THE RESISTOR MUST BE SOLDERED TO JUMPER WIRE J243. THESE COMPONENTS SHOULD BE COVERED WITH AN ISOLATION COVER. THIS MODIFICATION MUST NOT BE DONE UNDER THE PCB DUE TO THE “SAFETY RULES”. OUT |  |  |  |  |  |  |  |
| NO | PCB CODE | VESTEL CODE |  | COMPONENT NAME | POSITION NUMBER |  | QUANTITY |
| - | - | - |  | - | - |  | - |

IN

| NO | PCB CODE | VESTEL CODE | COMPONENT NAME | POSITION NUMBER | QUANTITY |
| --- | --- | --- | --- | --- | --- |
| 1 | 11AK19E-3 | 30001329 | DIODE IN4007 | DX | 1 |
| 2 | 11AK19E-3 | 30000466 | RES CF 1/4W 1K J | RX | 1 |

| APPLICATION ENGINEER | APPLICATION ENG. CHIEF | APPLICATION ENG. MANAGER |
| --- | --- | --- |
| - | YILDIRAY BAŞKURT | BEYHAN DAĞITAN |

KALT-F.04-00.04