|  |  |  |  | PRODUCT CONTENT CHANGE BULLETIN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | REF NO |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ELEKTRONİK A.Ş ORGANİZE SANAYİ BÖL.MANİSA/ TURKEY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | UMB - 94 |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EXPLANATION: |  | This modification was made for cost-down. Not-1: Only using, 110º 170-270V modells. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| APPLICATION REQUEST DATE 20/ 3 /2001 |  |  |  |  |  |  |  |  |  |  |  | * IMMEDIATELY |  |  |  |  |  | * RUNNING CHANGE |  |  |  |  |  |  |  |  |  |
| AFFECTED PRODUCTS |  |  |  |  |  |  |  | ALL MODELS USING 11AK19P MOTHER BOARD , WITH 30002084 TRF SMPS AK19 90° (90-270V) 30002083 TRF SMPS AK19 90° (170-270V) 30002080 TRF SMPS AK19 110° (170-270V) SMPS TRANSFORMERS. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ADD. LIST |  |
| COMPONENTS OUT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| No | PCB Code / Pic. No |  |  |  | VESTEL Code |  |  |  | Description |  |  |  |  |  |  |  |  |  |  |  | Position |  |  |  |  |  | Amount |
| 1 |  |  |  |  | 30000353 |  |  |  | CAP EL 100UF 25V M |  |  |  |  |  |  |  |  |  |  |  | C848 (NOTE-1) |  |  |  |  |  | 1 |
| 2 |  |  |  |  | 30001288 |  |  |  | DIODE BYV27-200 |  |  |  |  |  |  |  |  |  |  |  | D813 |  |  |  |  |  | 1 |
| 3 |  |  |  |  | 30001315 |  |  |  | DIODE BYD33D |  |  |  |  |  |  |  |  |  |  |  | D811 |  |  |  |  |  | 1 |
| 4 |  |  |  |  | 20051438 |  |  |  | HE.ASY.19P-REG(2) |  |  |  |  |  |  |  |  |  |  |  | IC804 |  |  |  |  |  | NOT-2 |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COMPONENTS IN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| No | PCB Code / Pic. No |  |  |  | VESTEL Code |  |  |  | Description |  |  |  |  |  |  |  |  |  |  |  | Position |  |  |  |  |  | Amount |
| 1 |  |  |  |  | 30000376 |  |  |  | CAP EL 220UF 25V (OTO DİZGİ'ye uygun olmalıdır) |  |  |  |  |  |  |  |  |  |  |  | C848 (NOTE-1) |  |  |  |  |  | 1 |
| 2 |  |  |  |  | 30001315 |  |  |  | DIODE BYD33D |  |  |  |  |  |  |  |  |  |  |  | D813 |  |  |  |  |  | 1 |
| 3 |  |  |  |  | 30001315 |  |  |  | DIODE BYD33D |  |  |  |  |  |  |  |  |  |  |  | D817 |  |  |  |  |  | 1 |
| 4 |  |  |  |  | 30001668 |  |  |  | IC LM317T |  |  |  |  |  |  |  |  |  |  |  | IC804 |  |  |  |  |  | 1 |
| 5 |  |  |  |  | 30002583 |  |  |  | KISA DEVRE TELİ 0.6MM |  |  |  |  |  |  |  |  |  |  |  | S600 |  |  |  |  |  | 1 |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COMPONENT IN |  |  |  |  | ALTERNATIVES FOR COMPONENTS IN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | EXIST NOT EXIST |  |  |
|  |  |  |  |  | COMPONENT IN |  |  |  |  | ALTERNATIVE |  |  |  |  |  |  |  |  | DIFFERENCE COMP. |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ADD. LIST |  |  |  |  |  |  |  |  |
| ## RESPONSIBLE ENGINEER |  |  |  |  |  | ## App. Eng. Manager Approval |  |  |  |  |  |  |  |  |  | ## FINAL NOTES |  |  |  |  |  |  |  |  |  |  |  |
| NAME SURNAME / SIGNATURE ERCAN ÖZ 15-10-2001 |  |  | DATE/TIME |  |  | SIGNATURE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |