|  |  |  |  | PRODUCT CONTENT CHANGE BULLETIN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | REF NO |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ELEKTRONİK A.Ş ORGANİZE SANAYİ BÖL.MANİSA/ TURKEY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | UMB - xx |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EXPLANATION: |  | THE BREAKDOWN PROBLEM OF THE IC TDA 6111Q ON THE POSITIONS IC901, IC902 AND IC903 ON THE 11TP28 BASE BOARD. 11TP28 VER E:5 PCB SHOULD BE REPLACED WITH 11TP28-7 IMMEDIATELY. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| APPLICATION REQUEST DATE |  |  |  |  |  |  |  |  |  |  |  | * IMMEDIATELY |  |  |  |  |  |  | * RUNNING CHANGE |  |  |  |  |  |  |  |  |
| AFFECTED PRODUCTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ADD. LIST |  |
| COMPONENTS OUT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| No | PCB Code / Pic. No |  |  |  | VESTEL Code |  |  |  | Description |  |  |  |  |  |  |  |  | Position |  |  |  |  |  |  |  |  | Amount |
| 1 |  |  |  |  | 30016571 |  |  |  | PCB 11TP28 (VER:E5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| 2 | ##### 11 TP 28 VER-5 |  |  |  | 30000433 |  |  |  | CAP CER 1NF 1KV M B |  |  |  |  |  |  |  |  | C 937 |  |  |  |  |  |  |  |  | 1 |
| 3 | ##### 11 TP 28 VER-5 |  |  |  | 30000428 |  |  |  | SPARK GAP 300V |  |  |  |  |  |  |  |  | SP 901, SP XX |  |  |  |  |  |  |  |  | 2 |
| 4 | ##### 11 TP 28 VER-5 |  |  |  | 30000580 |  |  |  | RES CF 1/4W 22R J |  |  |  |  |  |  |  |  | R 940 |  |  |  |  |  |  |  |  | 1 |
| 5 | ##### 11 TP 28 VER-5 |  |  |  | 30001994 |  |  |  | FIXED COIL 22UH J AXIAL |  |  |  |  |  |  |  |  | J 921 |  |  |  |  |  |  |  |  | 1 |
| 6 | ##### 11 TP 28 VER-5 |  |  |  | 30002583 |  |  |  | KISA DEVRE TELİ 0.6MM |  |  |  |  |  |  |  |  | J 920 |  |  |  |  |  |  |  |  | 1 |
| 7 | ##### 11 TP 28 VER-5 |  |  |  | 30000295 |  |  |  | CAP CER 100NF 50V Z F |  |  |  |  |  |  |  |  | C 940 |  |  |  |  |  |  |  |  | 1 |
| 8 | ##### 11 TP 28 VER-5 |  |  |  | 30002876 |  |  |  | CABLE ASSY 1/25 AK19 EMC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COMPONENTS IN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| No | PCB Code / Pic. No |  |  |  | VESTEL Code |  |  |  | Description |  |  |  |  |  |  |  |  | Position |  |  |  |  |  |  |  |  | Amount |
| 1 |  |  |  |  | 30017573 |  |  |  | PCB 11TP28-7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| 2 | ##### 11 TP 28-7 |  |  |  | 30000428 |  |  |  | SPARK GAP 300V |  |  |  |  |  |  |  |  | SP 901, SP 903, SP 904 SP 905, SP 906, SP 907 |  |  |  |  |  |  |  |  | 6 |
| 3 | ##### 11 TP 28-7 |  |  |  | 30001994 |  |  |  | FIXED COIL 22UH J AXIAL |  |  |  |  |  |  |  |  | L 921 |  |  |  |  |  |  |  |  | 1 |
| 4 | ##### 11 TP 28-7 |  |  |  | 30002583 |  |  |  | KISA DEVRE TELİ 0.6MM |  |  |  |  |  |  |  |  | J 915, J 916 |  |  |  |  |  |  |  |  | 2 |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COMPONENT IN |  |  |  |  | ALTERNATIVES FOR COMPONENTS IN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | EXIST NOT EXIST |  |  |
|  |  |  |  |  | COMPONENT IN |  |  |  |  | ALTERNATIVE |  |  |  |  |  |  |  |  |  | DIFFERENCE COMP. |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ADD. LIST |  |  |  |  |  |  |  |
| ## RESPONSIBLE ENGINEER |  |  |  |  |  | ## App. Eng. Manager Approval |  |  |  |  |  |  |  |  |  | ## FINAL NOTES |  |  |  |  |  |  |  |  |  |  |  |
| NAME SURNAME / SIGNATURE |  |  | DATE/TIME |  |  | SIGNATURE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |