SE305 HELP

Introduction

SE305 is a program for collecting data from thermometer which is connected with a PC or notebook. One may process these data with graphic or text style interface. The program is very easy to use via the graphic user interface. The users do not have to search the function within a lot of menus. The major functions are all in the main window and they are indicated with large icons.

System Required:

Windows 95/98 or Windows XP\NT 4.0 or Vista 32bits.

Minimum Hardware Required:

486-100 MHz , 16 MB RAM ;

At least 5 MB byte hard disk space available to install THERMOLINK software.

Recommended display resolution 800X600 with High Color(16 bit).

Main Menu

Open  : Open an old file .

Save  : Save this data set to a file .

Print  : Print tabular of this data set.

Real Time –Run: Begin to graph real time data .

Real Time –Stop: Stop graphing real time data.

Sampling Rate - Time interval of recording data. If you want to change it, move mouse cursor to the digit you want to change and click, then input the new sampling rate, then the change will work immediately.

Option – Range : Setup range of Y axis .(When you find there is nothing in the graph, it could be that the data is out of range, you can adjust the range to fit the graph)

Option – Graph Customization : Setup dialog to set attributions of graph.(You can also double click on the graph.)

Data Logger  : Load recorded data which were saved in memory of meter to PC.

: At beginning, this button is disabled, when you drag a rectangle on the graph to zoom in the selected area, this will enabled the button, then you can click it to set the graph to original grid scale.

DataLogger

When you have thermometer connected to PC, select "Load Data" to load recorded data from the meter and there will be a progress indicator to show the loading progress. if error occurs, just click "Load Data" again. After the data was loaded, the left hand side will show how many data sets were loaded and detail information for each data set (start data, start time, recording rate and record numbers).

for examples: It will transfer first data set to graph and data list on the right hand side every time after you load recorded data from the thermometer and you can click at any data set to change the set for graph and tabular.

You can choose a rectangle area on the graph to zoom in for detail.