Wavetek 1395 Arbitrary Waveform Synth.

_____________________________________________________________________________

Introduction:

This instrument driver provides programming support for Wavetek 1395

Arbitrary Waveform Synthesizer. It contains functions for opening,

configuring, taking measurement from, and closing the instrument.

_____________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

-  the instrument is installed in the VXI mainframe and you are

using one of the following controller options:

Embedded controller

AT-MXI

MC-MXI

GPIB-VXI

-  the logical address supplied to the initialize function must

match the logical address of the instrument.

-  if you are using GPIB-VXI, there must be at least one unused

device name in the handler.

_____________________________________________________________________________

LabWindows Libraries:

In order to use this instrument driver outside of the LabWindows

environment, it must be linked with the following LabWindows libraries:

-  The appropriate VXI Library:

VXI386.LIB for embedded controllers

VXIAT.LIB for the AT-MXI

VXIMC.LIB for the MC-MXI

GPIB_C.LIB for GPIB-VXI (C users)

-  Format and I/O Library

-  LabWindows System Library

National Instruments strongly recommends that you use the LWMAKE utility to

build any standalone executable program based on this instrument driver.

_____________________________________________________________________________

Error Codes:

Error codes are returned as the return value of each instrument driver

function. A program should examine this value after each call to an

instrument driver function to determine if any error occurred. Possible

error codes and their meanings are listed with the corresponding instrument

driver function.

_____________________________________________________________________________

How To Use This Document:

This document is intended to be used as a programming reference manual.

It describes each function in the Wavetek 1395 Arbitrary Waveform Synth.

instrument driver. The functions are listed in alphabetical order with

a description of the function, C syntax of the function, a description

of each parameter, and possible error codes.

_____________________________________________________________________________

Function Tree Layout:

Wavetek 1395 Arbitrary Waveform Synth.     Function Name:

Initialize                              wtk1395_init

Configure

Frequency

Continuous                        wtk1395_cw

Raster                            wtk1395_raster

Sweep                             wtk1395_sweep

List                              wtk1395_list

Waveform Type

Standard                          wtk1395_stand

Trace                             wtk1395_trac

Get Trace from A24 Mem            wtk1395_getdat

Sequence                          wtk1395_sequence

Trigger

Trigger Enable                    wtk1395_trigen

Start Trigger                     wtk1395_starttrig

Advance Trigger                   wtk1395_adtrig

Software Trigger                  wtk1395_softtrig

Set Internal Trig. Rate           wtk1395_settrig

Markers                              wtk1395_markers

Voltage

Set DC Offset                     wtk1395_offset

Set Maximum Amplitude             wtk1395_ampl

Phase

Lock                              wtk1395_lock

Adjust                            wtk1395_adjust

Output

Enable Main                       wtk1395_outen

Filter                            wtk1395_filter

Sumbus Out                        wtk1395_sumout

Output Trigger                    wtk1395_outtrig

Clock                             wtk1395_clock

Sumbus Input                         wtk1395_sumin

Modulation                           wtk1395_mod

Reference Oscillator                 wtk1395_refosc

Trace Playback Mode                  wtk1395_trace_mode

Arbitrary Waveform

Trace

Create                            wtk1395_create

Copy                              wtk1395_copy

Delete                            wtk1395_delete

List Directory                    wtk1395_tradir

Available Trace Memory            wtk1395_free

File/Memory Transfer

Write Trace to File            wtk1395_writefile

Write File to Trace Mem        wtk1395_file_to_trace

Write File to A24 Memory       wtk1395_writeA24

Write A24 to Trace Mem         wtk1395_writetrace

Edit

Draw Line                      wtk1395_line

Define Points                  wtk1395_define_pts

Change Point                   wtk1395_point

Re-Scale                       wtk1395_rescale

Get Limits                     wtk1395_getlim

Get Point Value                wtk1395_getpoint

Get Trace Size                 wtk1395_getsize

Sequence

Define Sequence                   wtk1395_defseq

Add Sequence Element              wtk1395_linkseq

Accessories

Read Error Queue                     wtk1395_inst_err

Test                                 wtk1395_test

Calibrate                            wtk1395_cal

Low Level Functions

Reset                                wtk1395_reset

Write To Instrument                  wtk1395_write

Read Instrument Data                 wtk1395_read

Close                                   wtk1395_close

_____________________________________________________________________________

Instrument         : Wavetek 1395 Arbitrary Waveform Synth.

Instrument Name: Wavetek 1395 Arbitrary Waveform

Synthesizer

Description:     This instrument module provides

programming support for the

Wavetek 1395.  The module

is divided into the following

functions:

Functions/Classes:

(1) Initialize

Initializes the instrument and sets it to a

default configuration.

(2) Configure

A class of functions for configuring the

frequency, waveform(trace), trigger,

markers, phase, output, and voltage level

subsystems.

(3) Arbitrary Waveform

A class of functions to create and edit

traces and sequences to be used for arbitrary

waveform generation.

(4) Accessories

A class of functions for reading the

instrument error queue, and performing

tests and calibrations on the instrument.

(5) Low Level Functions

A class of lower level functions to

communicate with the instrument, and change

instrument parameters.

(6) Close

Takes the instrument offline.

_____________________________________________________________________________

Following functions are in alphabetical order.

_____________________________________________________________________________

Function Name      : wtk1395_adjust

C Syntax           : int wtk1395_adjust(Instrument_ID, Phase_Value);

int Instrument_ID;

int Phase_Value;

Purpose            : This function specifies the phase delay for the

output waveform.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Phase Value

Description:   This control specifies the phase

delay for the output waveform in

degrees.

Variable Type: Integer

Valid Range:   -180 to 180

Default Value:  0

Errors             :         0   Success

-1   Instrument ID out of range

-2   Phase Value out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_adtrig

C Syntax           : int wtk1395_adtrig(Instrument_ID, Source, Polarity,

Gate, Count_Cycles, Gate_Mode);

int Instrument_ID;

int Source;

int Polarity;

int Gate;

long Count_Cycles;

int Gate_Mode;

Purpose            : This routine performs the following operations:

- Configures the instrument for triggered operation.

- Selects the polarity of the advance trigger.

- Specifies the number(count) of event repetitions

initiated by the advance trigger.

- Selects the source of the advance trigger.

- Enables/disables the gated mode of operation.

- Selects synchronous or asynchronous gate mode.

NOTE:  The start trigger initiates an operation,

while the advance trigger advances to

subsequent functions within a sequence

structure.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Source

Description:   This control selects the source of

the start trigger.

Variable Type: Integer

Valid Range:   0 - Internal (default)

1 - External

2 - Chain

3 - EChain

Polarity

Description:   This control selects either the

positive or negative slope for

advance triggering.

Variable Type: Integer

Valid Range:   0 - Negative Slope

1 - Positive Slope (default)

Gate

Description:   This control enables or disables

gated trigger operation.

Variable Type: Integer

Valid Range:   0 - Disable (default)

1 - Enable

Count Cycles

Description:   This control specifies the number

of times to cycle through a trace

after a trigger signal is

received.

Variable Type: Long

Valid Range:   1 to 1,048,575 (non-seq waveforms)

1 to 524,288 (seq waveforms)

Default Value: 1

Gate Mode

Description:   This control selects synchronous

or asynchronous gate operation.

Variable Type: Integer

Valid Range:   0 - Asynchronous

1 - Synchronous (default)

Errors             :         0   Success

-1   Instrument ID out of range

-2   Source out of range

-3   Polarity out of range

-4   Gate out of range

-5   Count Cycles out of range

-6   Gate Mode out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_ampl

C Syntax           : int wtk1395_ampl(Instrument_ID, Maximum_Amplitude);

int Instrument_ID;

double Maximum_Amplitude;

Purpose            : This function configures the optimum output resolution

of user-defined traces by scaling the maximum amplitude

to the maximum memory value and the negative amplitude

to the minimum memory value. Refer to operation manual

for further details.

NOTE:  Unless a 50 ohm terminator (50 ohms in

parallel with the load and a 50 ohm cable

resistance in series) is used, the offset

voltage will be twice the entered value.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Maximum Amplitude

Description:   This control specifies the maximum

amplitude for optimum scaling.

Variable Type: Double

Valid Range:   0 to 7.5

Default Value: 1.0

Errors             :         0   Success

-1   Instrument ID out of range

-2   Maximum Amplitude out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_cal

C Syntax           : int wtk1395_cal(Instrument_ID, Calibration_Result);

int Instrument_ID;

int *Calibration_Result;

Purpose            : This routine performs the following actions:

- Instructs the instrument to perform an internal

self-calibration.

- Returns a two-byte, bit weighted error code in

hexadecimal format. (Detailed descriptions of the

error codes may be found in the instrument's

operation manual.) A zero indicates that no error

occurred.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Calibration Result

Description:   Displays the bit-weighted error

code from the instrument hardware

calibration in hexadecimal format.

A zero indicates that no

calibration errors occurred. Error

codes are listed in the instrument

operation manual.

Variable Type: Integer

Valid Range:   -32767 to 32768

Errors             :         0   Success

-1   Instrument ID out of range

226   Error polling instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_clock

C Syntax           : int wtk1395_clock(Instrument_ID, Configuration,

Output_Mode, User_Frequency);

int Instrument_ID;

int Configuration;

int Output_Mode;

double User_Frequency;

Purpose            : This function performs the following operations:

- Specifies if the CLK IN/OUT BNC is to be used for

input or output

- Specifies the output mode for the CLK IN/OUT BNC

- Specifies the output frequency of the CLK IN/OUT BNC

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Configuration

Description:   Specifies if the CLK IN/OUT BNC is

to be used for input or output.

Variable Type: Integer

Valid Range:   0 = Output

1 = Input (Default)

Output Mode

Description:   Selects the output mode for the

CLK IN/OUT BNC when the clock

configuration is set for output.

User Frequency mode disables all

other output modes and sources a

clock signal at the specified

frequency. Raster and synthesizer

modes do not disable other output

sources and use the raster and

synthesizer frequencies,

respectively, as source

frequencies.

Variable Type: Integer

Valid Range:   0 - User Frequency (default)

1 - Raster

2 - Synthesizer

User Frequency

Description:   Specifies the frequency of the

output clock when the clock

configuration is set for output

and the clock mode is set to User

Frequency.

Variable Type: Double

Valid Range:   1E-1 to 1E8

Default Value: 1E3

Errors             :         0   Success

-1   Instrument ID out of range

-2   Configuration out of range

-3   Output Mode out of range

-4   User Frequency out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_close

C Syntax           : int wtk1395_close(Instrument_ID);

int Instrument_ID;

Purpose            : The close routine performs the following operations:

-  removes the entry for the instrument from the

Instrument Table.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Errors             :         0   Success

-1   Instrument ID out of range

221   Unable to close device

232   Device not initialized

_____________________________________________________________________________

Function Name      : wtk1395_copy

C Syntax           : int wtk1395_copy(Instrument_ID, Source, Target_Trace);

int Instrument_ID;

char *Source;

char *Target_Trace;

Purpose            : This function copies either a standard or user-defined

trace (source trace) to a new location in trace memory,

callable by the target trace name.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Source

Description:   This control specifies the name of

the standard or user-defined

trace to be copied.

Variable Type: String

Target Trace

Description:   This control specifies the name of

the trace to which the source is

copied.

Variable Type: String

Errors             :         0   Success

-1   Instrument ID out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_create

C Syntax           : int wtk1395_create(Instrument_ID, New_Trace_Name,

Point_Count);

int Instrument_ID;

char *New_Trace_Name;

int Point_Count;

Purpose            : This function creates a user waveform in trace memory

called by the trace name with the specified number of

points.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

New Trace Name

Description:   This control specifies the name of

the trace to be created.

Variable Type: String

Point Count

Description:   This control specifies the number

of data points to be allocated for

the new trace in trace memory.

Variable Type: Integer

Valid Range:   0 to 32767

Default:       4096

NOTE:  While a single trace may have up to 32767

points, it is recommended to use traces of

only 4096 to maintain compatibility with

other source modes.

Errors             :         0   Success

-1   Instrument ID out of range

-3   Point Count out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_cw

C Syntax           : int wtk1395_cw(Instrument_ID, Frequency);

int Instrument_ID;

double Frequency;

Purpose            : This routine performs the following operations:

- Configures the frequency mode for continuous wave

operation.

- Sets the frequency value for continuous trace mode.

Note:  The default trace mode for standard

waveforms is continuous and the default

trace mode for arbitrary waveforms is

raster.  An error will be generated if the

instrument is not in the continuous trace

mode.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Frequency

Description:   Sets the output frequency for the

waveform when the trace output

is in raster mode. Floating point

notation (e.g. 12E3) is required.

Variable Type: Double

Valid Range:   1E-6 to 2.5E6

Default Value: 1E3

Errors             :         0   Success

-1   Instrument ID out of range

-2   Frequency out of range

10   Instrument not in continuous

trace mode

226   Error polling instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_define_pts

C Syntax           : int wtk1395_define_pts(Instrument_ID, Trace_Name,

_of_Points, Point_Array);

int Instrument_ID;

char *Trace_Name;

int _of_Points;

int Point_Array[];

Purpose            : This function defines the point values for a specified

trace using an array of integers.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Trace Name

Description:   This control specifies the name of

the trace in which the points will

be defined.

Variable Type: String

Default Value: "trace1"

# of Points

Description:   Specifies the number of points

that will be defined for the given

trace (i.e. the number of elements

in Point Array)

Variable Type: Integer

Valid Range:   0 to 32,767 (4096)

(this value is bounded by the

number of elements in Point Array)

NOTES: The maximum size of a waveform is

(32,767 - Available Memory (Points In

Use)).

Waveforms should be limited to 4096 data

points to maintain compatibility with

all output modes.

Point Array

Description:   This control specifies the new

voltage value of the point to be

changed.

Variable Type: Integer Array

Valid Range:   Each element of the array is

bounded by the following criteria:

If the Set Maximum Amplitude

function is performed, then the

maximum amplitude is scaled to

the value 4095 and the negative

maximum amplitude is scaled

to 0.

Errors             :         0   Success

-1   Instrument ID out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_defseq

C Syntax           : int wtk1395_defseq(Instrument_ID, Sequence_Length,

Trigger_Sense, Trigger_Mode);

int Instrument_ID;

int Sequence_Length;

int Trigger_Sense;

int Trigger_Mode;

Purpose            : This function performs the following functions:

- Configures the instrument for the sequence mode.

- Disables the fixed mode.

- Sets up a sequence table of length specified by the

user (up to 4 elements.)

Note:  The elements in the sequence table must be

created and linked together using the Link

Sequence function.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Sequence Length

Description:   Sets the number of traces in the

sequence table.

Variable Type: Integer

Valid Range:   2 to 4

Trigger Sense

Description:   This control selects the

active portion of the advance

trigger signal. Edge selects the

rising edge of the trigger signal

to initiate an advance. Level

selects the trigger signal level

to initiate an advance.

When the current function element

is configured for Triggered

Advance and an advance is

generated, the next function in

the sequence list is output.

Variable Type: Integer

Valid Range:   0 - Level

1 - Edge  (default)

Trigger Mode

Description:   Defines the transition mode

between the current function and

the next function in the

sequence table.  Synchronous mode

waits until the end of the current

function before moving to the

next. Asynchronous mode selects

the next function immediately,

even if the current function is

not completed.

Variable Type: Integer

Valid Range:   0 - Asynchronous

1 - Synchronous  (default)

Errors             :         0   Success

-1   Instrument ID out of range

-2   Sequence length out of range

-3   Trigger sense out of range

-4   Trigger mode out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_delete

C Syntax           : int wtk1395_delete(Instrument_ID, Function, Trace_Name);

int Instrument_ID;

int Function;

char *Trace_Name;

Purpose            : This function deletes all traces or a single, specified

trace from trace memory.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Function

Description:   This control selects either to

delete all traces or a single

trace. Trace name is disregarded

if Delete All is selected.

Variable Type: Integer

Valid Range:   0 - Single Trace (default)

1 - All Traces

Trace Name

Description:   This control specifies the name of

the trace to be deleted.

Variable Type: String

Errors             :         0   Success

-1   Instrument ID out of range

-2   Function out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_file_to_trace

C Syntax           : int wtk1395_file_to_trace(Instrument_ID, File_Name,

Trace_Name);

int Instrument_ID;

char *File_Name;

char *Trace_Name;

Purpose            : This function writes the contents of a file (trace data

previously stored) into trace memory.

This function operates for AT-MXI, embedded VXI, or

GPIB-VXI. For embedded VXI and AT-MXI, a word-serial

write is performed, bypassing the A24 memory space.

For large volumes of data, the functions Write Data to

A24 and Write A24 to Trace should be used.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

File Name

Description:   The name of the file into which

the trace is written.

Variable Type: String

Default Value: "trace1"

Trace Name

Description:   The name of the trace in trace

memory to be written.

Variable Type: String

Default Value: "trace1"

Errors             :         0   Success

-1   Instrument ID out of range

226   Error polling instrument

228   Error writing to instrument from file

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

305   No room in trace memory

320   Error reading file from disk

_____________________________________________________________________________

Function Name      : wtk1395_filter

C Syntax           : int wtk1395_filter(Instrument_ID, Filter_State, Type);

int Instrument_ID;

int Filter_State;

int Type;

Purpose            : This function performs the following operations

- Enables/disables the 20 MHz low pass filter between

the output of the waveform DAC and the pre-amplifier.

- Selects either a 4-pole Bessel filter (flattest

response with constant group delay) or a 7-pole,

6-zero Elliptic (steepest cutoff.)

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Filter State

Description:   This control enables/disables the

20 MHz filter between output of

the waveform DAC and the

pre-amplifier.

Variable Type: Integer

Valid Range:   0 - Disabled

1 - Enabled (default)

Type

Description:   This control selects the type of

filter enabled between the

waveform DAC and the

pre-amplifier (in the output

section.)

Variable Type: Integer

Valid Range:   0 - 7 pole, 6 zero Elliptic Filter

for flattest response with

constant group delay.(default)

1 - 4 pole Bessel Filter for

steepest response.

Errors             :         0   Success

-1   Instrument ID out of range

-2   Filter State out of range

-3   Type out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_free

C Syntax           : int wtk1395_free(Instrument_ID, Points_Available,

Points_In_Use);

int Instrument_ID;

long *Points_Available;

long *Points_In_Use;

Purpose            : This function returns a string specifying the number

of data points available and the number of data points

in use.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Points Available

Description:   Returns the number of points

available in the instrument's

memory.

Variable Type: Long

Points In Use

Description:   Returns the number of points

already used in the instrument's

memory.

Variable Type: Long

Errors             :         0   Success

-1   Instrument ID out of range

226   Error polling instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_getdat

C Syntax           : int wtk1395_getdat(Instrument_ID);

int Instrument_ID;

Purpose            : This function performs the following operations:

- Returns the data located in the first 8 KBytes of

the VXIbus A24 shared memory and selects this

waveform as the current output function.

- Configures the function mode as SHAPE.

- Disables the USER function mode.

- Sets the trace mode to continuous wave.

- Sets the source marker to SYNchronized, triggering

off the output waveforms zero-crossing.

- Enables the elliptic output filter.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Errors             :         0   Success

-1   Instrument ID out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_getlim

C Syntax           : int wtk1395_getlim(Instrument_ID, Trace_Name, Start,

Stop);

int Instrument_ID;

char *Trace_Name;

int *Start;

int *Stop;

Purpose            : This function returns the playback limits specified by

the trace name.

NOTE:  The playback limits may be different from

the actual trace size. Only the portion of

the specified trace, partitioned by the

trace limits are available as an output

source.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Trace Name

Description:   This control specifies the trace

name from which limits are

returned.

Variable Type: String

Start

Description:   Returns the starting playback

limit of the specified trace.

Variable Type: Integer

Stop

Description:   Returns the stopping playback

limit of the specified trace.

Variable Type: Integer

Errors             :         0   Success

-1   Instrument ID out of range

226   Error polling instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_getpoint

C Syntax           : int wtk1395_getpoint(Instrument_ID, Trace_Name,

Point_Index, Point_Value);

int Instrument_ID;

char *Trace_Name;

int Point_Index;

int *Point_Value;

Purpose            : This function returns the value of a single point in

trace memory, specified by a trace name and an index.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Trace Name

Description:   This control specifies the name of

the trace in which the point is

found.

Variable Type: String

Point Index

Description:   This control selects the point

index of the specified trace.

Variable Type: Integer

Valid Range:   0 to 32,767 (4095)

NOTES: The maximum size of a waveform is

(32,767 - Available Memory (Points In

Use)).

Waveforms should be limited to 4095 data

points to maintain compatibility with

all output modes.

Point Value

Description:   This control stores the point

(voltage) value from the specified

trace.

Variable Type: Integer

Valid Range:   0 to 4095

Errors             :         0   Success

-1   Instrument ID out of range

-3   Point Index out of range

226   Error polling instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_getsize

C Syntax           : int wtk1395_getsize(Instrument_ID, Trace_Name,

Trace_Size);

int Instrument_ID;

char *Trace_Name;

int *Trace_Size;

Purpose            : This function returns the size (number of indexed data

points) of a specified trace.

NOTE:  The playback limits may be different from

the actual trace size. Only the portion of

the specified trace partitioned by the

trace limits are available as an output

source.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Trace Name

Description:   This control specifies the name of

the trace for which the size is

found.

Variable Type: String

Trace Size

Description:   This control returns the size

of the specified trace. If trace

limits have been set, then the

number of points between the

limits will be returned.

Variable Type: Integer

Valid Range:   0 to 32767

Errors             :         0   Success

-1   Instrument ID out of range

226   Error polling instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_init

C Syntax           : int wtk1395_init(Logical_Address, Interface, ID_Query,

Reset_Device, Instrument_ID);

int Logical_Address;

int Interface;

int ID_Query;

int Reset_Device;

int *Instrument_ID;

Purpose            : This routine performs the following initialization

actions:

-  Opens the instrument by storing information about

the instrument in the Instrument Table.

-  Initializes the VXI interface (NI-VXI or NI-GPIB)

accordingly.

-  Performs an identification query on the Instrument.

-  Sends initialization commands to the instrument.

-  Returns an Instrument ID which is used to

differentiate between instruments of the same

model type.  This value will be used to identify

the instrument in subsequent calls.

Parameter List     : Logical Address

Description:   Specifies the Logical Address of

the VXI instrument.  This value

must correspond to the current

internal setting of the

instrument.

Variable Type: Integer

Valid Range:   0 - 255

Default Value: 1

Interface

Description:   Specifies which protocol is used

to communicate with the VXI

mainframe.

NI-VXI should be selected for

MC-MXI, AT-MXI, or VXIpc-386.

NI-GPIB should be selected for the

GPIB-VXI.

Variable Type: Integer

Valid Range:   1 - NI-VXI

2 - NI-GPIB

Default Value: 1 - NI-VXI

ID Query

Description:   Specifies if an ID Query is sent

to the instrument during the

initialization procedure.

Variable Type: Integer

Valid Range:   1 = Yes

0 = No

Default Value: 1 - Yes

NOTE:  Under normal circumstances the ID Query

insures that the instrument initialized

over the GPIB is the type supported by

this driver. However circumstances may

arise where it is undesirable to send an

ID Query to the instrument. In those

cases; set this control to Skip Query

and this function will initialize the

GPIB and the Command arrays in the driver,

without doing an ID Query.

Reset Device

Description:   Specifies if the instrument is to

be reset to its power-on settings

during the initialization

procedure.

Variable Type: Integer

Valid Range:   1 = Yes

0 = No

Default Value: 1 - Yes

NOTE: If you do not want the instrument reset.

Set this control to No while initializing

the instrument.

Instrument ID

Description:   Returns an Instrument ID that is

used in all subsequent function

calls to select the instrument at

the address specified.

If more than one instrument of the

same model type is used, this ID

will be used to differentiate

between them. The first instrument

of this model type initialized

will be assigned an Instrument ID

of 1.

Variable Type: Integer

Errors             :         0   Success

-1   Logical Address out of range

-2   Interface out of range

-3   ID Query out of range

-4   Reset Device out of range

220   Unable to open instrument

221   Unable to close instrument

222   Unable to initialize VXI library

223   Instrument identification query failed

224   Error clearing instrument

230   Error writing to instrument

231   Error reading from instrument

233   Error configuring GPIB address

236   Error interpreting instrument response

239   Error setting time out

241   Error configuring repeat addressing

_____________________________________________________________________________

Function Name      : wtk1395_inst_err

C Syntax           : int wtk1395_inst_err(Instrument_ID, Instr__Error_Code);

int Instrument_ID;

int *Instr__Error_Code;

Purpose            : This function reads all errors from the error queue

and returns the code for the last error.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Instr. Error Code

Description:   This output returns the error code

read from the instrument error

queue.

To convert  this value to a

Wavetek Error code, subtract 200

and then multiply it by -1.

Then look in appendix D of the

Wavetek 1395 Operator's Manual.

Variable Type: Integer

Errors             :         0   Success

-1   Instrument ID out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_line

C Syntax           : int wtk1395_line(Instrument_ID, Trace_Name,

Point_1_Index, Point_1_Value, Point_2_Index,

Point_2_Value);

int Instrument_ID;

char *Trace_Name;

int Point_1_Index;

int Point_1_Value;

int Point_2_Index;

int Point_2_Value;

Purpose            : This function performs the following operations

- Draws a line between two specified points in a

trace.

- Erases previous values between the two indices.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Trace Name

Description:   This control specifies the name of

the trace in which the line is

drawn.

Variable Type: String

Point 1 Index

Description:   This control specifies the index

of the beginning point of the line

to be drawn.

Variable Type: Integer

Valid Range:   0 to 32,767 (4095)

NOTES: The maximum size of a waveform is

(32,767 - Available Memory (Points In

Use)).

Waveforms should be limited to 4095 data

points to maintain compatibility with

all output modes.

Point 1 Value

Description:   This control specifies the voltage

value of the first point of the

line to be drawn.

Variable Type: Integer

Valid Range:   0 to 4095

NOTE:  If the Set Maximum Amplitude function is

performed, then the maximum amplitude is

scaled to the value 4095 and the negative

maximum amplitude is scaled to 0.

Point 2 Index

Description:   This control specifies the index

of the ending point of the line

to be drawn.

Variable Type: Integer

Valid Range:   0 to 32,767 (4095)

NOTES: The maximum size of a waveform is

(32,767 - Available Memory (Points In

Use)).

Waveforms should be limited to 4095 data

points to maintain compatibility with

all output modes.

Point 2 Value

Description:   This control specifies the voltage

value of the second point of the

line to be drawn.

Variable Type: Integer

Valid Range:   0 to 4095

NOTE:  If the Set Maximum Amplitude function is

performed, then the maximum amplitude is

scaled to the value 4095 and the negative

maximum amplitude is scaled to 0.

Errors             :         0   Success

-1   Instrument ID out of range

-3   Point 1 Index out of range

-4   Point 1 Value out of range

-5   Point 2 Index out of range

-6   Point 2 Value out of range

226   Error polling instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_linkseq

C Syntax           : int wtk1395_linkseq(Instrument_ID, Sequence_Index,

Trace_Name, Dwell, Advance);

int Instrument_ID;

int Sequence_Index;

char *Trace_Name;

long Dwell;

int Advance;

Purpose            : This function creates one element in a sequence table,

and links the element with others using the sequence

index.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Sequence Index

Description:   This control specifies the index

number of the sequence table

element to be added. Do not skip

an index number when creating a

sequence table.

Variable Type: Integer

Valid Range:   0 - First element in sequence

table.

1 - Second element in sequence

table.

2 - Third element in sequence

table.

3 - Fourth element in sequence

table.

Trace Name

Description:   This control specifies the name

of a trace in instrument trace

memory or a standard waveform from

the list of onboard waveforms

(1 cycle, 4 KBytes each.) This

waveform will be ordered by

the sequence index number assigned

by Sequence Index.

Variable Type: String

NOTE:  Sweep mode is compatible with sequence

mode only when the waveforms selected by

trace name are standard waveforms, or are

user-defined functions of length 4 KBytes

(4096 Bytes.)

Dwell

Description:   This control sets the number of

times to cycle through this

function in the sequence table. A

zero will cause the function to be

repeated indefinitely.

Variable Type: Long

Valid Range:   0 to 65535

Default:       1

Advance

Description:   This control selects the

conditions under which the

sequence advances to the next

function in the sequence table.

Automatic advances immediately

after the current index number is

called, triggered mode requires an

Advance Trigger to proceed to the

next function in the sequence

table.

The advance trigger is configured

in the Define Sequence function.

Variable Type: Integer

Valid Range:   0 - Triggered

1 - Automatic (default)

Errors             :         0   Success

-1   Instrument ID out of range

-2   Index parameter out of range

-4   Dwell parameter out of range

-5   Advance parameter out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_list

C Syntax           : int wtk1395_list(Instrument_ID, Frequency_List,

List_Length);

int Instrument_ID;

double Frequency_List[];

int List_Length;

Purpose            : This routine performs the following operations:

- Configures the source function mode for list

operation.

- Defines the list of frequencies for the instrument

to sweep

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Frequency List

Description:   An array containing the list of

sample rates or frequencies to be

generated.  For example, if you

want to sweep a list of three

frequencies you would have

freq_arr[0] = freq #1

freq_arr[1] = freq #2

freq_arr[2] = freq #3

where freq_arr = your array

freq #n  = desired freq

Variable Type: Double Array

Valid Range:   Each element of the array is

bounded by the following

restrictions:

- Frequency values may range from

1e-3 to 2e7 Hz

- List elements not specified will

have a default value of 1e3 Hz

- No more than 1024 list elements

can be specified

List Length

Description:   Specifies the number of

frequencies or array elements to

be written to the instrument.  For

example, if you have defined your

frequency array as follows:

freq_arr[0] = 10e3

freq_arr[1] = 20e3

freq_arr[2] = 30e3

you would enter 3 into this

control to sweep these

frequencies.

Variable Type: Integer

Valid Range:   1 to 1024 (bounded by # of

array elements)

Note:  Make sure that you do not specify a length

that is longer than the number of elements

you have in the array.

Errors             :         0   Success

-1   Instrument ID out of range

-3   List Length out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_lock

C Syntax           : int wtk1395_lock(Instrument_ID, Lock);

int Instrument_ID;

int Lock;

Purpose            : This function matches phase with another Wavetek 1395

on the VXIbus.

NOTE:  This function can be used in conjunction

with the Phase Adjust function to create a

precise phase offset between two Wavetek

1395's.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Lock

Description:   This control locks phase with

another Wavetek 1395 on the

VXIbus.

Variable Type: Integer

Valid Range:   0 - Phase Lock Off (default)

1 - Phase Lock On

Errors             :         0   Success

-1   Instrument ID out of range

-2   Lock out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_markers

C Syntax           : int wtk1395_markers(Instrument_ID, Trace_Name,

Sync_Marker, Sync_Type, Trigger_Marker,

Position_Marker, _of_Points,

Marker_Point_Array);

int Instrument_ID;

char *Trace_Name;

int Sync_Marker;

int Sync_Type;

int Trigger_Marker;

int Position_Marker;

int _of_Points;

int Marker_Point_Array[];

Purpose            : This function configures the marker output of the

instrument.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Trace Name

Description:   This control specifies the trace

for which the trigger and position

markers will be set.

Variable Type: String

Default Value: "trace1"

Sync Marker

Description:   This control enables or disables

sync marker output for the

instrument.

Variable Type: Integer

Valid Range:   0 = Disable

1 = Enable

Default Value: 0 - Disable

Sync Type

Description:   Selects the type of sync marker to

be output by the instrument.

Variable Type: Integer

Valid Range:   0 = BBits

1 = ZCross

(See descriptions below)

Default Value: 1 - ZCross

Sync Marker Descriptions:

BBits - Marker signal is driven from a bit in

Trace RAM.  The bit is set so the

marker is active for the first several

points of the trace.

ZCross - Selects the output of a comparator.

The comparator output is high if the

output signal amplitude is above zero

volts and low if the output signal

amplitude is below zero volts.

Trigger Marker

Description:   This control enables or disables

the trigger marker for the

specified trace.

Variable Type: Integer

Valid Range:   0 = Disable

1 = Enable

Default Value: 0 - Disable

Position Marker

Description:   Determines if all marker points

are to be turned off for the

specified trace, or the points

specified in Marker Point Array

are to be turned on.

Variable Type: Integer

Valid Range:   0 = All Off

1 = Use Array

(See descriptions below)

Default Value: 0 - All Off

Position Marker Descriptions:

All Off - turn all the marker points off for

the specified trace

Use Array - the marker points specified in

Point Array will be turned on for

the specified trace

# of Points

Description:   Specifies the number of points to

be turned on for the designated

trace.  This value corresponds to

the number of elements in Marker

Point Array that will be turned

on.

Variable Type: Integer

Valid Range:   Bounded by number of elements in

Marker Point Array.

Default Value: 1

Note:  Specifying a value larger than the number

of elements in Marker Point Array will

cause an error.

Marker Point Array

Description:   Array that defines the points

which are to have markers set high

for the specified waveform.  For

example, if you have a waveform

with 1000 points and want to turn

on the marker for the first and

last point, you would enter 2 into

the # of Points control and define

your array as follows:

Arr[0] = 1;

Arr[1] = 1000;

where Arr = array you create

Variable Type: Integer Array

Valid Range:   Valid values for each element of

the array are bounded by the

number of points in the specified

waveform.

Errors             :         0   Success

-1   Instrument ID out of range

-3   Point Index parameter out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_mod

C Syntax           : int wtk1395_mod(Instrument_ID, Modulation_State,

Modulation_Mode);

int Instrument_ID;

int Modulation_State;

int Modulation_Mode;

Purpose            : This function performs the following operations:

- Enables/disables the amplitude modulation

input(AM IN BNC).

- Selects either amplitude modulation or suppressed

carrier mode.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Modulation State

Description:   This control enables/disables the

modulation input.

Variable Type: Integer

Valid Range:   0 - disable (default)

1 - enable

Modulation Mode

Description:   This control selects the type of

modulation, either the standard

0 to 100% amplitude modulation

or double-sideband suppressed

carrier modulation.

Variable Type: Integer

Valid Range:   0 - Suppressed carrier mode

1 - Amplitude Modulation (default)

Errors             :         0   Success

-1   Instrument ID out of range

-2   Modulation State out of range

-3   Modulation Mode out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_offset

C Syntax           : int wtk1395_offset(Instrument_ID, Offset_Voltage);

int Instrument_ID;

double Offset_Voltage;

Purpose            : This function generates a DC offset of a specified

magnitude.

NOTE:  Unless a 50 ohm terminator (50 ohms in

parallel with the load and a 50 ohm cable

resistance in series) is used, the offset

voltage may be twice the entered value.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Offset Voltage

Description:   This control specifies the DC

offset voltage generated at the

main output.

Variable Type: Double

Valid Range:   -7.5 to 7.5

Default Value:  0.0

Errors             :         0   Success

-1   Instrument ID out of range

-2   Offset Voltage out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_outen

C Syntax           : int wtk1395_outen(Instrument_ID, Output_State);

int Instrument_ID;

int Output_State;

Purpose            : This function enables/disables the MAIN OUT BNC.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Output State

Description:   This control enables/disables the

MAIN OUT BNC.

Variable Type: Integer

Valid Range:   0 - Disable

1 - Enable (default)

Errors             :         0   Success

-1   Instrument ID out of range

-2   Output State out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_outtrig

C Syntax           : int wtk1395_outtrig(Instrument_ID, Type,

Trigger_Line_Number, Qualifier, Line_State);

int Instrument_ID;

int Type;

int Trigger_Line_Number;

int Qualifier;

int Line_State;

Purpose            : This function performs the following operations

- Selects a target for the output trigger, either ECL

or TTL trigger lines on the VXIbus.

- Selects the source of the trigger from bit markers

or sequence complete qualifiers

NOTE:  The ECL trigger lines can be separately

enabled/disabled with multiple function

calls. Only one TTL trigger line may be

active.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Type

Description:   This control selects either the

ECL or TTL trigger lines on the

VXIbus for targets.

Variable Type: Integer

Valid Range:   0 - TTL Lines  (default)

1 - ECL Lines

Trigger Line Number

Description:   This control specifies the trigger

line number for either ECL or TTL

triggering.

Variable Type: Integer

Valid Range:   0 - Line 0  (ECL or TTL)

1 - Line 1  (ECL or TTL)

2 - Line 2  (TTL only)

3 - Line 3  (TTL only)

4 - Line 4  (TTL only)

5 - Line 5  (TTL only)

6 - Line 6  (TTL only)

7 - Line 7  (TTL only)

Qualifier

Description:   This control selects the source of

the trigger qualifier marker. The

trigger qualifier marker is used

to condition the selected trigger

to be output by the module.

Variable Type: Integer

Valid Range:   0 - Trigger Marker Bit

1 - Position Marker Bit

2 - Burst Complete

3 - Loop Complete

4 - Internal

Line State

Description:   This control enables/disables the

output trigger line selected by

the Trigger Line Number control.

Variable Type: Integer

Valid Range:   0 - Line disabled (default)

1 - Line enabled

Errors             :         0   Success

-1   Instrument ID out of range

-2   Type out of range

-3   Trigger Line Number out of range

-4   Qualifier out of range

-5   Line State out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_point

C Syntax           : int wtk1395_point(Instrument_ID, Trace_Name,

Point_Index, Point_Value);

int Instrument_ID;

char *Trace_Name;

int Point_Index;

int Point_Value;

Purpose            : This function changes the voltage value of a single

point specified by a trace name and an index number.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Trace Name

Description:   This control specifies the name of

the trace in which the point is to

be changed.

Variable Type: String

Point Index

Description:   This control specifies the index

of the point to be changed.

Variable Type: Integer

Valid Range:   0 to 32,767 (4095)

NOTES: The maximum size of a waveform is

(32,767 - Available Memory (Points In

Use)).

Waveforms should be limited to 4095 data

points to maintain compatibility with

all output modes.

Point Value

Description:   This control specifies the new

voltage value of the point to be

changed.

Variable Type: Integer

Valid Range:   0 to 4095

NOTE:  If the Set Maximum Amplitude function is

performed, then the maximum amplitude is

scaled to the value 4095 and the negative

maximum amplitude is scaled to 0.

Errors             :         0   Success

-1   Instrument ID out of range

-3   Point Index out of range

-4   Point Value out of range

226   Error polling instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_raster

C Syntax           : int wtk1395_raster(Instrument_ID, Scan_Rate);

int Instrument_ID;

double Scan_Rate;

Purpose            : This routine performs the following operations:

- Configures the frequency mode for continuous wave

operation.

- Sets the trace scan rate for raster trace mode

Note:  The default trace mode for standard

waveforms is continuous and the default

trace mode for arbitrary waveforms is

raster.  An error will be generated if the

instrument is not in the raster trace

mode.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Scan Rate

Description:   Specifies the scan rate for the

waveform when the trace output

is in raster mode.  The actual

frequency output is calculated as

follows:

Freq = Rate * #Cycles / Length

where Rate    = Scan Rate

#Cycles = # of wave cycles

Length  = points in wave

Variable Type: Double

Valid Range:   1E-1 to 5E7

Default Value: 5E7

Errors             :         0   Success

-1   Instrument ID out of range

-2   Scan Rate out of range

10   Instrument not in raster trace mode

226   Error polling instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_read

C Syntax           : int wtk1395_read(Instrument_ID, Number_Bytes_To_Read,

Read_Buffer, NumBytes_Read);

int Instrument_ID;

int Number_Bytes_To_Read;

char *Read_Buffer;

long *NumBytes_Read;

Purpose            : Reads data from instrument buffer and returns it to

the specified variable in memory.

NOTES: (1) Because the instrument may return both

numeric and text data in response to

queries this function returns the data

in string format.

(2) If valid data is not available at the

instrument's output buffer when this

function is called the instrument will

hang up and the function will not

return until it times out.  If the

time-out is disabled this function

will hang indefinitely and it may be

necessary to reboot the computer to

break out.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Number Bytes To Read

Description:   The number of bytes specified by

this control should be greater

than or equal to the number of

bytes which are to be read from

the instrument. If the actual

number of bytes to be read is

greater than the number this

control specifies then multiple

reads will be necessary to empty

the instrument's output buffer.

If the instrument's output buffer

is not emptied the instrument may

return invalid data when future

reads are performed.

Variable Type: Integer

Default Value: 50 (See NOTE 2)

NOTES: (1) If NO DATA is available at the

instrument's output buffer when this

function is called the instrument will

hang up and the function will not

return until it times out.  If the

time-out is disabled, this function

will hang indefinitely and it may be

necessary to reboot the computer.

(2) If the number of bytes expected is

greater than 50 the value of this

control may be increased. If the

actual number of bytes read is larger

than the declared size of the read

buffer a run-time error will be

generated.

Read Buffer

Description:   The incoming data from the

instrument is placed into this

variable.

Variable Type: String

NOTES: (1) Because the Instrument may return both

numeric and text data in response to

queries this function returns the data

in string form.

(2) This function does not overwrite any

old data left in the string variable

from the last time the function was

called. String data in LabWindows is

terminated with an ASCII null(Hex 0)

and string manipulation functions will

only recognize data before the ASCII

null.

(3) The declared size of the string

variable must be greater than the

actual number of bytes read from the

instrument if it is not a run-time

error will be generated.

NumBytes Read

Description:   This variable contains the actual

number of bytes read from the

instrument. This is the value

which is returned by the read

function. The global variable

"ibcnt" holds the result of GPIB

reads and the global variable

"bytes" holds the result of VXI

word serial reads.

Variable Type: Long

NOTE: If the actual number of bytes read is less

than the number of bytes specified in

number of bytes to read then the output

buffer is probably empty. If the number of

bytes read is 0 and the instrument returns

a 231 error code the read function failed.

The most probable cause for the read to

fail is because there was no data available

at the instrument's output buffer.

Errors             :

0   Success

-1   Instrument ID out of range

231   Error reading from instrument

232   Instrument not initialized

_____________________________________________________________________________

Function Name      : wtk1395_refosc

C Syntax           : int wtk1395_refosc(Instrument_ID, Source);

int Instrument_ID;

int Source;

Purpose            : This function selects the source for the reference

oscillator.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Source

Description:   This control selects a source for

the reference oscillator.

Variable Type: Integer

Valid Range:   0 - Internal (default)

1 - External (TRIG IN BNC)

2 - ECL Trig 0

3 - ECL Trig 1

Errors             :         0   Success

-1   Instrument ID out of range

-2   Source out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_rescale

C Syntax           : int wtk1395_rescale(Instrument_ID, Trace_Name,

New_Size);

int Instrument_ID;

char *Trace_Name;

int New_Size;

Purpose            : This function performs the following operations:

- Copies the contents of the specified trace into a

new location.

- Horizontally scales the trace indices to the

specified number of points.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Trace Name

Description:   This control specifies the name of

the trace to be re-scaled.

Variable Type: String

New Size

Description:   This control specifies the number

of points in the re-scaled trace.

Variable Type: 0 to 32,767 (4095)

NOTES: The maximum size of a waveform is

(32,767 - Available Memory (Points In

Use)).

Waveforms should be limited to 4095 data

points to maintain compatibility with

all output modes.

Errors             :         0   Success

-1   Instrument ID out of range

-3   New Size parameter out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_reset

C Syntax           : int wtk1395_reset(Instrument_ID);

int Instrument_ID;

Purpose            : This function reconfigures all parameters to their

default state.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Errors             :         0   Success

-1   Instrument ID out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_sequence

C Syntax           : int wtk1395_sequence(Instrument_ID);

int Instrument_ID;

Purpose            : This function performs the following operations:

- Configures the source function mode as USER,

selecting user defined traces as source.

- Disables the SHAPE mode.

- Selects the currently defined sequence stored in

memory

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Errors             :         0   Success

-1   Instrument ID out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_settrig

C Syntax           : int wtk1395_settrig(Instrument_ID, Period);

int Instrument_ID;

double Period;

Purpose            : This function sets the period of an internal signal

source. The timer signal acts as a trigger when it is

the selected trigger source.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Period

Description:   This control specifies the period

of the internal trigger signal.

Variable Type: Double

Valid Range:   2E-7 to 1E4

Default Value: 1E-3

Errors             :         0   Success

-1   Instrument ID out of range

-2   Period out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_softtrig

C Syntax           : int wtk1395_softtrig(Instrument_ID);

int Instrument_ID;

Purpose            : This routine initiates an immediate trigger signal

synchronized with next clock pulse.

NOTES: The trigger clock can be adjusted by the

Set Trigger Rate function.

The trigger mode must be enabled

separately with the Enable Trigger

function before this function will

operate.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Errors             :         0   Success

-1   Instrument ID out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_stand

C Syntax           : int wtk1395_stand(Instrument_ID, Waveform_Type);

int Instrument_ID;

int Waveform_Type;

Purpose            : This routine performs the following operations:

- Configures the function mode as SHAPE, to use

standard waveforms as the source.

- Disables the USER function mode.

- Selects the standard waveform to use as the source.

- Sets the trace mode to continuous wave.

- Sets the source marker to SYNchronized, triggering

off the output waveforms zero-crossing.

- Enables the elliptic output filter.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Waveform Type

Description:   This control selects the waveform

to used as the source function.

For a more detailed explanation of

each waveform, see NOTE.

Variable Type: Integer

Valid Range:   0 - DC

1 - Square Wave

2 - HF Square

3 - Sinusoid

4 - Pos Haversine

5 - Neg Haversine

6 - Pos Ramp

7 - Neg Ramp

8 - Rand Noise

9 - Triangle

10 - Test

NOTE:  The following is a more detailed

description of the standard waveform

types.

DC - An unvarying signal with respect to

time with an amplitude of 0.

Square Wave - A square wave signal.

HF Square -  High frequency square wave.

This square wave has faster rise and

fall times than the standard square wave.

Its maximum frequency is 25MHz.

Sinusoid - A sinusoidal signal.

Pos Haversine - A positive haversine

signal.

Neg Haversine - A negative haversine

signal.

Pos Ramp - A positive ramp signal.

Neg Ramp - A negative ramp signal.

Rand Noise - Periodic random noise.

Triangle - A triangle wave signal.

Test - An alternating pattern of high and

low values. The values 4095 and 1 are

alternately written to every point in the

trace. This pattern is useful for testing

very low frequencies.

Errors             :         0   Success

-1   Instrument ID out of range

-2   Waveform Type out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_starttrig

C Syntax           : int wtk1395_starttrig(Instrument_ID, Source, Polarity,

Gate, Count_Cycles, Gate_Mode);

int Instrument_ID;

int Source;

int Polarity;

int Gate;

long Count_Cycles;

int Gate_Mode;

Purpose            : This routine performs the following operations

- Configures the instrument for triggered operation.

- Selects the polarity of the start trigger.

- Specifies the number (count) of event repetitions

initiated by the start trigger.

- Selects the source of the start trigger.

- Enables/disables the gate mode of operation.

- Selects synchronous or asynchronous gate mode.

NOTE:  The start trigger initiates an operation,

while the advance trigger advances to

subsequent functions within a sequence

structure.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Source

Description:   This control selects the source of

the start trigger.

Variable Type: Integer

Valid Range:   0 - Internal (default)

1 - External

2 - Chain

3 - EChain

4 - VXIbus Trigger Line 0

5 - VXIbus Trigger Line 1

6 - VXIbus Trigger Line 2

7 - VXIbus Trigger Line 3

8 - VXIbus Trigger Line 4

9 - VXIbus Trigger Line 5

10 - VXIbus Trigger Line 6

11 - VXIbus Trigger Line 7

Polarity

Description:   This control selects either the

positive or negative slope for

start triggering.

Variable Type: Integer

Valid Range:   0 - Negative Slope

1 - Positive Slope (default)

Gate

Description:   This control enables or disables

gated trigger operation.

Variable Type: Integer

Valid Range:   0 - Disable (default)

1 - Enable

Count Cycles

Description:   This control specifies the number

of times to cycle through a trace

after a trigger signal is

received.

Variable Type: Long

Valid Range:   1 to 1,048,575 (non-seq waveforms)

1 to 524,288 (seq waveforms)

Default Value: 1

Gate Mode

Description:   This control selects synchronous

or asynchronous gate operation.

Variable Type: Integer

Valid Range:   0 - Asynchronous

1 - Synchronous (default)

Errors             :         0   Success

-1   Instrument ID out of range

-2   Source out of range

-3   Polarity out of range

-4   Gate out of range

-5   Count Cycles out of range

-6   Gate Mode out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_sumin

C Syntax           : int wtk1395_sumin(Instrument_ID, Sumbus_Input,

Attenuation_dB);

int Instrument_ID;

int Sumbus_Input;

int Attenuation_dB;

Purpose            : This function perform the following operations:

- Enables the sumbus input terminals.

- Sets an attenuation value for the sumbus inputs on

the backplane.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Sumbus Input

Description:   This control enables/disables the

sumbus inputs on the backplane.

Variable Type: Integer

Valid Range:   0 - disable (default)

1 - enable

Attenuation (dB)

Description:   This control selects an

attenuation factor for the sumbus

inputs.

Variable Type: Integer

Valid Range:   0 - 0 dB

1 - 6 dB

2 - 12 dB

3 - 18 dB

4 - 24 dB

5 - 30 dB

6 - 36 dB

7 - 42 dB

Errors             :         0   Success

-1   Instrument ID out of range

-2   Sumbus Input out of range

-3   Attenuation (dB) out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_sumout

C Syntax           : int wtk1395_sumout(Instrument_ID, Sumbus_Out);

int Instrument_ID;

int Sumbus_Out;

Purpose            : This function configures the instrument to source

sumbus signals to the backplane.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Sumbus Out

Description:   This control enables/disables

the analog sumbus output.

Variable Type: Integer

Valid Range:   0 - Disabled (default)

1 - Enabled

Errors             :         0   Success

-1   Instrument ID out of range

-2   Sumbus-Out out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_sweep

C Syntax           : int wtk1395_sweep(Instrument_ID, Count, Direction, Mode,

Sweep_Time, Spacing, Start_Frequency,

Stop_Frequency);

int Instrument_ID;

double Count;

int Direction;

int Mode;

double Sweep_Time;

int Spacing;

double Start_Frequency;

double Stop_Frequency;

Purpose            : This routine performs the following operations:

- Configures the source function mode for sweep

operation.

- Sets the number of sweeps enabled by each

triggering event.

- Selects sweep direction.

- Selects the sweep mode.

- Sets the duration of the sweeps.

- Selects a linear or logarithmic

scale for sweeping.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Count

Description:   This control sets the number of

sweeps enabled by a triggering

event.

Variable Type: Double

Valid Range:   1 to 1e6

Default Value: 1.0

Direction

Description:    This control selects the direction

the sweep traverses from the start

and stop frequencies.

Variable Type: Integer

Valid Range:   0 - Down

1 - Up (default)

Mode

Description:   This control selects the sweep

characteristics - reset, reverse,

and trigger. See NOTE for further

details.

Variable Type: Integer

Valid Range:   Cont Reset - 0

Trig Reset - 1

Half Reset - 2

Cont Reverse - 3

Trig Reverse - 4

Half Reverse - 5

NOTE:  The following is a detailed description

of each sweep mode:

Cont Reset - Sweeps from the start

frequency to the stop frequency and

then returns to the start frequency

in a continuous loop.

Trig Reset - Waits for a trigger signal,

sweeps from the start frequency to the

stop frequency, and then resets to the

start frequency.

Half Reset - Waits for a trigger signal,

sweeps from the start frequency to

the stop frequency, and then waits for

another trigger before returning to the

start frequency.

Cont Reverse - Sweeps from the start

frequency to the stop frequency and then

sweeps back to the start frequency in a

continuous loop.

Trig Reverse - Waits for a trigger,

sweeps from the start frequency to the

stop frequency, and then sweeps back

to the start frequency.

Half Reverse - Waits for a trigger,

sweeps from the start frequency to

the stop frequency, and then waits for

another trigger before sweeping back to

the start frequency.

Sweep Time

Description:   This control selects the duration

of a single sweep (in seconds).

Variable Type: Double

Valid Range:   30e-3 to 1e3

Default Value: 1.0

Spacing

Description:   This control selects the time vs.

frequency characteristic, either

logarithmic or linear.

Variable Type: Integer

Valid Range:   0 - Logarithmic

1 - Linear (default)

Start Frequency

Description:   This control selects the frequency

at which the sweep operation

begins.

Variable Type: Double

Valid Range:   1E-3 to 50E6

Default Value: 1E3

Stop Frequency

Description:   This control selects the frequency

at which the sweep operation

ends.

Variable Type: Double

Valid Range:   1E-3 to 50E6

Default Value: 1E3

Errors             :         0   Success

-1   Instrument ID out of range

-2   Count out of range

-3   Direction out of range

-4   Mode out of range

-5   Sweep Time out of range

-6   Spacing out of range

-7   Start Frequency out of range

-8   Stop Frequency out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_test

C Syntax           : int wtk1395_test(Instrument_ID, Function, Test_Result);

int Instrument_ID;

int Function;

int *Test_Result;

Purpose            : This routine performs the following actions:

- Instructs the instrument to perform either a

destructive test on trace memory(RAM) or a

non-destructive test of its hardware systems.

- Returns a two-byte, bit-weighted error code in

hexadecimal format. (Detailed descriptions of the

error codes may be found in the instrument's

operation manual.) A zero indicates that no error

occurred.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Function

Description:   Chooses either a non-destructive

hardware systems test, or a

destructive trace memory(RAM) test

Variable Type: Integer

Valid Range:   0 -  RAM Test

1 -  System  (default)

NOTE: All contents of trace memory will be erased

if RAM test is performed.

Test Result

Description:   Displays the bit-weighted error

code from the instrument hardware

test in hexadecimal format. A

zero indicates that no error

occurred. Error codes can be found

in the instrument operation

manual.

Variable Type: Integer

Valid Range:   -32768 to 32767

Errors             :         0   Success

-1   Instrument ID out of range

-2   Function parameter out of range

226   Error polling instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_trac

C Syntax           : int wtk1395_trac(Instrument_ID, Trace_Name);

int Instrument_ID;

char *Trace_Name;

Purpose            : This function performs the following operations:

- Configures the source function mode as USER,

selecting user defined traces as source.

- Disables the SHAPE mode.

- Sets trace mode to RASTER.

- Sets source marker to SYNchronized and bit-marker

trigger qualification.

- Disables the output filter.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Trace Name

Description:   This control specifies the name of

the trace used as the source

function, and must be a

pre-defined trace residing in

trace memory.

Variable Type: String

Default Value: "trace1"

Errors             :         0   Success

-1   Instrument ID out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_trace_mode

C Syntax           : int wtk1395_trace_mode(Instrument_ID, Playback_Mode);

int Instrument_ID;

int Playback_Mode;

Purpose            : Selects the trace playback mode for either raster or

continuous playback.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Playback Mode

Description:   Specifies the trace playback mode

for the instrument.

Variable Type: Integer

Valid Range:   0 - Raster

1 - Continuous Wave (default)

(See descriptions below)

Playback Mode Descriptions:

Continuous Wave:  a fixed 50 MHz scan rate is

used and phase accumulation

is used for frequency

control; useful primarily

for playing back standard

functions like sinusoid,

triangle, and square

Raster:  scan rate can be adjusted by the

selected reference oscillator; used

to play back arbitrary waveforms

which typically define a complex

pattern of amplitude versus time

Errors             :         0   Success

-1   Instrument ID out of range

-2   Playback Mode out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_tradir

C Syntax           : int wtk1395_tradir(Instrument_ID, Function, Directory);

int Instrument_ID;

int Function;

char *Directory;

Purpose            : This function returns a string containing the names,

sizes, and limits of all defined traces.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Function

Description:   This control selects the output of

the Directory function as either

the names of the traces in trace

memory; or the names, sizes, and

start and stop frequencies.

Variable Type: Integer

Valid Range:   0 - Names only (default)

1 - All Info (names, sizes, start

and stop frequencies)

Directory

Description:   This control stores a string

containing the names, sizes, and

limits of all defined traces in

the format

<name1, size1, start1, stop1;

name2, size2, start2, stop2 ...

stopn>

If the function is set to Names

Only, then the format of the

output string is as follows

<name1, name2, name3,...namen>

Variable Type: String

Errors             :         0   Success

-1   Instrument ID out of range

-2   Function out of range

226   Error polling instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_trigen

C Syntax           : int wtk1395_trigen(Instrument_ID, Trigger_Mode);

int Instrument_ID;

int Trigger_Mode;

Purpose            : This routine selects between a continuous or triggered

mode of operation.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Trigger Mode

Description:   This control enables or disables

triggering. Disabling triggering

configures the instrument for

continuous mode of operation.

Variable Type: Integer

Valid Range:   0 - Disable (default)

1 - Enable

Errors             :         0   Success

-1   Instrument ID out of range

-2   Trigger Mode out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_write

C Syntax           : int wtk1395_write(Instrument_ID, Write_Buffer);

int Instrument_ID;

char *Write_Buffer;

Purpose            : Writes commands and queries to the instrument to

modify parameters and query device settings.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Write Buffer

Description:   The user can use this control to

send common commands and queries

to the instrument. This control

can also be used to write any

valid command to the instrument.

Variable Type: String

Default Value: "*RST"

NOTES: (1) The command or query to be sent to the

instrument may be a literal enclosed

in double quotes i.e. "*RST" or may be

contained in a variable of type

string.

(2) If the number of bytes to be written

is greater than the size of "cmd" in

the source code then the size of cmd

must be increased. The original size

of cmd is 50 bytes (cmd[50]) the

largest cmd may be declared as is

32,767 bytes.

Errors             :

0   Success

-1   Instrument ID out of range

230   Error writing to instrument

232   Instrument not initialized

_____________________________________________________________________________

Function Name      : wtk1395_writeA24

C Syntax           : int wtk1395_writeA24(Instrument_ID, File_Name);

int Instrument_ID;

char *File_Name;

Purpose            : This function writes the contents of a file (trace

data) to the A24 shared memory space in the VXI

chassis. The data can be downloaded into an

instrument on the chassis with the Write A24 to Trace

Memory or Get A24 Shared Data functions.

Note:          This function does not work for

GPIB-VXI; use Write File to Trace.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

File Name

Description:   The name of the file into which

the trace is written.

Variable Type: String

Default Value: "trace1"

Errors             :         0   Success

-1   Instrument ID out of range

101   Device not found

102   File not present in specified path

103   Error opening specified file

105   Function not valid for GPIB Interface

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

235   Error writing to A24 Address Space

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_writefile

C Syntax           : int wtk1395_writefile(Instrument_ID, Trace_Name,

File_Name);

int Instrument_ID;

char *Trace_Name;

char *File_Name;

Purpose            : This function writes a single trace to a file. If the

file does not exist, it is created.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Trace Name

Description:   The name of the trace in trace

memory to be written.

Variable Type: String

Default Value: "trace1"

File Name

Description:   The name of the file into which

the trace is written.

Variable Type: String

Default Value: "trace1"

Errors             :         0   Success

-1   Instrument ID out of range

226   Error polling instrument

229   Error reading from instrument to file

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.

_____________________________________________________________________________

Function Name      : wtk1395_writetrace

C Syntax           : int wtk1395_writetrace(Instrument_ID, Trace_Name,

Existing_Trace, Trace_Size);

int Instrument_ID;

char *Trace_Name;

int Existing_Trace;

int Trace_Size;

Purpose            : This function reads the specified number of bytes from

the A24 memory space and writes into the trace memory.

Note:          This function does not operate

under the GPIB_VXI platform,

use the Write File to Trace

function.

Parameter List     : Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1  to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

instruments that can be controlled

by this driver in a single system.

The value is set internally in the

driver. The typical value is 10.

Trace Name

Description:   The name of the trace into which

the data is written.

Variable Type: String

Default Value: "trace1"

Existing Trace?

Description:   Specifies whether the trace to be

written exists in trace memory or

must be created.

Variable Type: Integer

Valid Range:   0 = No (default)

1 = Yes

Trace Size

Description:   Specifies the number of points to

be written to the trace.

Variable Type: Integer

Valid Range:   1 to 32767

Default Value: 4096

Errors             :         0   Success

-1   Instrument ID out of range

-3   Existing Trace out of range

-4   Trace Size out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

300   Instrument Dependent Error - Execute

the Read Error Queue function for

more information.