PRACTICE EXERCISE

1.	What type of gears are used in the manual transmission to make it quieter?

a.  Bevel

b.  Straight spur

c.  Helical

2.	Most manual transmission gear cases are constructed of

a.  cold-rolled steel.

b.  cast iron.

c.  forged steel.

3.	What part of the conventional transmission prevents shifting into two gears at the same time?

a.  Interlock device

b.  Poppet ball

c.  Synchronizer

4.	If the transmission is quiet in neutral with the clutch engaged, but noisy when the clutch is disengaged, the trouble is probably in the

a.  synchronizer clutch.

b.  drive gear.

c.  release bearing.

5.	A vehicle should be parked several hours before checking the transmission oil level to permit the

a.  oil to cool and shrink.

b.  foam to cool and settle.

c.  pressure to escape.

ANSWERS TO PRACTICE EXERCISE

1.	c	(page 48)

2.	b	(page 35)

3.	a	(page 39)

4.	c	(page 57)

5.	b	(page 55)