APPENDIX D

AIRCRAFT SURVIVABILITY

# SECTION I.  FUNDAMENTALS AND THREAT CONSIDERATIONS

D-1.	FUNDAMENTALS OF AIRCRAFT SURVIVABILITY

ASE protects tactical helicopters operating throughout the battlefield while conducting their assigned missions.  Aircraft survivability encompasses a vast array of disciplines.  ASE does not equate to the whole of aircraft survivability.  ASE is a portion of EW, which is but one pillar that supports IO/IW.  FM 3-13 (FM 100-6) changed EW terminology to three functions:  electronic attack, electronic protection, and electronic support.

- Electronic Attack.  EA (formerly ECM) is that division of EW involving the use of electromagnetic or directed energy.  EA is used to attack personnel, facilities, and equipment.  Its intent is to degrade, neutralize, or destroy enemy combat capability.  EA includes actions taken to prevent or reduce the enemy’s effective use of the electromagnetic spectrum through jamming, destruction, and electromagnetic deception.  EA also includes the employment of weapons using either electromagnetic or directed energy; for example, using lasers, radio frequency, and particle beams as their primary destructive mechanism or using sources of electromagnetic energy as the primary means of terminal weapons guidance to damage or destroy personnel, facilities, or equipment.  ASE systems include chaff, flares, radar jamming, and infrared jamming.

- Electronic Protection.  EP (formerly ECCM) is part of EW.  EP involves those actions taken to protect personnel, facilities, and equipment from the effects of friendly or enemy EW actions that may degrade, neutralize, or destroy friendly combat capability.  To minimize its vulnerability to electronic attack, EP should be considered for all battlefield systems deriving operational capabilities from the electromagnetic spectrum.  Included are optical, electronic, infrared, and radar target acquisition and NCTR systems, as well as smart weapons systems’ sensors, fuses, guidance, and control components.  ASE systems include antenna design, signature reduction, and infrared-absorbing paint.

- Electronic Support.  ES (formerly ESM) is that part of EW involving actions tasked by, or under the direct control of, an operational commander.  The purpose of ES is to search for, intercept, identify, and locate sources of radiated electromagnetic energy for immediate threat recognition.  ES supports EW operations and other tactical actions such as threat avoidance, homing, and targeting.  ES focuses on surveillance of the electromagnetic spectrum to support the commander’s immediate decision-making requirements for the employment of EW or other tactical actions.  These activities may include threat avoidance, targeting, or homing.  ES is normally provided by organic intelligence and sensing devices based on EW technology integrated into other weapon systems.  ES may also be provided by assets from other echelons that can provide combat information to the supported command.  ES ensures that EA and EP applications receive the input needed to operate effectively.  Examples of ES actions are battlefield systems executing direction-finding operations, detecting and identifying enemy missions or other electromagnetically-measured signatures for immediate exploitation, locating high-value targets for electronic attack, and providing threat-avoidance information.  ASE systems include radar, laser, and infrared missile-detecting sets.

D-2.	TENETS

ASE reduces aircraft vulnerability, thus allowing the aircrew to accomplish its immediate mission and to survive to fight another day.  ASE tenets support the methodology to achieving survivability.  ASE tenets are a five-fold approach to ensuring that Army aircrews are able to accomplish their mission again and again.  These five tenets include the following—in order of least cost and most effective to the greatest cost and least effective.

- Tactics (EP).  Proper tactics reduce exposure times to enemy weapons.  Nap-of-the-earth flight not only limits LOS exposure times but also places the aircraft’s radar, infrared, and optical signature in a cluttered environment.  NOE tactics, combined with ASE protection and standoff ranges, allow Army aviation to not only survive but also to perform its mission on the battlefield.  ASE protection is severely degraded when the aircraft is not flown tactically sound—such as against a blue-sky background.

- Signature Reduction (EP).  These measures are taken into account by engineering or design changes such as flat canopies, exhaust suppressers, and coating the aircraft with low-infrared reflective paint.  Signature reduction alone greatly increases survivability.  Without signature reduction, ASE effectiveness is degraded or, in some cases, erased.  The aviator chooses how much signature to expose to the threat, thereby also performing signature control.

- Warning (ES).  The next step in ASE protection is to warn aircrews when they are about to be engaged, allowing them time to react.  Radar and laser-detecting sets and infrared missile warning systems are examples.

- Jamming and Decoying (EA).  When aircrews must stay on station, despite warnings, countermeasures are required.  Countermeasures that can jam/decoy the fire control or guidance systems of threat weapons include chaff, flares, and radar and IR jammers.

- Aircraft Hardening (Vulnerability Reduction).  Aircraft hardening provides for ballistic tolerance, redundant critical flight systems, and crashworthy features.  Aircraft hardening minimizes the damage to an aircraft after it has been hit.

- Sequence.  Sound tactical flight and signature reduction provide the baseline.  Warning leads to jamming; each tenet is sequential, starting from the most effective and least expensive to the least effective and most expensive.

D-3.	THREAT CONSIDERATIONS

This information on threat considerations provides a general knowledge of threat systems that can be applied to specific threats on a case-by-case basis.  This information is not system specific.

- Threat Engagement Sequence.  All weapon systems must complete a series of events, called an engagement sequence, to affect the target (aircraft).  Any missed step in the engagement sequence forces the threat to start over again.  Weapon systems sensors must—

- Detect.

- Acquire.

- Track.

- Launch and guide (or fire and ballistics).

- Assess damage.

- Example of Threat System.  The five elements required to compute an antiaircraft artillery fire-control solution are range, azimuth, elevation, velocity, and time of flight.  If one of the fire elements is incorrect, the AAA system will not hit the target.

- Time and Space.  The threat must detect, acquire, track (establish fire-control solution), and fire at the aircraft.  The TOF of the projectile must be determined.  The threat must predict where the aircraft target will be—within a few meters—when its ordnance travels to a point in space and time.

- Tools.  Tactics, signature reduction, warning, jamming, and decoys are available to preclude a successful threat engagement.  If hit, the aviator may have to count on aircraft hardening.

- Acquisition Versus Track.  The difference between detection and acquisition compared to tracking is important.  In detection and acquisition, the threat weapon system does not have refined data to fire at a target.  The threat weapon system must track the aircraft long enough to determine range, azimuth, elevation, and velocity to predict when and where to fire to hit its target.  Indications of search or acquisition activity may indicate, to the aircrew, time to increase its vigilance—such as changing mode of flight and actively searching for masking terrain features.  Tracking indications alert the aircrew to an immediate action requirement—masking or, when terrain is not readily available, ASE decoys and evasive maneuvers.

- Engagement Envelope.  All threat systems are confined by physics.  Each system has a maximum altitude and range in which its projectile will travel.  In addition, all threat systems have a minimum and maximum effective altitude and range.  These numbers are computed against a cooperative engagement—such as nonmaneuvering aircraft, blue-sky background, flat terrain, and steady velocity, if any.  The effective envelope for a threat system is based on a 50 percentile.  That is, at the maximum (or minimum) effective range (or altitude), the weapon system is able to hit the target one out of two times.  As the target gets further into the threat’s envelope, the probability of a first-shot kill increases.  As the target gets further outside of the threat envelope, the probability decreases until the target is outside the threat’s maximum range (or altitude), where the target is physically impossible to hit.

- Decreasing the Probability of Hit.  The aircrew can make the engagement more difficult for the threat.  A stationary target, for example, allows the threat to adjust each shot off the last until a shot hits the aircraft.  A more difficult engagement would be a moving, constant-velocity shot.  A prediction can be made, and if a miss occurs, an adjustment can be based on the last shot.  The most difficult engagement is against a moving target that varies range, altitude, elevation, and velocity.  Prediction is impossible because all four factors are changing at differing rates.

D-4.	THREAT WEAPON SENSORS

There are four major types of threat weapon sensors:  radar, infrared, laser and DEW, and optical/electro-optical.  These must be man portable or transportable by land, sea, or aerial platforms.  The actual sensor type and guidance package for each threat should be determined and its inherent capabilities and limitations understood.  (In-depth information about particular threat systems is available from the unit ASE/electronic warfare officer or TOO.)

- Radar.  Direct-threat radar weapons require LOS to hit the target.  Direct-threat radar weapons are either fire-controlled AAA or, for missile systems, controlled by command, SARH, active-radar homing, TVM, or GAS.  Radar weapons must detect, acquire, track, launch and guide (or fire a ballistic solution), and assess damage.  Radar systems have trouble with ground clutter.  To pick out targets from ground clutter, radar systems can detect movement using MTI, Doppler (continuous-wave radar), or pulse Doppler.  Some modern radar systems can and do track not only the movement of the aircraft itself but also the movement of rotor blades.  A few older radar systems had blind speeds—called a Doppler notch—where they could not detect an aircraft flying a specific speed towards or away from the radar.  However, modern radar systems cancel blind speeds.  Even with older radar systems, an aircraft had difficulty maintaining constant speed and angle to or from the one radar; it is impossible to be in the Doppler notch of more than one radar.  Radar systems can be detected, avoided, decoyed, jammed, and destroyed by direct and indirect fires—self, artillery, and antiradiation missiles.

- Infrared.  All infrared direct-threat weapons require LOS to be established before launch; the in-flight missile must maintain LOS with the target until impact or detonation of the proximity fuse.  IR missiles require the operator to visually detect the target and energize the seeker before the sensor acquires the target.  The operator must track the target with the seeker caged to the LOS until it is determined that the seeker is tracking the target and not background objects—such as natural or man-made objects to include vehicles, the sun, or reflected energy of the sun off clouds.  The IR sensor is also susceptible to atmospheric conditions (haze or humidity), the signature of the aircraft and its background, flares, decoys, and jamming.  Generally, IR systems are difficult to—

- Detect before launch (passive sensor).

- Predict where they may be located (portability).

- Respond to (short time of flight after launch).

- Hard kill (requires shooting at an in-flight missile).

- Laser and Directed-Energy Weapons.  Laser/DEW weapons are two distinct categories—laser-guided or laser-aided weapons and pure laser/DEW weapons.  Laser-guided or laser-aided weapons use the laser for ranging, tracking, or guiding functions for conventional explosive missiles or projectiles.  Pure laser/DEW weapons use laser and other forms of DEW to inflict damage to the aircraft or its sensors (as a by-product, the aircrew’s eyes may be damaged).  Pure laser/DEW weapons are not required to burn a hole in the target to destroy it—although these weapons are reaching that capability.  Simply igniting fuel vapor near vents or burning through fuel lines is effective as well as glazing the cockpit glass so that the aircrew cannot see out.  Inherently, laser/DEW weapons are of short duration, hard to detect, extremely hard to decoy or jam, and hard to kill.  Fortunately, they must rely upon LOS and atmospheric conditions and have a somewhat short range.

- Optical/Electro-Optical.  Optical/EO sensors are used as either the primary or secondary sensor for all weapon systems.  They rely on LOS; however, they are—with very few exceptions—completely passive.  They are limited by human eyes, atmospheric conditions, distance, jitter, and in many cases, by darkness.  The optical/EO sensors are most difficult to detect, seldom can be decoyed, and can be jammed in the sense of obscurants but, when located, can be hard killed.

SECTION II.  AIRCRAFT SURVIVABILITY EQUIPMENT

## D-5.	  CATEGORIES

## ASE systems fall into three categories:  aircraft signature reduction, situational awareness, and active countermeasures.

- Aircraft Signature Reduction:  All tactical helicopters are painted with nonreflective IR-absorbing paint.  Reducing the aircraft exhaust-gas signature aids the effectiveness of the AN/ALQ-144A IR missile jammer by reducing missile lock-on ranges.  AH-64 aircraft have exhaust suppression called Black Hole, which reduces the IR signature and aids the effectiveness of the AN/ALQ-144A IR missile jammer.  The radar and IR signatures of tactical helicopters are least when viewed from the front.  The maximum IR signature is from the rear quadrants; the maximum radar signature is from the side aspects.  The aircrews can decrease the signature exposed to threats by changing the aircraft’s aspect.

- Situational Awareness:  All tactical helicopters are equipped with pulsed-wave RSDS—that is, AN/APR-39A(V)1—which alerts the aircrew to radar activity.  AH-64 aircraft have additional situational awareness provided by the AN/AVR-2/2A, which alerts aircrews to laser activity.  Aircrews use the cues from the RSDS to change modes of flight (contour to NOE) or increase vigilance (actively seek masking terrain features).

- Active Countermeasures:  ASE countermeasures are required when masking terrain is not available to buy time until the aircraft can maneuver to masking terrain or outside of threat range.  The AN/ALQ-144A(V)1 can jam IR threats.  The M-130 with chaff can decoy radar threats.

D-6.	AH-64 ASE SUITE

## The AH-64 is equipped with a suite of ASE (Figure D-1).  This suite protects the aircraft while it performs its unique missions.

Figure D-1.  AH-64 ASE suite.

## D-7.	SYSTEM DESCRIPTIONS

## The following paragraphs briefly describe each ASE system.  The information also covers configuration requirements that are available to optimize the ASE system.

- AN/APR-39A(V)1.  The AN/APR-39A(V)1 RSDS is an upgraded version of the AN/APR-39(V)1.  It uses a digital processor, alphanumeric symbology display, and synthetic voice warning to alert the aircrew to radar-directed AD threat systems.  It covers C-/D-band and E- through M-band pulsed-wave radar.  The theater-specific emitter identification database software is reprogrammable.

- AN/AVR-2/2A.  The AN/AVR-2/2A laser-detecting set is a passive laser-warning system.  It provides input to the AN/APR-39A(V)1 to detect laser energy.  The 2A version is also used as sensors for the MILES/AGES.  The system has a reprogrammable EID.

# AN/ALQ-144A(V)1.  The AN/ALQ-144A(V)1 (Figure D-2) countermeasure set is an active, continuously operating, omnidirectional, IR jammer system for helicopters.  It confuses or decoys threat IR missile systems.  On an aircraft that has been equipped with low-reflective paint and engine exhaust suppressers, the AN/ALQ-144A(V)1 CMS jams all known threat infrared missile systems  The system has specific JPN settings that must be set before flight.

- M-130.  The M-130 general-purpose dispenser dispenses chaff and flares.  The system is operated either manually or automatically through interface with other countermeasure systems.  The chaff protects against radar-directed antiaircraft weapon systems, while the flares protect against IR-directed missile systems.  When dispensing chaff, the M-130 reduces or eliminates the enemy’s ability to use radar-controlled antiaircraft weapons to hit and destroy aircraft.  When dispensing flares, the M-130 reduces or eliminates the enemy’s ability to use infrared-guided missiles to hit and destroy aircraft.  When the M-130 is set to dispense chaff, the electronic control module must be set with the program setting for the aircraft before flight.

## D-8.	CONFIGURATION SETTINGS

## Configuration settings for ASE are located on the classified multiservice electronic combat bulletin board system, sponsored by the ARAT, located at Eglin Air Force Base, Florida.  Connection to the MSEC-BBS requires an accredited computer, communications software, null modem cable, and a STU-III.  The MSEC-BBS must be contacted to ensure that each unit has the most current ASE settings for each theater of operations.

D-9.	TACTICAL OPERATIONS OFFICERS AND ASE/EWOS

For ASE to provide effective protection during a mission, configuration settings must be optimized for the threats encountered.  The TOO at the brigade and battalion/squadron staff assists the S3 in mission planning for aircraft survivability during the mission.  TOE places the TOO in the aviation company/troop as a CW3, in battalion/squadron operations as a CW4, and in the brigade/regiment as a CW5.  The TOO is identified by the SQI I such as 152DI.  The ASE/EW officer is a CW2 in the aviation company/troop.  The ASE/EW officer has the ASI of H3 (for example, 152D0H3).  The ASE/EW officer ensures that optimum ASE configuration settings are prepared for each flight.  DA Pamphlet 611-21 describes the TOO position as a warrant officer qualified to—

- Plan, schedule, coordinate, and brief tactical and nontactical missions.

- Operate the aviation mission planning system.

- Develop, plan, coordinate and brief electronic warfare operations.

- Manage flying-hour programs and ASE programs.

### SECTION III.  OPERATIONAL EMPLOYMENT CONSIDERATIONS

## D-10.	GENERAL

## Aircraft survivability functions (Figure D-3) must be included throughout mission planning, rehearsal, execution, and recovery operations.  Intelligence drives the operations.  Mission planning begins with the receipt of the situation and mission.  It continues through completion of mission execution and after-action review.  From receipt of the enemy situation and mission, aircraft survivability functions should be planned and implemented.

# Figure D-2.  AN/ALQ-144A setting.

## Figure D-3. Roles and functions.

## D-11.	MISSION PLANNING

## ASE and EW must be considered in all phases of mission planning.  The level of planning is always predicated on the time, information, and personnel available.  OPORDs for military operations are extensive in scope and contain information that acts as a baseline for most unit operations.

- OPORD.  The OPORD is generated upon receipt of the enemy and friendly situation, the mission, and the commander’s intent.  The EW annex (Figure D-4) supports the OPORD, using this information.  The enemy and friendly situations are further defined, emphasizing the EW capabilities that each has to find, fix, jam, deceive, disrupt, or destroy each other.  Once the situation is clearly defined, the mission is analyzed to evaluate the risk to friendly forces while accomplishing the mission within the prescribed guidelines.  After the risk assessment is complete, risk-reduction techniques are specified in the execution instructions.  These techniques require the commander’s approval if the mission constraints need to be altered significantly from the original intent.  The next step is to determine service support for EW and command and signal guidance needed for the EW phase of the mission.

- FRAGO.  Once the OPORD (and EW annex) is generated (Figure D-5), it becomes the base document.  For specific missions, complete OPORDs may not always be required.  In these instances, FRAGOs outlining the changes from the basic OPORD are created and issued to affected units.  Upon receipt of the FRAGO, staff planners must evaluate the information available and revalidate the EW annex.  Any changes to the EW annex must be detailed and disseminated to the aircrews as part of the mission briefing.

## D-12.	CONSIDERATIONS IN RISK MANAGEMENT

- Identify the Risk.  In a thorough risk assessment, detailed information about threat system operating procedures, tactics, system capabilities, and locations must be analyzed to determine the enemy’s EW advantages or disadvantages.  The capabilities and limitations of friendly EW systems must be compared to the threat’s to assess the mission’s level of risk.  The S2 and TOO identify the following:

- Operating frequencies of radar threats.

- RF threats that can or cannot be detected.

- RF threats that radar-jamming equipment will affect.

- RF threats that can be decoyed.

- IR threats that may be encountered.

- IR threats that can be detected.

- IR threats that can be jammed or decoyed.

- Laser/DEW threats that can or cannot be detected.

- Optical/electro-optical threats.

- Assess the Risk.  The S2 and TOO will prioritize the threat systems and optimize ASE settings for the highest priority threats.  The S2 and TOO determine the level of risk based on the threat’s capabilities and limitations, the capabilities and limitations of the ASE, and the mission (Figure D-6).  They use the highest risk to determine the overall risk to the mission.  If the risk from IR threats is high, then the overall mission risk would continue to be high risk.  The risk assessment worksheet (Figure D-7) is used to determine what is causing the highest risks so that controls can be developed to reduce those risks.

SECURITY CLASSIFICATION

ISSUING HEADQUARTERS

LOCATION

DAY, MONTH, YEAR, HOUR, ZONE

ANNEX I (ELECTRONIC WARFARE) TO OPORD XXXX-XX (U)

( )  References:  List basic documents required.

1.  ( ) Situation

- ( )  Enemy.  Refer to Annex B.  Provide an estimate of the enemy’s communications, noncommunications, and EW systems capabilities, limitations, and vulnerabilities including the ability to interfere with the accomplishment of the EW mission stated herein.  Determine the ability to detect radar altimeter, Doppler, FM, VHF, and UHF communications and the ability to interrogate transponder for modes 1, 2, 3A, and 3C.  Determine air defense EW systems and analyze parameters (for example, frequencies, PRF, PRI, scan type, and wavelength) for use in risk analysis.

- ( )  Friendly.  Provide a list of friendly EW systems available for the mission (for example, communications, noncommunications, navigation, sensors, countermeasures, and electro-optical systems).  Include friendly EW assets that can exploit and disrupt the enemy’s usage of the electromagnetic spectrum.

- ( )  Assumptions.  State any assumptions about friendly or enemy EW capabilities and possible courses of action that may influence the planning or execution of EW operations.

2.  ( )  Mission.  State the mission to be accomplished by EW operations to support the mission in the basic plan.

3.  ( )  Execution

- ( )  Concept of Operations.  Summarize the scope of EW operations and the methods and resources to be used.  Include TTPs for the threats that may be encountered.

- ( )  Tasks.  In separate subparagraphs, assign individual tasks to EWOs and crews including instructions and references.

- ( )  Coordinating Instructions.  Place instructions applicable to two or more subunits in the final subparagraph.

- ( )  Guiding principles.  State or refer to policies, doctrine, tactics, techniques, and procedures that provide guidance to be followed.  Establish any additional guidance and authorized deviations from standardized practices.  Describe any constraints that may apply to the mission.

- ( )  Special measures.  Provide any special procedure to be used that is not provided elsewhere.

4.  ( )  Support.  Specify support units to provide EW service support.  Include verification of threat parameters and ASE settings through the ARAT.

Figure D-4.  Suggested format for an electronic warfare annex to an operations order.

5. ( ) Command and Signal. Provide such information as IFF mode settings and mode activation/deactivation line, ASE configuration settings, Have Quick settings, SINCGARS settings, A2C2 frequencies, AWACS contact points, and brevity codes. Acknowledge: Name (commander’s last name) Rank (commander’s rank) OFFICIAL: APPENDICES: DISTRIBUTION: (SECURITY CLASSIFICATION) (This example of an EW appendix is unclassified. An actual annex should show proper classification markings for each paragraph.)

Figure D-4.  Suggested format for an electronic warfare annex to an operations order (concluded).

Figure D-5.  OPORD and FRAGO.

- Make Decisions and Develop Controls.  The S2 and TOO will determine the optimum ASE configuration settings for each aircraft type and the threats in the mission area.

- Threats that are highly lethal and not countered by ASE are identified.  The PIR can then be developed and submitted by the S2 to higher headquarters (for example, the SA-X is lethal and no organic countermeasures are present.  This threat poses a high risk to mission accomplishment.  Where is the SA-X located in the area of operations?  The latest time of value for this information is XXXX hours.).

- Risk-reduction techniques are applied to minimize the risk and enhance the probability of survival.  Risk-reduction measures include the following:

- Plan mission time earlier or later to take advantage of night operations.

- Use only suppressed aircraft for the higher risk portions of the mission.

- Request escort aircraft to suppress threats.

- Plan SEAD at critical points to reduce vulnerability.

- Prepare LZ/PZ with indirect fires.

- Alter flight routes to avoid known AD areas.

- Include the deception plan in the false insertion.

- Reduce electronic signature (EMCON).

- Determine formation/sortie size.

- The ASE/EW mission briefing disseminates information and instructions to the aircrews before the mission begins.  The briefing will alert aircrews to the risks associated with the threats and the optimum ASE settings and review the tactics specific to the mission.  These tactics include evasive maneuvers, actions on contact, multiship breakup and reformation procedures, and rules of engagement for countermeasure weapons’ employment.  Figure D-6 contains a sample ASE/EW mission briefing to assist ASE/EWOs in completing this task.

- Implement Controls and Supervise.  Commanders and aircrews must take an active role in reducing risks by implementing controls and supervising their implementation.

- Commanders ensure that ASE/EW considerations and configuration settings are considered and briefed to all aircrews and maintenance personnel.

- During preflight checks, aircrews ensure that ASE configuration settings are correct.

- During the mission, aircrews ensure that IFF codes are activated and deactivated at proper times and locations during flight.

- During the after-action review, debriefings from aircrews are collected.

- Aircrews report ASE/EW problems—such as ambiguities, false alarms, equipment failures, or shortcomings—to higher headquarters.

- Aircrews collect data and ensure that they are input to AMPS for the next mission (such as threat data, countermeasure responses, locations of false alarms, or friendly systems reported as threat).

SECTION IV.  MISSION EXECUTION

## D-13.	MISSION EXECUTION

## During the mission, aircrews should be familiar with the ASE situational awareness displays and expected threat indications.  Some actions must be performed without delay.  With the visual indications of a gun or missile fired at the aircraft or ASE indications of radar track or launch, the aircrew has only seconds to prevent the aircraft from being engaged.

- Reacting to Threat Engagements.  There are three distinct parts of reacting to threat engagements:

- Indication (determine immediate actions).

- Evasive maneuver (when masking terrain is not readily available).

- Actions on contact (decision to continue or abort mission).

- Crew Coordination.  To perform evasive maneuvers, crew members must rehearse crew coordination.  Standardized terminology such as “Missile three o’clock, break right” and “Breaking right” should be used to avoid confusion.  At other times, such as radar search or acquisition, indications do not require evasive maneuvering.

- Multiship Considerations.  Formations and spacing intervals should be selected that provide all aircraft with maneuver space to evade hostile fire.  Standardized terminology—such as chalk two breaking right missile or chalk three tracers three o’clock breaking left—should be used.  Briefings should include evasive formation breakup procedures and how to reform the formation after breaking the engagement.  An aircraft’s ASE indications must be communicated to other aircraft in the formation because it may be the only aircraft receiving the indications because of terrain, narrow radar beam, altitude, or maintenance problems.

## D-14.	CONCLUSION

## Survivability for Army aviation on the modern battlefield and in SASO requires extensive coordination with other staffs.  Because Army aviation can cover broad spaces at high speeds, coordination for airspace and fire-control measures is paramount.  The TOO and ASE/EWO are trained to incorporate ASE/EW considerations into mission planning and execution.  ASE is only effective if configured properly and used with tactics to counter the threat’s capabilities.  Army aviation must plan to make maximum use of the electromagnetic spectrum and fully exploit the weaknesses of the threat’s EW capabilities.

# ASE/EW BRIEFING FORMAT

OVERALL RISK: Low Medium High

CAUSED BY: Mission profile ASE suite Threat

ASE and IFF configuration settings:

ASE can detect:

ASE cannot detect:

ASE can jam:

ASE cannot jam:

Primary threats: IR RF EO Laser/DEW

Risk-reduction measures:

Changes to standard TTPs:

QUESTIONS:

Figure D-6.  Suggested format for an ASE mission brief.

Figure D-7.  ASE risk assessment worksheet.