MONITOR A PLATOON EARLY

WARNING SYSTEM AN/TRS-2

071-331-1002

CONDITIONS

Given an operational platoon early warning system (PEWS) with detectors in place and a copy of the installation sketch.

STANDARDS

Identify intruders as to location, type of intruder (personnel or vehicle), and direction of movement, when possible.

TRAINING AND EVALUATION

Training Information Outline

1.	Detect an intruder (DSPL position).

a.	When the receiver is first turned on, an │8.│8.│8.│ appears in the display window and the tone sounds for about 10 seconds. At the end of the 10 seconds, the display should clear and the tone should stop.

b.	When the detector senses an intruder, the message is sent to the receiver. The detector ID number appears on the receiver display.

c.	The ID number is followed by the intruder classification (P for personnel or C for vehicles).

d.	At the same time the intruder message is seen in the display, the tone is heard in the headset.

e.	As the intruder moves out of the detector’s range, the detector stops transmitting.

f.	Even though the detector is no longer transmitting, the memory in the receiver continues to display the message from the detector that was activated.

g.	When new messages from other detectors are received, they are momentarily displayed with decimal points (│1. │0. │P.│) and tone.

h.	When no new messages are being received, previously received (old) messages (without decimal points) are repeated in the display window in rotation, starting with the lowest ID number. (For example: │  │4 │P │, │  │6 │P │,  │  │8 │P │.)

i.	If both personnel and vehicles are detected, they are displayed and repeated in rotation, again starting with the lowest ID number. (For example: │  │2 │P │,  │  │3 │P │,   │  │10│C │, │  │12│P │,  │  │13│C│.)

j.	Turning the DSPL-TONE-OFF switch to TONE makes the display go blank, but does not erase the receiver memory.

k.	Pressing the TEST-RESET button on the receiver erases the memory and clears the receiver. This action allows the operator to determine which detectors are still being set off by intruders.

Note:	If the same ID numbers (with decimals and audio alert) are displayed, more than one intruder may be near the detector.

2.	Detect an intruder (TONE position).

a.	Operation in the TONE position is the same as in the DSPL position, except that no display appears in the window.

b.	If the tone is heard in the headset, the DSPL-TONE-OFF switch should be turned to the DSPL position.

c.	The ID number and classification of the intruder appear on the display window.

d.	Any messages stored in the receiver memory are also displayed in rotation.

Note:	Switching from DSPL to TONE or TONE to DSPL does not erase the receiver’s memory.

3.	Identify the reporting detector.

a.	After noting the ID number(s) of the detector(s) reporting, refer to the installation sketch to find the area and location of the detector and the intruder.

b.	Use the installation sketch. Note which detectors are reporting. It may be possible to determine the movements of a large number of intruders.

Note:	When detectors are reporting at a high rate (many intruders), there is a greater chance of false-alarm messages.

EVALUATION PREPARATION

Setup: At the test site, provide the equipment and situation given in the task conditions statement. The detectors must be placed so that the soldier being tested cannot see them. Provide a vehicle and another soldier to activate the detectors.

Brief Soldier: Tell the soldier to monitor the receiver, report if a detector is activated, and report whether a person or vehicle has activated the detector.

EVALUATION GUIDE

Performance Measures	Results

1.	Turn on the receiver and wait for the display windows to clear.	P       F

2.	Identify the detector by the number being activated.	P       F

3.	Identify whether personnel or a vehicle activated the detector.	P       F

4.	Identify the difference between an old message and a new message.	P       F

Performance Measures	Results

5.	Note an activation of a detector with the receiver in the TONE

position. Identify whether the detector was activated by personnel

or by a vehicle. State the number of the detector activated.	P       F

6.	Identify the enemy’s direction of travel.	P       F

FEEDBACK

Score the soldier GO if all steps are passed. Score the soldier NO-GO if any steps are failed. If the soldier fails any steps, show what was done wrong and how to do it correctly.

REFERENCES

Required				Related

None					TM 11-5895-1047-10