CHAPTER 1

RISK MANAGEMENT

1-1.  BACKGROUND.  Leaders must develop techniques that will save resources.  Because the Army must be prepared to operate worldwide, the training mission has become increasingly demanding and so have the risks inherent in that mission.  This increase in risks requires leaders to minimize or balance risks with essential mission needs.

1-2.  DEFINITION.  Risk is the possibility of a loss.  The loss can be death, injury, property damage, or mission failure.  Risk management identifies risks associated with a particular operation and weighs these risks against the overall training value to be gained.  The three risk management basics are to--

a.  Accept no unnecessary risk.

b.  Accept necessary risks that produce a net Army benefit.

c.  Make risk decisions at the proper command level.

1-3.  RISK MANAGEMENT PROCESS.  The risk management process uses the following approach:

a.  Detect hazards and associated risks.  Look for risks in each phase of the training or operation.

b.  Assess the risk.  Ask these questions:

- What is the chance of a mishap?

- What degree of injury or equipment damage is possible?

NOTE: A low chance of a mishap and a high probability of minor injury equals low risk.  A low chance of a mishap and a high probability of a fatality equals high risk.

c.  Develop risk control alternates and make risk decisions.  Develop risk control alternatives and make risk decisions.  If you cannot eliminate the risk, then you must control it without sacrificing essential mission requirements.  You can control some risks by modifying task standards, operational procedures, training requirements, and maintenance standards.  Decisions take several forms:

- Selecting from available controls.

- Trading off mission elements against risk controls.

- Determining if controls reduce the risk to an acceptable level considering the mission benefits.

d.  Implement the risk control measures.  You must integrate procedures to control risks into plans, orders, standing operating procedures (SOPs), and training.  You must also integrate them into other means to ensure that the risk reduction measures will be used during actual operations.

e.  Supervise the operations.  Leaders use the same supervision techniques such as on-the-scene spot-checks and performance indicators to monitor risk controls that they use to monitor overall operations.

f.  Evaluate the results.  Include the effectiveness of risk management controls when you assess the operational results.

1-4.  RISK ASSESSMENT ELEMENTS.  Assessing risks has no hard and fast rules.  Different training tasks involve different elements that can affect training safety.  However, seven elements are central to the safe completion of most driver training tasks:

- Soldier qualification

- Vehicle type

- Weather

- Terrain

- Supervision

- Equipment

- Time of day

Using matrices that assign a numerical value to each of the elements is one way to quickly appreciation the overall risks.  The following matrices are examples of risk assessments for the seven elements common to driver training missions.

NOTE:  The factors are arbitrarily weighted; modify them based on your particular mission and unit.

a.  Measure soldier qualification by comparing the level of task difficulty to the soldier's military driving experience.

| # SOLDIER QUALIFICATION RISK VALUE ### DRIVING EXPERIENCE |  |  |  |
| --- | --- | --- | --- |
| TASK | LICENSED OVER 1 YEAR | LICENSED UNDER 1 YEAR | UNLICENSED |
| COMPLEX | 3 | 4 | 5 |
| ROUTINE | 2 | 3 | 4 |
| SIMPLE | 1 | 2 | 3 |

EXAMPLE:  Unlicensed drivers learning to drive a five-speed manual transmission in an M813 would have a risk value of 5.

b.  Measure vehicle type by comparing the vehicle configuration to the location of the training tasks.

| # VEHICLE TYPE RISK VALUE ### VEHICLE CONFIGURATION |  |  |  |
| --- | --- | --- | --- |
| LOCATION OF TRAINING | LIGHT TRUCKS | MEDIUM TRUCKS | TRACTOR/ SEMITRAILERS |
| ROAD | 3 | 4 | 5 |
| TRAINING AREA | 2 | 3 | 4 |
| MOTOR POOL | 1 | 1 | 1 |

EXAMPLE:  Driving a 44-passenger bus over the road would have a risk value of 4 (the same as a medium truck).

c.  Measure weather by comparing temperature with moisture/ visibility conditions.

| # WEATHER RISK VALUE ### VISIBILITY/MOISTURE |  |  |  |
| --- | --- | --- | --- |
| TEMPERATURE FAHRENHEIT | CLEAR DRY | FOG/HUMID/ DRIZZLE | DUST/RAIN/ SNOW/ICE |
| 0-31˚ or 90˚+ | 3 | 4 | 5 |
| 32˚-59˚ | 2 | 3 | 5 |
| 60˚-89˚ | 1 | 3 | 5 |

EXAMPLE:  A task conducted outdoors at a temperature of 20˚F with snow or ice would have a risk value of 5.

d.  Measure terrain by comparing the physical features of the land with the road network that exists in the area.

| TERRAIN RISK VALUE ### TRAFFICABILITY |  |  |  |
| --- | --- | --- | --- |
| TYPE OF TERRAIN | STREETS/ HIGHWAYS | CONGESTED STREETS/HIGHWAYS | TRAILS CROSS-COUNTRY |
| JUNGLE/MOUNTAINS/ DESERTS | 3 | 4 | 5 |
| HILLS | 2 | 4 | ## 4 |
| FLAT/ROLLING | 1 | 3 | 3 |

EXAMPLE:  Driver training conducted at Fort Bragg over trails would have a risk value of 3.

e.  Measure supervision by comparing the instructor-to-student ratio to the training task location.

| SUPERVISION RISK VALUE |  |  |  |
| --- | --- | --- | --- |
| INSTRUCTOR-TO-STUDENT RATIO | CLASSROOM | TRAINING AREA/ MOTOR POOL | ON/OFF ROAD |
| LOW | 3 | 4 | 5 |
| AVERAGE | 2 | 3 | 4 |
| HIGH | 1 | 2 | 3 |

EXAMPLE:  An instructor to student ratio of 1:8 for on-road driving would have a risk value of 5.

f.  Measure equipment by comparing the equipment’s age to the maintenance level.

| # EQUIPMENT RISK VALUE |  |  |  |
| --- | --- | --- | --- |
| EQUIPMENT AGE | C-1 | C-2 | C-3 |
| OLD | 3 | 4 | 5 |
| AVERAGE | 2 | 3 | 4 |
| NEW | 1 | 2 | 3 |

EXAMPLE:  A 20-year-old M813, maintained as C-2, would have a risk value of 4.

g.  Measure time of day by comparing the level of light to familiarity with the route.

| # TIME OF DAY RISK VALUE |  |  |  |
| --- | --- | --- | --- |
| ROUTE FAMILIARITY | DAY | DAWN/DUSK | NIGHT |
| NEVER DRIVEN ROUTE | 3 | 4 | 5 |
| DRIVEN ROUTE 1-3 TIMES | 2 | 3 | 4 |
| FAMILIAR ROUTE | 1 | 2 | 3 |

EXAMPLE:  A driving task over a familiar route that starts during the day but ends at dusk would have a risk value of 2.

h.  After assessing all the risks, total the values.  Apply the sum to a quick-reference gauge.

| # QUICK REFERENCE GAUGE |  |  |  |
| --- | --- | --- | --- |
| RISK LEVEL | LOW RISK | CAUTION | HIGH RISK |
| VALUES | 7-12 | 13-18 | 19-35 |

When two or more elements have a risk value of 5, consider the overall rating as high risk.  (A risk value of 5 signifies hazards inherent in that task.  Analyzed the task for ways to reduce or eliminate the danger.)

1-5.  DECISION AID.  The level of the decision maker should correspond to the level of the risk.  The greater the risk, the more senior the final decision maker should be.  This matrix is a proposed decision aid to assist in determining the leadership decision-making level.

| # DECISION AID |  |  |
| --- | --- | --- |
| RISK | POINTS | DECISION LEVEL |
| LOW | 7-12 | SENIOR INSTRUCTOR |
| CAUTION | 13-18 | COMPANY COMMANDER |
| HIGH | 19-35 | BATTALION COMMANDER |

a.  Operations with a value of 7 to 12 are low risk, and normal SOPs apply.

b.  A value of 13 to 18 is a caution area that warrants complete unit command involvement.  Give a caution rating special consideration if one or two elements have significantly raised the overall risk level.  For example, a risk value of 4 in the weather element category indicates the soldiers are more susceptible to cold injuries and require closer supervision.  If you cannot lower the risk level, the company commander must approve the training mission.

c.  Operations with a value of 19 to 35 or that have two or more areas with a risk value of 5 are high risk.  Procedures in paragraph 1-5b apply.  If you cannot lower the risk level, the battalion commander must approve the mission.

1-6.  RISK CONTROL ALTERNATIVES.  The following options can help control risk:

a.  Eliminate the hazard totally, if possible, or substitute a less hazardous alternative.

b.  Reduce the magnitude of the hazard or provide barriers to control the hazard.

c.  Modify operational procedures to minimize risk exposure consistent with mission needs.

d.  Train and motivate personnel to act effectively to avoid hazards.

1-7.  RISK CONTROL MEASURES.  Leaders must monitor the training to ensure risk control measures are followed.  Never underestimate subordinates’ ability to sidetrack a decision they do not understand or support.  You must also monitor the impact of risk reduction procedures when they are implemented to see that they really work.  This is especially true of new and untested procedures.

1-8.  PAYOFFS.  Risk management lets you use realistic training scenarios without a high potential cost in accidents.  It also minimizes personnel and materiel losses in day-to-day training activities.

SAMPLE RISK ASSESSMENT WORK SHEET

TRAINING TASK: _________________________________________________________

POINTS:

__________1.  SOLDIER QUALIFICATION

| TASK | LICENSED OVER 1 YEAR | LICENSED UNDER 1 YEAR | UNLICENSED |
| --- | --- | --- | --- |
| COMPLEX | 3 | 4 | 5 |
| ROUTINE | 2 | 3 | 4 |
| SIMPLE | 1 | 2 | 3 |

__________2.  VEHICLE TYPE

| LOCATION OF TRAINING | LIGHT TRUCKS | MEDIUM TRUCKS | TRACTOR/ SEMITRAILERS |
| --- | --- | --- | --- |
| ROAD | 3 | 4 | 5 |
| TRAINING AREA | 2 | 3 | 4 |
| MOTOR POOL | 1 | 1 | 1 |

__________3.  WEATHER

| TEMPERATURE FAHRENHEIT | CLEAR/ DRY | FOG/HUMID/ DRIZZLE | DUST/RAIN/ SNOW/ICE |
| --- | --- | --- | --- |
| 0-31˚ OR 90˚+ | 3 | 4 | 5 |
| 32˚-59˚ | 2 | 3 | 5 |
| 60˚-89˚ | 1 | 3 | 5 |

________4.  TERRAIN

| TYPE OF TERRAIN | STREETS/ HIGHWAYS | CONGESTED STREETS/HIGHWAYS | TRAILS/ CROSS-COUNTRY |
| --- | --- | --- | --- |
| JUNGLE/MOUNTAINS/ DESERTS | 3 | 4 | 5 |
| HILLS | 2 | 4 | ## 4 |
| FLAT/ROLLING | 1 | 3 | 3 |

__________5.  SUPERVISION

| INSTRUCTOR-TO-STUDENT RATIO | CLASSROOM | TRAINING AREA/ MOTOR POOL | ON/OFF ROAD |
| --- | --- | --- | --- |
| LOW | 3 | 4 | 5 |
| AVERAGE | 2 | 3 | 4 |
| HIGH | 1 | 2 | 3 |

__________6.  EQUIPMENT

| EQUIPMENT AGE | C-1 | C-2 | C-3 |
| --- | --- | --- | --- |
| OLD | 3 | 4 | 5 |
| AVERAGE | 2 | 3 | 4 |
| NEW | 1 | 2 | 3 |

__________7.  TIME OF DAY

| ROUTE FAMILIARITY | DAY | DAWN/DUSK | NIGHT |
| --- | --- | --- | --- |
| NEVER DRIVEN ROUTE | 3 | 4 | 5 |
| DRIVEN ROUTE 1-3 TIMES | 2 | 3 | 4 |
| FAMILIAR ROUTE | 1 | 2 | 3 |

__________ TOTAL POINTS

| # QUICK REFERENCE GAUGE |  |  |  |
| --- | --- | --- | --- |
| RISK LEVEL | LOW RISK | CAUTION | HIGH RISK |
| VALUES | 7-12 | 13-18 | 19-35 |

| # DECISION AID |  |  |
| --- | --- | --- |
| RISK | POINTS | DECISION LEVEL |
| LOW | 7-12 | SENIOR INSTRUCTOR |
| CAUTION | 13-18 | COMPANY COMMANDER |
| HIGH | 19-35 | BATTALION COMMANDER |

APPROVED BY: ______________________________________ DATE: ______________