| #### CHAPTER 7 |  |
| --- | --- |
| TRUCK TRANSPORT OF PASSENGERS |  |
| SECTION I - TRAINING |  |
| LESSON TITLE: | TRANSPORT PASSENGERS IN A TRUCK |
| TASK NUMBER: | 551-721-1386 (Transport Passengers in a Truck) |

A.  TRAINING OBJECTIVE.

|  | TASK: | Transport passengers in a truck. |
| --- | --- | --- |
|  | CONDITION: | Given instruction, DD Form 1970, DA Form 2404, a pencil, vehicle operator's manual, equipment records folder, rags, lubricants, coolant, motor pool, driver training route, passengers, and a passenger-carrying wheeled vehicle with BII. |
|  | STANDARDS: | Without injury to passengers or damage to equipment, arrive at the destination. Observe all safety procedures and local traffic laws. Ensure the vehicle's passenger-carrying capacity is not exceeded. |

B.  INTERMEDIATE TRAINING.  None.

C.  ADMINISTRATIVE INSTRUCTIONS.

1.  Training time:  As scheduled.

2.  Training location:  Motor pool and driver training route as scheduled.

3.  Training type:  Conference, demonstration, and practical exercise.

4.  Students:  Scheduled personnel.

5.  Principal and assistant instructors required:  One primary instructor for the conference and one assistant instructor for every three students for the demonstration and practical exercise.

6.  Training aids and equipment:  Rags, lubricants, coolant, DA Form 2404, DD Form 1970, a pencil, vehicle operator's manual, equipment records folder, and a passenger-carrying wheeled vehicle with BII for every three students.

7.  References:  AR 385-55, FM 21-305, and appropriate vehicle operator's manual.

D.  SEQUENCE OF ACTIVITY.

NOTE:  Before attending this training, students must have successfully passed the EOCCT at Chapter 6.

1.  Introduction:

a.  Interest device.

b.  Tie-in.

c.  Lesson objective (paragraph A).

d.  Procedures:

(1)  Explanation.

(2)  Practical exercise.

(3)  Summary.

2.  Explanation and demonstration:

a.  Safety precautions:

(1)  Do not transport personnel in balanced (pintle-connected) trailers.

(2)  When transporting personnel in convoys, do not use the last vehicle in the convoy to transport passengers.

(3)  When more than one person (besides the driver) is transported in a cargo truck, there must be adequate fixed seating.

(4)  When the vehicle is in motion, all occupants must be seated.

(5)  The number of passengers transported in cargo vehicles in over-the-road service is restricted to the designated seating capacity.

(6)  Riding on loads or partial loads of non-hazardous material is permitted only when it would be dangerous to ride in the cab or front seat or when guards or servicing personnel are required.  Personnel will ride on loads only when--

(a)  loads are adequately secured.

(b)  passengers have enough room to keep their bodies entirely within the top and sides of the vehicle.

b.  Vehicle systems.  Before driving the vehicle, be sure it is safe.  Do a thorough PMCS.  Pay particular attention that the following items are in good working order:

(1)  Service brakes.

(2)  Parking brake.

(3)  Steering mechanism.

(4)  Lights and reflectors.

(5)  All tires (front wheels must not have recapped or regrooved tires).

(6)  Wheels and rims.

(7)  Horn.

(8)  Windshield wipers.

(9)  Rearview mirrors.

(10)  Seats.  The seats must be safe for passengers and must be securely fastened to the truck.

(11)  Cargo cover and end curtains.

(12)  Troop safety strap.

(13)  Tailgate.

(14)  Emergency equipment.  Each vehicle must have a fire extinguisher and a highway warning kit (three emergency reflectors or three warning triangles).

c.  Driver procedures:

(1)  Before loading passengers, ensure the transmission is placed in neutral (some automatics are placed in park); the parking brake is set; chock blocks (if required) are in place; the engine is shut off; and the ignition key is removed (if equipped).

(2)  Brief passengers on safety requirements.

(a)  Caution all passengers about unsafe positions.  (An unsafe position could be standing; attempting to ride between the cab and body; hanging on sides, running boards, or fenders; sitting on tailgates or sides of the truck; or extending arms or legs.)

(b)  Warn passengers not to jump from cargo beds and, after dismounting, to move away from traveled portions of the roadway.

(3)  Lower the tailgate.

(4)  Release the safety strap.

(5)  Help the passengers loading.

(6)  Be sure all passengers are seated.

(7)  Adjust the vehicle tarpaulin according to weather conditions.

### CAUTION

Always be sure there is adequate ventilation to prevent the accumulation of exhaust gases in the cargo compartment.

(8)  Secure the safety strap.

(9)  Secure the tailgate.

(10)  Drive to the destination, adhering to local traffic laws.  Because the load that is carried involves human lives, you must make driver adjustments to safeguard passengers.  This involves being extra careful.  Driving tips peculiar to transporting passengers follow.

(a)  Speed.

- Crashes result from excessive speed, often when rain or snow has made the road slippery.

- Every banked curve has a safe design speed.  In good weather, the posted speed is safe for cars, but it may be too high for many heavy vehicles.

- On a road with good traction, the vehicle may roll over if traveling too fast around a curve.

- On a road with poor traction, traveling too fast around a curve might cause the vehicle to slide off the curve.

- Reduce the vehicle speed for curves.  If the vehicle leans toward the outside on a banked curve, it is going too fast.

(b)  Railroad crossings.

- Stop the vehicle between 15 and 50 feet before railroad crossings.

- Listen and look in both directions for trains.

- After a train has passed and before crossing, be sure another train is not coming in the other direction on other tracks.

- If the vehicle has a manual transmission, do not change gears while crossing the tracks.

(c)  Other types of road crossings.  Slow down and carefully check for other vehicles at the following locations:

- Streetcar crossings.

- Railroad tracks used only for industrial switching within a business district.

- Locations where a policeman or flagman is directing traffic.

- A traffic signal showing green.

- Crossings marked exempt crossing.

(c)  Drawbridges.

- Stop at drawbridges that do not have a signal light or traffic control attendant.  Stop at least 50 feet before the draw of the bridge.

- Look to be sure the draw is completely closed before crossing.

- Slow down.  Be sure it is safe to cross when--

– a traffic light shows green.

– the bridge has an attendant or traffic officer that controls traffic whenever the bridge opens.

(11)  Lower the tailgate.

(12)  Release the safety strap.

(13)  Help passengers unloading.

(14)  Secure the safety strap and tailgate.

d.  Prohibited practices.

(1)  Avoid fueling a vehicle with passengers on board.  Never refuel in a closed building with passengers on board.

(2)  Do not talk with passengers or engage in any other distracting activity while driving.

(3)  Do not tow or push a disabled vehicle with passengers aboard either vehicle, unless getting off would be unsafe.  Only tow the vehicle to the nearest safe spot to discharge passengers.

e.  Demonstrate loading procedures to the students.

3.  Practical exercise:

a.  Assign students to vehicles.  Issue vehicle operator's manuals, pencils, DA Form 2404, DD Form 1970, passengers, and equipment records folder.  Tell students the location of rags, lubricants, and coolant.

b.  Students perform before-operation PMCS.

c.  Students practice transporting personnel from origin to destination.  During-operation PMCS is also conducted at this time.

NOTE:  As each student practices driving, an assistant instructor rides in the front seat next to the driver.  The other two students ride in the troop seats as passengers and rotate driving duties.  The assistant instructor explains driving techniques, ensures the driver is aware of driving situations, and conducts after-action reviews with each driver.  Now is the time to pass on valuable experience and correct any bad driving habits.

d.  Students perform after-operation PMCS and ensure all operator entries required on DA Form 2404 and DD Form 1970 are accurate, complete, and legible.

4.  Evaluation:  Check each student's performance of PMCS and transporting personnel.

5.  Summary:

a.  Recap main points.

b.  Allow for questions.

c.  Clarify questions.

d.  Give closing statement.

6.  Retraining:  Retrain No-Gos and slow learners.

E.  SAFETY RESTRICTIONS.

1.  Before loading or unloading passengers, ensure the transmission is placed in neutral (some automatics are placed in park); the parking brake is set; chock blocks (if required) are in place; the engine is shut off; and the ignition key is removed (if equipped).

2.  Ensure students remove all jewelry and ID tags before performing PMCS.

3.  Ensure students pay particular attention to the cautions and warnings listed in the operator's manual.

4.  Ensure ground guides are used when backing the vehicle.

5.  Ensure a safe following distance and speeds (as determined by the local command) are maintained when driving on the driver-training route.

6.  Ensure all occupants wear seat belts (if equipped) while vehicle is in operation.

7.  Ensure all occupants use hearing protection when required.

F.  ADDITIONAL COMMENTS AND INFORMATION.  Recommended instructional time is 7 hours (.5 conference, .5 demonstration, and 6.0 practical exercise, including 1.0 PMCS).

| SECTION II - TESTING |  |
| --- | --- |
| LESSON TITLE: | TRANSPORT PASSENGERS IN A TRUCK |
| TASK NUMBER: | 551-721-1386 (Transport Passengers in a Truck) |

A.  TRAINING OBJECTIVE.

|  | TASK: | Pass the written test. |
| --- | --- | --- |
|  | CONDITION: | In a classroom, given an examination booklet and a pencil. |
|  | STANDARDS: | Answer correctly 14 of 20 questions within 30 minutes. Use either the primary written test or the alternate written test. |

B.  INTERMEDIATE TRAINING.  None.

C.  ADMINISTRATIVE INSTRUCTIONS.

1.  Training time:  As scheduled.

2.  Training location:  Classroom as scheduled.

3.  Training type:  Test evaluation.

4.  Students:  Personnel as scheduled.

5.  Principal and assistant instructors required:  One instructor for every 20 students.

6.  Training aids and equipment:  Examination booklet and pencil.

7.  References:  AR 385-55, FM 21-305, and appropriate vehicle operator's manual.

D.  SEQUENCE OF ACTIVITY.

1.  Introduction:

a.  Interest device.

b.  Tie-in.

c.  Lesson objective (paragraph A).

d.  Procedures:

(1)  Explanation.

(2)  Test evaluation.

(3)  Summary.

2.  Explanation and demonstration:  Administer written examination.  Use either the primary or alternate test.

3.  Evaluation:  Check written test results.

4.  Summary:

a.  Recap main points.

b.  Allow for questions.

c.  Clarify questions.

d.  Give closing statement.

5.  Retraining:  Retrain and retest No-Gos.

NOTE:  Those students who passed this training and who must transport personnel in a bus must successfully complete the training in Chapter 8.

E.  SAFETY RESTRICTIONS.  None.

F.  ADDITIONAL COMMENTS AND INFORMATION.  Recommended testing time is 1 hour.

TRANSPORT PASSENGERS IN A TRUCK

WRITTEN TEST (PRIMARY)

| NAME |  | RANK | DATE |
| --- | --- | --- | --- |
| Multiple choice: Read each question carefully and write the answer which is most correct on the blank line to the left. |  |  |  |
| ______ 1. | When transporting personnel in a convoy, passengers-- a. May ride in the cargo bed of any truck. b. May ride in pintle-connected trailers. c. Will not be transported in the last vehicle of the convoy. d. Will not be transported in the first three vehicles in the convoy. |  |  |
| ______ 2. | When more than one person (other than the driver) is transported in a cargo truck, there must be-- a. Seat belts installed in the bed. b. Adequate fixed seating. c. A personnel heater installed in the bed. d. Steps fabricated for mounting and dismounting the bed area. |  |  |
| ______ 3. | The number of passengers transported in cargo vehicles in over-the-road service is restricted to-- a. 1/2 the designated seating capacity. b. 1/3 the designated seating capacity. c. 3/4 the designated seating capacity. d. the designated seating capacity. |  |  |
| ______ 4. | Front tires on passenger carrying vehicles must not be-- a. Regrooved or recapped. b. Radials. c. Bias-ply. d. New. |  |  |
| ______ 5. | Each passenger carrying vehicle must have-- a. A rotating amber warning light. b. A fire extinguisher and a highway warning kit (three emergency reflectors or three warning triangles). c. Two spare fuel cans. d. Three spare electrical fuses. |  |  |
| ______ 6. | A thorough before-operation PMCS of a passenger- carrying vehicle includes-- a. Ensuring all safety items are in good working order. b. Lubricating the vehicle. c. Washing the vehicle undercarriage to remove accumulated mud and grease. d. Draining the fuel filter. |  |  |
| ______ 7. | After loading passengers and before moving the vehicle, you should-- a. Release the safety strap. b. Lower the tailgate. c. Be sure there is adequate ventilation to prevent accumulation of exhaust gases in the cargo bed. d. All of the above. |  |  |
| ______ 8. | Troop seats must be-- a. Used in conjunction with seat belts. b. Folded when not in use. c. Made of rigid steel. d. Securely fastened to the truck. |  |  |
| ______ 9. | A passenger safety briefing includes-- a. Instructions on the use of cargo heaters. b. Instructions on the use of emergency flares. c. Warnings for passengers not to jump from cargo beds. d. Instructions on the use of seat belts installed in the cargo bed. |  |  |
| ______ 10. | When loading passengers, you should-- a. Lower the tailgate. b. Release the safety strap. c. Help the passengers loading. d. All of the above. |  |  |
| ______ 11. | On a road with poor traction, traveling too fast around a curve might cause-- a. The vehicle to slide off the curve. b. The vehicle to coast. c. The vehicle to stop abruptly. d. Nothing. |  |  |
| ______ 12. | Speed limits on curves are standardized for-- a. Buses. b. Cars. c. Tractor-trailers. d. Small, straight trucks. |  |  |
| ______ 13. | When stopping at a railroad crossing, you must stop-- a. No closer than 5 feet and no farther than 30 feet from the nearest track. b. No closer than 10 feet and no farther than 40 feet from the nearest track. c. No closer than 15 feet and no farther than 50 feet from the nearest track. d. No closer than 60 feet and no farther than 90 feet from the nearest track. |  |  |
| ______ 14. | You are approaching a drawbridge. No signal device or attendant is present. You must-- a. Stop at least 50 feet before the draw of the bridge. b. Stop no closer than 75 feet before the draw of the bridge. c. Not stop as that might cause an accident. d. Speed up and cross quickly before the bridge is opened. |  |  |
| ______ 15. | When is it best to wear your seat belt? a. Only when you will be driving over 35 mph. b. Only if the truck transports more than 16 people. c. Only when driving off post. d. Anytime the vehicle is in motion. |  |  |
| ______ 16. | With passengers loaded, you must never refuel your truck-- a. With a higher grade of fuel. b. In a closed building. c. Without a static chain. d. With any door open. |  |  |
| ______ 17. | On a road with good traction, traveling too fast around a curve may cause-- a. The vehicle to coast. b. The vehicle to stop abruptly. c. The vehicle to roll over. d. The vehicle to hydroplane. |  |  |
| ______ 18. | If the vehicle leans toward the outside on a banked curve, this means-- a. It is going too fast. b. It is going too slow. c. Nothing; this is normal. d. Nothing; you just downshifted. |  |  |
| ______ 19. | When crossing railroad tracks, do not-- a. Listen and look in both directions for trains. b. Check for another train coming in the other direction, on other tracks, before crossing. c. Change gears while crossing the tracks, if the vehicle has a manual transmission. d. Stop the vehicle before the railroad crossing. |  |  |
| ______ 20. | It is prohibited to-- a. Tow or push a disabled vehicle with passengers aboard either vehicle. b. Help passengers unloading. c. Caution passengers about unsafe positions such as standing or hanging on the sides. d. Warn passengers not to jump from cargo beds. |  |  |

TRANSPORTING PASSENGERS IN A TRUCK

WRITTEN TEST ANSWER SHEET (PRIMARY)

| 1. | C | 11. | A |
| --- | --- | --- | --- |
| 2. | B | 12. | B |
| 3. | D | 13. | C |
| 4. | A | 14. | A |
| 5. | B | 15. | D |
| 6. | A | 16. | B |
| 7. | C | 17. | C |
| 8. | D | 18. | A |
| 9 | C | 19. | C |
| 10. | D | 20. | A |

TRANSPORT PASSENGERS IN A TRUCK

WRITTEN TEST (ALTERNATE)

| NAME |  | RANK | DATE |
| --- | --- | --- | --- |
| Multiple choice: Read each question carefully and write the answer which is most correct on the blank line to the left. |  |  |  |
| ______ 1. | When is it best to wear your seat belt? a. Only when you will be driving over 35 mph. b. Only if the truck transports more than 16 people. c. Only when driving off post. d. Anytime the vehicle is in motion. |  |  |
| ______ 2. | You are approaching a drawbridge. No signal device or attendant is present. You must-- a. Stop at least 50 feet before the draw of the bridge. b. Stop no closer than 75 feet before the draw of the bridge. c. Not stop as that might cause an accident. d. Speed up and cross quickly before the bridge is opened. |  |  |
| ______ 3. | When stopping at a railroad crossing, you must stop-- a. No closer than 5 feet and no farther than 30 feet from the nearest track. b. No closer than 10 feet and no farther than 40 feet from the nearest track. c. No closer than 15 feet and no farther than 50 feet from the nearest track. d. No closer than 60 feet and no farther than 90 feet from the nearest track. |  |  |
| ______ 4. | When crossing railroad tracks, do not-- a. Listen and look in both directions for trains. b. Check for another train coming in the other direction, on other tracks, before crossing. c. Change gears while crossing the tracks, if the vehicle has a manual transmission. d. Stop the vehicle before the railroad crossing. |  |  |
| ______ 5. | On a road with good traction, traveling too fast around a curve may cause-- a. The vehicle to coast. b. The vehicle to stop abruptly. c. The vehicle to roll over. d. The vehicle to hydroplane. |  |  |
| ______ 6. | It is prohibited to-- a. Tow or push a disabled vehicle with passengers aboard either vehicle. b. Help passengers unloading. c. Caution passengers about unsafe positions such as standing or hanging on the sides. d. Warn passengers not to jump from cargo beds. |  |  |
| ______ 7. | If the vehicle leans toward the outside on a banked curve, this means-- a. It is going too fast. b. It is going too slow. c. Nothing; this is normal. d. Nothing; you just downshifted. |  |  |
| ______ 8. | On a road with poor traction, traveling too fast around a curve might cause-- a. The vehicle to slide off the curve. b. The vehicle to coast. c. The vehicle to stop abruptly. d. Nothing. |  |  |
| ______ 9. | A passenger safety briefing includes-- a. Instructions on the use of cargo heaters. b. Instructions on the use of emergency flares. c. Warnings for passengers not to jump from cargo beds. d. Instructions on the use of seat belts installed in the cargo bed. |  |  |
| ______ 10. | When loading passengers, you should-- a. Lower the tailgate. b. Release the safety strap. c. Help the passengers loading. d. All of the above. |  |  |
| ______ 11. | Troop seats must be-- a. Used in conjunction with seat belts. b. Folded when not in use. c. Made of rigid steel. d. Securely fastened to the truck. |  |  |
| ______ 12. | Speed limits on curves are standardized for-- a. Buses. b. Cars. c. Tractor-trailers. d. Small, straight trucks. |  |  |
| ______ 13. | The number of passengers transported in cargo vehicles in over-the-road service will be restricted to-- a. 1/2 the designated seating capacity. b. 1/3 the designated seating capacity. c. 3/4 the designated seating capacity. d. The designated seating capacity. |  |  |
| ______ 14. | When transporting personnel in a convoy, passengers-- a. May ride in the cargo bed of any truck. b. May ride in pintle-connected trailers. c. Will not be transported in the last vehicle of the convoy. d. Will not be transported in the first three vehicles in the convoy |  |  |
| ______ 15. | With passengers loaded, you must never refuel your truck-- a. With a higher grade of fuel. b. In a closed building. c. Without a static chain. With any door open. |  |  |
| ______ 16. | When more than one person (other than the driver) is transported in a cargo truck, there must be-- a. Seat belts installed in the bed. b. Adequate fixed seating. c. A personnel heater installed in the bed. d. Steps fabricated for mounting and dismounting the bed area. |  |  |
| ______ 17. | A thorough before-operation PMCS of a passenger- carrying vehicle includes-- a. Ensuring all safety items are in good working order. b. Lubricating the vehicle. c. Washing the vehicle undercarriage to remove accumulated mud and grease. d. Draining the fuel filter. |  |  |
| ______ 18. | Front tires on passenger carrying vehicles must not be-- a. Regrooved or recapped. b. Radials. c. Bias-ply. d. New. |  |  |
| ______ 19. | After loading passengers and before moving the vehicle, you should-- a. Release the safety strap. b. Lower the tailgate. c. Be sure there is adequate ventilation to prevent accumulation of exhaust gases in the cargo bed. d. All of the above. |  |  |
| ______ 20. | Each passenger carrying vehicle must have-- a. A rotating amber warning light. b. A fire extinguisher and a highway warning kit (three emergency reflectors or three warning triangles). c. Two spare fuel cans. d. Three spare electrical fuses. |  |  |

TRANSPORTING PASSENGERS IN A TRUCK

WRITTEN TEST ANSWER SHEET (ALTERNATE)

| 1. | D | 11. | D |
| --- | --- | --- | --- |
| 2. | A | 12. | B |
| 3. | C | 13. | D |
| 4. | C | 14. | C |
| 5. | C | 15. | B |
| 6. | A | 16. | B |
| 7. | A | 17. | A |
| 8. | A | 18. | A |
| 9 | C | 19. | C |
| 10. | D | 20. | B |