| #### CHAPTER 8 |  |
| --- | --- |
| BUS TRANSPORT OF PASSENGERS |  |
| SECTION I - TRAINING |  |
| LESSON TITLE: | TRANSPORT PASSENGERS IN A BUS |
| TASK NUMBER: | 551-721-1387 (Transport Passengers in a Bus) |

A.  TRAINING OBJECTIVE.

|  | TASK: | Transport passengers in a bus. |
| --- | --- | --- |
|  | CONDITION: | Given instruction, DD Form 1970, DA Form 2404, a pencil, vehicle operator's manual, equipment records folder, rags, lubricants, coolant, motor pool, driver training route, passengers, and a bus with BII. |
|  | STANDARDS: | Without injury to passengers or damage to equipment, arrive at the destination. Observe all safety procedures and local traffic laws. Ensure the vehicle passenger carrying capacity is not exceeded. |

B.  INTERMEDIATE TRAINING.  None.

C.  ADMINISTRATIVE INSTRUCTIONS.

1.  Training time:  As scheduled.

2.  Training location:  Motor pool and driver training route as scheduled.

3.  Training type:  Conference, demonstration, and practical exercise.

4.  Students:  Scheduled personnel.

5.  Principal and assistant instructors required:  One primary instructor for the conference and one assistant instructor for every three students for the demonstration and practical exercise.

6.  Training aids and equipment: Training aids and equipment:  Rags, lubricants, coolant, DA Form 2404, DD Form 1970, pencils, vehicle operator's manual, equipment records folder, and a bus with BII for every three students.

7.  References:  AR 385-55, FM 21-305, and appropriate vehicle operator's manual.

D.  SEQUENCE OF ACTIVITY.

NOTE:  Before attending this training, students must have successfully passed the EOCCT at Chapter 6.

1.  Introduction:

a.  Interest device.

b.  Tie-in.

c.  Lesson objective (paragraph A).

d.  Procedures:

(1)  Explanation.

(2)  Practical exercise.

(3)  Summary.

2.  Explanation and demonstration:

a.  Vehicle systems.  Before driving the vehicle, be sure it is safe.  Do a thorough before-operation PMCS.  Pay particular attention that the following items are in good working order:

(1)  Service brakes.

(2)  Parking brake.

(3)  Steering mechanism.

(4)  Lights and reflectors.

(5)  All tires (front wheels must not have recapped or re-grooved tires).

(6)  Wheels and rims.

(7)  Horn.

(8)  Windshield wipers.

(9)  Rearview mirrors.

(10)  Seats.  The seats must be safe for passengers and must be securely fastened to the bus.

(11)  Emergency equipment.  Each vehicle must have a fire extinguisher and a highway warning kit (three emergency reflectors or three warning triangles). The bus must also have spare electrical fuses unless equipped with circuit breakers.

(12)  Access doors and panels.  As you check the outside of the bus, close any open emergency exits.  Also close any open access panels (for baggage, rest room service, or engine) before driving.

(13)  Bus interior.  People sometimes damage unattended buses.  Always check the bus interior before driving to ensure rider safety.  Aisles and stairwells must always be clear.  The following parts of your bus must be in safe working condition:

(a)  Each handhold and railing.

(b)  Floor covering.

(c)  Signaling devices, including the rest room emergency buzzer, if the bus has a rest room.

(d)  Emergency exit handles.

(14)  Emergency exits.  Never drive with an open emergency exit door or window.  The emergency exit sign on an emergency door must be clearly visible.  If there is a red emergency door light, it must work.  Turn it on at night or any time you use your outside lights.

(15)  Roof hatches.  You may lock some emergency roof hatches in a partly open position for fresh air.  Do not leave them open as a regular practice.  Keep in mind the bus has higher clearance while driving with them open.

b.  Ensure passengers are properly loaded at the start of a trip.

(1)  Ensure the parking brake is set.

(2)  Stand at the entrance of the bus.

(3)  Caution passengers to board the bus cautiously.

(4)  Help those that need help negotiating the stairs.

(5)  Regulate the flow of passengers onto stairs to prevent overcrowding.

(6)  Follow the last passenger onto the bus.

c.  Ensure passengers are properly seated and luggage is properly stowed.  Baggage must be stowed to protect passengers from injury and to allow the driver to move freely and easily.

(1)  Walk the length of the aisle.  Check for obstructions.

(2)  Check the overhead luggage rack for stacked or loose items.

(3)  Check for proper placement of any odd-shaped baggage.

(4)  Check and secure with strapping any heavy objects.

(5)  Ensure riders can exit by any window or door in an emergency.

d.  Make pre-departure announcement to the passengers.

(1)  State your name.

(2)  Announce the final destination and stops along the way.

(3)  Remind passengers that it is safer to remain seated when traveling.

(4)  Inform passengers that the bus will stop for those departing.

(5)  Check that passengers are seated before moving the bus.

(6)  Check that standing passengers are behind the white line (in buses designed to allow standing).

e.  Cite the rules which passengers must follow in order to provide a safe comfortable environment.

(1)  Explain rules on smoking, eating, and drinking on board.

(2)  Explain rules on consumption of alcoholic beverages.

(3)  Explain rules on the use of radios and tape players.

(4)  Explain rules on keeping arms and heads inside the bus especially when the vehicle is moving.

f.  Pick up and discharge passengers en route to the final destination.

(1)  Activate turn signals and brake lights before stopping.

(2)  Announce the location and reason for stopping.

(3)  Request all passengers to remain seated until the bus has stopped.

(4)  Steer gradually and bring the bus to a smooth stop.

(5)  Open exit doors only after the bus has completely stopped.

(6)  Get out of the bus to help passengers who need help.

(7)  After reentering the bus, check to see if the departing passengers are clear of the bus before moving it.

g.  Discharge an unruly passenger from the bus.

(1)  Inform the passenger that he is not behaving properly.

(2)  Inform the passenger a second time.  Indicate that ejection is next.

(3)  Look for an appropriate location (safe and well lighted) to discharge the passenger.

(a)  On post.  Exchange complex or any other built-up area that has a telephone.

(b)  Off post.

- A bus terminal would be optimum.

- A rest area or service station would be adequate.

(4)  The bottom line is that the driver must ensure the safety of all passengers.

h.  Driving tips.  Drive to the destination, adhering to local traffic laws.  Some driving tips peculiar to transporting passengers follow.

(1)  Intersections.

(a)  Bus crashes most often happen at intersections.  Use caution, even if a signal or stop sign controls other traffic.

(b)  School and mass transit buses sometimes scrape off mirrors or hit passing vehicles when pulling out from a bus stop.  Remember the clearance your bus needs and watch for poles and tree limbs at stops.

(c)  Know the size of the gap the bus needs to accelerate and merge with traffic.  Wait for the gap to open before leaving the stop.  Never assume other drivers will brake to give you room when you signal or start to pull out.

(2)  Speed on curves.

(a)  Crashes result from excessive speed, often when rain or snow has made the road slippery.

(b)  Every banked curve has a safe design speed.  In good weather, the posted speed is safe for cars, but it may be too high for many buses.

(c)  On a road with good traction, the bus may roll over if traveling too fast around a curve.

(d)  On a road with poor traction, traveling too fast around a curve might cause the bus to slide off the curve.

(e)  Reduce speed for curves.  If the bus leans toward the outside on a banked curve, it is going too fast.

(3)  Railroad crossings.

(a)  Stop the bus between 15 and 50 feet before railroad crossings.

(b)  Listen and look in both directions for trains.  Open the forward door if it would improve your ability to hear or see an approaching train.

(c)  After a train has passed and before crossing, make sure another train is not coming in the other direction on other tracks.

(d)  If the bus has a manual transmission, do not change gears while crossing the tracks.

(4)  Other types of road crossings.  Slow down and carefully check for other vehicles at the following locations:

(a)  Streetcar crossings.

(b)  Railroad tracks used only for industrial switching within a business district.

(c)  Locations where a policeman or flagman is directing traffic.

(d)  Traffic signals showing green.

(e)  Crossings marked exempt crossing.

(5)  Drawbridges.

(a)  Stop at drawbridges that do not have a signal light or traffic control attendant.  Stop at least 50 feet before the draw of the bridge.

(b)  Look to be sure the draw is completely closed before crossing.

(c)  Slow down.  Be sure it is safe to cross when--

- there is a traffic light showing green.

- the bridge has an attendant or traffic officer that controls traffic whenever the bridge opens.

i.  Prohibited practices.

(1)  Avoid fueling a vehicle with passengers on board.  Never refuel in a closed building with passengers on board.

(2)  Do not talk with passengers or engage in any other distracting activity while driving.

(3)  Do not let hazardous materials on the bus.

(4)  Do not tow or push a disabled vehicle with passengers aboard either vehicle, unless getting off would be more risky for the passengers.  Only tow the vehicle to the nearest safe spot to discharge passengers.

j.  Demonstrate loading procedures to the students.

3.  Practical exercise:

a.  Assign students to vehicles.  Issue vehicle operator's manuals, pencils, DA Form 2404, DD Form 1970, passengers, and equipment records folder.  Tell students the location of rags, lubricants, and coolant.

b.  Students perform before-operation PMCS.

c.  Students practice transporting personnel from origin to destination.  During-operation PMCS is also conducted at this time.

NOTE:  As each student practices driving, an assistant instructor rides in a front seat as close to the driver as possible.  The other two students ride as passengers and rotate driving duties.  The assistant instructor explains driving techniques, ensures the driver is aware of driving situations, and conducts after-action reviews with each driver.  Now is the time to pass on valuable experience and correct any bad driving habits.

d.  Students perform after-operation PMCS and ensure all operator entries required on DA Form 2404 and DD Form 1970 are accurate, complete, and legible.

4.  Evaluation:  Check each student's performance of PMCS and transporting personnel.

5.  Summary:

a.  Recap main points.

b.  Allow for questions.

c.  Clarify questions.

d.  Give closing statement.

6.  Retraining:  Retrain No-Gos and slow learners.

E.  SAFETY RESTRICTIONS.

1.  When the vehicle is in motion, all occupants must be seated.

2.  The number of passengers transported in buses in over-the-road service will be restricted to the designated seating capacity.

3.  Before loading or unloading passengers, ensure the transmission is placed in neutral (some automatics are placed in park); the parking brake is set; the engine is shut off; and the ignition key is removed.

4.  Ensure students remove all jewelry and ID tags before performing PMCS.

5.  Ensure students pay particular attention to the cautions and warnings listed in the operator's manual.

6.  Ensure ground guides are used when backing the vehicle.

7.  Ensure a safe following distance and speeds (as determined by the local command) are maintained when driving on the driver-training route.

8.  Ensure all drivers wear seat belts while vehicle is in operation.

F.  ADDITIONAL COMMENTS AND INFORMATION.  Recommended instructional time is 7 hours (.5 conference, .5 demonstration, and 6.0 practical exercise, including 1.0 PMCS).

| SECTION II - TESTING |  |
| --- | --- |
| LESSON TITLE: | TRANSPORT PASSENGERS IN A BUS |
| TASK NUMBER: | 551-721-1387 (Transport Passengers in a Bus) |

A.  TRAINING OBJECTIVE.

|  | TASK: | Pass the written test. |
| --- | --- | --- |
|  | CONDITION: | In a classroom, given an examination booklet and pencil. |
|  | STANDARDS: | Answer correctly 14 of 20 questions within 30 minutes. Use either the primary written test or the alternate written test. |

B.  INTERMEDIATE TRAINING.  None.

C.  ADMINISTRATIVE INSTRUCTIONS.

1.  Training time:  As scheduled.

2.  Training location:  Classroom as scheduled.

3.  Training type:  Test evaluation.

4.  Students:  Personnel as scheduled.

5.  Principal and assistant instructors required:  One instructor for every 20 students.

6.  Training aids and equipment:  Examination booklet and pencil.

7.  References:  AR 385-55, FM 21-305, and appropriate vehicle operator's manual.

D.  SEQUENCE OF ACTIVITY.

1.  Introduction:

a.  Interest device.

b.  Tie-in.

c.  Lesson objective (paragraph A).

d.  Procedures:

(1)  Explanation.

(2)  Test evaluation.

(3)  Summary.

2.  Explanation and demonstration:  Administer written examination.  Use either the primary or alternate test.

3.  Evaluation:  Check written test results.

4.  Summary:

a.  Recap main points.

b.  Allow for questions.

c.  Clarify questions.

d.  Give closing statement.

5.  Retraining:  Retrain and retest No-Gos.

E.  SAFETY RESTRICTIONS.  None.

F.  ADDITIONAL COMMENTS AND INFORMATION.  Recommended testing time is 1 hour.

TRANSPORT PASSENGERS IN A BUS

WRITTEN TEST (PRIMARY)

| NAME |  | RANK | DATE |
| --- | --- | --- | --- |
| Multiple choice: Read each question carefully and write the answer which is most correct on the blank line to the left. |  |  |  |
| ______ 1. | When is it recommended that you wear a seat belt? a. Never. b. Only at speeds in excess of 25 mph. c. Always. d. Only when traveling on the interstate highways or turnpikes. |  |  |
| ______ 2. | Recapped tires are permitted on the steering axle provided-- a. They have at least 4/32-inch tread depth. b. They have at least 2/32-inch tread depth. c. They have passed a safety inspection. d. Never. |  |  |
| ______ 3. | All of the following are required during your pre-trip inspection except-- a. Checking handholds and railings to see if secure. b. Checking cylinder wall thickness. c. Ensuring emergency exit handles are secure. d. Checking to see if turn signals and brake lights are operating properly. |  |  |
| ______ 4. | When stopping at a railroad crossing, you must stop-- a. No closer than 15 feet and no farther than 50 feet. b. No closer than 20 feet and no farther than 60 feet. c. No closer than 25 feet and no farther than 50 feet. d. No closer than 10 feet and no farther than 15 feet. |  |  |
| ______ 5. | Which of the following is true about refueling your bus? a. You must refuel with all windows opened fully. b. Never refuel with passengers aboard. c. Always use a static cable. d. Never refuel with passengers aboard when in a closed building. |  |  |
| ______ 6. | You are approaching a drawbridge. No signal device or attendant is present. You must-- a. Stop at least 50 feet before the draw of the bridge. b. Stop no closer than 75 feet before the draw of the bridge. c. Not stop as that might cause an accident. d. Speed up and cross quickly before the bridge is raised. |  |  |
| ______ 7. | If you must discharge an unruly or disruptive passenger, you should-- a. Stop immediately and discharge the passenger. b. Try to counsel to him before discharging him from the bus. c. Choose a location that is most inconvenient to him. d. Choose a location that is safe and well lighted. |  |  |
| ______ 8. | Which of the following emergency equipment is required on your bus? a. First aid kit, tire chains, and tire-changing equipment. b. Fire extinguisher, tire chains, and first aid equipment. c. Fire extinguisher, a highway warning kit (3 reflective triangles or 3 emergency reflectors), and spare electric fuses, unless bus is equipped with circuit breakers. d. Fire extinguisher, accident reporting kit, and a highway warning kit (3 reflective triangles or 3 emergency reflectors). |  |  |
| ______ 9. | Speed limits on curves are standardized for-- a. Automobiles. b. Buses. c. Tractor-trailers. d. Small straight trucks. |  |  |
| ______ 10. | Buses may have recapped or re-grooved tires-- a. On any or all of the wheels. b. Only when the average speed will be less than 40 mph. c. Only on the front wheels. d. Anywhere except the front wheels. |  |  |
| ______ 11. | Your bus is disabled. The bus, with passengers aboard, may be towed or pushed to a safe place only-- a. By another bus with its four-way flashers on. b. By a 27,000 GVWR or larger tow truck. c. If the distance is less than 500 yards. d. If getting off the bus would be more risky for the passengers. |  |  |
| ______ 12. | When inspecting your bus, you must be sure that-- a. Every handhold and railing is secure. b. Passenger signaling devices are working. c. Emergency exit handles are secure. d. All of the above. |  |  |
| ______ 13. | If your bus is equipped with an emergency exit door, it must-- a. Be secured when the bus is being driven. b. Always have a red door light turned on. c. Not have any signs, stickers or markings near it. d. All of the above |  |  |
| ______ 14. | Baggage and freight must be secured to-- a. Allow the driver to move freely and easily. b. Protect passengers from injury if carry-on items shift or fall. c. Allow passengers to exit by any door or window in an emergency. d. All of the above. |  |  |
| ______ 15. | Never drive with-- a. Your seat belt securely fastened. b. An emergency exit door or window open. c. Recapped tires on the non-steering axles. d. Baggage doors latched. |  |  |
| ______ 16. | Bus crashes most often happen-- a. At railroad crossings. b. On expressways. c. At intersections. d. At school zones. |  |  |
| ______ 17. | On a road with poor traction, traveling too fast around a curve might cause the bus to-- a. Slide off the curve. b. Coast. c. Stop abruptly. d. Do nothing. |  |  |
| ______ 18. | On a road with good traction, traveling too fast around a curve may cause the bus to-- a. Coast. b. Stop abruptly. c. Roll over. d. Hydroplane. |  |  |
| ______ 19. | If the bus leans toward the outside on a banked curve, this means-- a. It is going too fast. b. It is going too slow. c. Nothing; this is normal. d. Nothing; you just downshifted. |  |  |
| ______ 20. | When crossing railroad tracks, do not-- a. Listen and look in both directions for trains. b. Check for another train coming in the other direction, on other tracks, before crossing. c. Change gears while crossing the tracks, if the bus has a manual transmission. d. Stop the bus before the railroad crossing. |  |  |

TRANSPORTING PASSENGERS IN A BUS

WRITTEN TEST ANSWER SHEET (PRIMARY)

| 1. | C | 11. | D |
| --- | --- | --- | --- |
| 2. | D | 12. | D |
| 3. | B | 13. | A |
| 4. | A | 14. | D |
| 5. | D | 15. | B |
| 6. | A | 16. | C |
| 7. | D | 17. | A |
| 8. | C | 18. | D |
| 9 | A | 19. | A |
| 10. | D | 20. | C |

TRANSPORT PASSENGERS IN A BUS

WRITTEN TEST (ALTERNATE)

| NAME |  | RANK | DATE |
| --- | --- | --- | --- |
| Multiple choice: Read each question carefully and write the answer which is most correct on the blank line to the left. |  |  |  |
| ______ 1. | If the bus leans toward the outside on a banked curve, this means-- a. It is going too fast. b. It is going too slow. c. Nothing; this is normal. d. Nothing; you just downshifted. |  |  |
| ______ 2. | On a road with good traction, traveling too fast around a curve may cause the bus to-- a. Coast. b. Stop abruptly. c. Roll over. d. Hydroplane. |  |  |
| ______ 3. | On a road with poor traction, traveling too fast around a curve might cause the bus to-- a. Slide off the curve. b. Coast. c. Stop abruptly. d. Do nothing. |  |  |
| ______ 4. | Your bus is disabled. The bus, with passengers aboard, may be towed or pushed to a safe place only-- a. By another bus with its four-way flashers on. b. By a 27,000 GVWR or larger tow truck. c. If the distance is less than 500 yards. d. If getting off the bus would be more risky for the passengers. |  |  |
| ______ 5. | Buses may have recapped or re-grooved tires-- a. On any or all of the wheels. b. Only when the average speed will be less than 40 mph. c. Only on the front wheels. d. Anywhere except the front wheels. |  |  |
| ______ 6. | Speed limits on curves are standardized for-- a. Automobiles. b. Buses. c. Tractor-trailers. d. Small, straight trucks. |  |  |
| ______ 7. | 7. Which of the following emergency equipment is required on your bus? a. First aid kit, tire chains, and tire-changing equipment. b. Fire extinguisher, tire chains, and first aid equipment. c. Fire extinguisher, a highway warning kit (3 reflective triangles or 3 emergency reflectors), and spare electric fuses, unless the bus is equipped with circuit breakers. d. Fire extinguisher, accident reporting kit, and a highway warning kit (3 reflective triangles or 3 emergency reflectors). |  |  |
| ______ 8. | If your bus is equipped with an emergency exit door, it must-- a. Be secured when the bus is being driven. b. Always have a red door light turned on. c. Not have any signs, stickers, or markings near it. d. All of the above. |  |  |
| ______ 9. | 9. You are approaching a drawbridge. No signal device or attendant is present. You must-- a. Stop at least 50 feet before the draw of the bridge. b. Stop no closer than 75 feet before the draw of the bridge. c. Not stop as that might cause an accident. d. Speed up and cross quickly before the bridge is raised. |  |  |
| ______ 10. | Baggage and freight must be secured to-- a. Allow the driver to move freely and easily. b. Protect passengers from injury if carry-on items shift or fall. c. Allow passengers to exit by any door or window in an emergency. d. All of the above. |  |  |
| ______ 11. | If you must discharge an unruly or disruptive passenger, you should-- a. Stop immediately and discharge the passenger. b. Try to counsel to him before discharging him from the bus. c. Choose a location that is most inconvenient to him. d. Choose a location that is safe and well lighted. |  |  |
| ______ 12. | Bus crashes most often happen-- a. At railroad crossings. b. On expressways. c. At intersections. d. At school zones. |  |  |
| ______ 13. | When stopping at a railroad crossing, you must stop-- a. No closer than 15 feet and no farther than 50 feet. b. No closer than 20 feet and no farther than 60 feet. c. No closer than 25 feet and no farther than 50 feet. d. No closer than 10 feet and no farther than 15 feet. |  |  |
| ______ 14. | When crossing railroad tracks, do not-- a. Listen and look in both directions for trains. b. Check for another train coming in the other direction, on other tracks, before crossing. c. Change gears while crossing the tracks, if the bus has a manual transmission. d. Stop the bus before the railroad crossing. |  |  |
| ______ 15. | Which of the following is true about refueling your bus? a. You must refuel with all windows opened fully. b. Never refuel with passengers aboard. c. Always use a static cable. d. Never refuel with passengers aboard when in a closed building. |  |  |
| ______ 16. | Never drive with-- a. Your seat belt securely fastened. b. An emergency exit door or window open. c. Recapped tires on the non-steering axles. d. Baggage doors latched. |  |  |
| ______ 17. | When inspecting your bus, you must make sure that-- a. Every handhold and railing is secure. b. Passenger signaling devices are working. c. Emergency exit handles are secure. d. All of the above. |  |  |
| ______ 18. | All of the following are required during your pre-trip inspection except-- a. Checking handholds and railings to see if secure. b. Checking cylinder wall thickness. c. Ensuring emergency exit handles are secure. d. Checking to see if turn signals and brake lights are operating properly. |  |  |
| ______ 19. | Recapped tires are permitted on the steering axle provided-- a. They have at least 4/32 inch tread depth. b. They have at least 2/32 inch tread depth. c. They have passed a safety inspection. d. Never. |  |  |
| ______ 20. | When is it recommended that your wear a seat belt? a. Never. b. Only at speeds in excess of 25 mph. c. Always. d. Only when traveling on the interstate highways or turnpikes. |  |  |

TRANSPORTING PASSENGERS IN A BUS

WRITTEN TEST ANSWER SHEET (ALTERNATE)

| 1. | A | 11. | D |
| --- | --- | --- | --- |
| 2. | C | 12. | C |
| 3. | A | 13. | A |
| 4. | D | 14. | C |
| 5. | D | 15. | D |
| 6. | A | 16. | B |
| 7. | C | 17. | D |
| 8. | A | 18. | B |
| 9 | A | 19. | D |
| 10. | D | 20. | C |