Section 2.0	Scope of Work

The Scope of Work for this proposed project is outlined as follows:

2.1 Major Equipment – The Owner will supply two (2) GE   Frame VII FA Gas fueled Gas Turbine Generators with associated equipment. The power will supply the adjacent substation at 230KV.

The Frame VII FA is configured to operate on natural gas only which will be supplied to the plant at 250 PSIG. The Owner will furnish the following outlined associated equipment:

.. Two PEEC Electrical Buildings

.. Two Starting Transformers 18KV Primary

.. Two Exhaust Stacks

.. Two 100% Capacity Fuel Gas Compressors 250 psig to 500 psig

2.2 The Contractor will provide the Balance of Plant which includes the following outlined list of major equipment:

.. Redundant Fuel Gas Filters

..Fuel Gas Compressor L.O. and Gas Discharge - Cooling Tower w/3 ea. 50% capacity circulation            pumps

.. Cleaning Owner provided 264,000 gallon Diesel Storage for use as Raw water/Firewater          Storage Tank

.. Water Treatment Systems (RO & EDI)

.. Oily Water Drain System with Oily Water Separator

.. One (1) diesel fueled 500KW Emergency Generator

.. Control/Office Building

.. Electric Utility Building

.. Water Treatment/Firewater Pump/Warehouse Building

.. Guard House

.. Two 18KV Generator Breakers

.. Two Sets of 18KV ISO Phase Bus Ducts

.. Two Aux Transformer 18KV / 4.16KV 15 MVA

2.3 The Contractor will furnish the Engineering and Detailed Design of the facility including Civil, Foundations, Structural, Mechanical, Electrical, Instrumentation, and Control System.

2.4 The Contractor will furnish the equipment and materials including installation of same as follows:

.. Site Preparation, Foundations, Gravel, Roads, Parking, and Fencing

.. Mechanical installation of the various items of equipment with pipe, valves, fittings, supports, etc

.. Electrical Installation of the various items of equipment with cabling, conduit, duct banks, etc.

.. Contractor will install Owner supplied buildings including the PEEC Buildings

.. Contractor will furnish and install Control/Office Building

.. Contractor will furnish and install Water Treatment/Firewater Pump/Warehouse Building

..Contractor will furnish and install an Electric Utility Building

2.5 In addition to the above the Contractor will supply:

.. Project Management, QA/QC, Safety, and Training

.. Plant Commissioning and Start Up

.. Plant Performance and Reliability Testing

.. Furnish Plant Documentation including Vendor Information, Warranty, Engineering ..Calculations, O&M Manuals, and As Built Drawings.

Contractor will furnish commissioning, start-up, and performance testing services for the Frame 7FA GTG’s.

- This Technical Scope Document will become an exhibit to the EPC Contract.

THIS DOCUMENT IS CONFIDENTIAL.  IT IS DESIGNED AND INTENDED FOR efaces’S USE.  THIS DOCUMENT IS FOR THE SOLE PURPOSE OF EVALUATING CONTRACTOR’S PROPOSAL FOR THE (2) GE Frame 7FA POWER PROJECT.