CVG SIDOR POWER PROJECT SITE “A”

DATE:  17 July 2010, Saturday

PROJECT #: 410-3202

LOCATION: SIDOR Industrial Area, Puerto Ordaz, Venezuela

SITE MANAGER: Patrick Melody

TEMPERATURE:  88 F

RANGE: 85 to 95 F

SITE CONDITIONS:  Partly Cloudy/Heavy Rains Over Night

PERSONNEL ON SITE:

| Lugo, Bill | Project Director |  |  |
| --- | --- | --- | --- |
| Siros, James | Mechanical Supt. | Newan, Miguel | Mat'l. Handler (Local) |
| Bird, Jason | Technical Assistant | Izquierdo, Weiser | Mat'l. Handler (Local) |
| Melody, Patrick | Site Manager | Herman, Flores | Tool Room (Local) |
| Little, John | Mat’l. Control | Monasterios, O | Safety ( Local) |
| Frawely, Ted | Elect. Supt. |  |  |
| Blacke, Edward | Safety Manager |  |  |
|  |  | Zambrano Natalia | Document Control |

SUBCONTRACTOR PERSONNEL:

Welders                 6				Carpenters		6

Fabricators           5				Laborers		8

Erector                15				Operators		6

Mechanic              2				Truck Drivers	2

Operator               2

Total                    52

- GENERAL ITEMS

    - Design and procurement for the project needs to be completed as soon as possible to support current project schedule. Daily meetings are held with the design team and/or field engineers to follow up on the design and to discuss design changes.

- Receipt of cathodic design and material is currently restraining erection of the Raw Water and Demin Water tank.

- DCS wiring and raceway design is not complete. DCS equipment is still in fabrication. Equipment is being expedited to prevent it from being critical.

- CLIENT ISSUES/CONCERNS:

    - Location of existing gas line between 7EA and Control Building remains undeterminable after test pitting. Caution needs to be taken in excavations for foundations on west side of 7EA.

    - SIDOR has indicated they have an onsite Demin water source that is available for our use if required. SIDOR will supply tank trucks as needed to furnish approximately 85 gpm. SIDOR is interested in negating any schedules impact from the delay in removing power poles.

    - Preliminary discussions were held with SIDOR to discuss gas blows and alternative options. To the extent possible, SIDOR would like to minimize the need for gas blows.

- CIVIL:

    - Switch yard foundations ongoing

    - Switch yard - Site grading

    - Dewatering of site due to heavy overnight rains continues. Third night of heavy rains.

- CONCRETE FOUNDATIONS:

    - GT 300 Dewater PEECC foundation

    - Raw Water Tank set forms

- MECHANICAL:

    - GT 200 Fabricating pipe to gas compressor

    - GT 300 Install piping inside the unit

    - GT 300 Welding inside the stack

    - Fabricate fire line piping

    - Excavating for fire line at gas compressor

    - GT 300 Modify oil supply line

- ELECTRICAL:

6.1.6	Sialca a local electrical contractor who is being considered for award of the electrical scope of work will be on site to assist in a joint material take off to verify the accuracy of the electrical BOM.

- INSTRUMENTATION AND CONTROLS:

- SCHEDULED ITEMS:

    - General

      - CPS Schedule updates on going. We expect to have the schedule updated by 21 July 10.

    - Contract Milestone Payments

      - Completed Milestone Payments (50%)

Complete

      - Gas Turbines on Foundation (10%)

Complete

8.1.2.3         Civil Foundations Complete (25%)

August 20, 2010

8.1.2.4       Electrical/Mechanical Complete (10%)

August 28, 2010

      - Start- up Complete and Ready to Export Power (5%)

Sept. 15, 2010

Note (*):These dates are being revised based upon payments received on 14 July 2010. We are in the process of updating our schedule and the target dates will be revised upon completion of the update.

    - Target Ready for Start-Up Dates

      - Unit 100 – LM6000

Aug 31, 2010

      - Unit 200 – LM6000

Aug 31, 2010

      - Unit 300 – 7EA

Aug 28, 2010

Note (*): These dates are being revised based upon payments received on 14 July 2010. We are in the process of updating our schedule and the target dates will be revised upon completion of the update.

- CRITICAL AREAS OF CONCERN:

Design and procurement activities for the project need to be completed as soon as possible to support current project schedule. We continue to have daily communications with the design team in Tulsa and Sedalia as well as the field engineers on the design progress.

We have requested a design completion date for the outstanding scope of work to include in our schedule. This information has not been received yet.

Numerous scopes of work/material quotations have been prepared and are awaiting execution. A Subcontractor has been selected for erection of Pre Engineered Building. Purchase order will be issued early next week. The final proposal for tank erection has been received. Purchase order for tank erection will be issued early next week.

Dual fuel conversion activities are being evaluated as potentially impacting the critical path.

    - SAFETY:

        - Develop JSA as needed

        - Inspection of subcontractor power tools.

        - Inspection of motorized equipment prior to usage.

        - Scaffold and trenching inspections ongoing.

        - A training course for work in existing switch yard was held today

    - OUTSTANDING DRAWINGS:

        - Cathodic Protection Dwgs

        - Cable tray drawings for Water Treatment, Gas Compressor, and Control Maintenance Bldgs.

        - DCS design is not complete. Design has not been released for fabrication. This issue could impact critical path

    - DRAWINGS ISSUED:

11.	PICTURES:

PEECC Foundation