Section 2.0   Equipment List - Detailed Division of Responsibility between Owner and Contractor:

Note: The Owner has purchased two (2) dual fueled GE LM 6000 PC Sprint GTGs and two (2) dual fueled LM 2500+ GTG’s for this project.  This change order Scope of Work is to add liquid fuel capability to the Balance of Plant.

| Material/Responsibility |  |  | Qty | Description |  |
| --- | --- | --- | --- | --- | --- |
| OWNER Will Provide |  |  |  |  |  |
|  |  |  | 1 Lot | Project Site free and clear of rock and ground water |  |
|  |  |  | 1 Lot | #2 Diesel Fuel Supply Delivered to the Plant Boundary by truck. |  |
|  |  |  | 1 Lot | Diesel Fuel Supply for Commissioning and Startup |  |
|  |  |  | 1 Lot | Permits for, Building, Construction, Operations, etc. |  |
|  |  |  | 1 Lot | Construction & Commissioning Auxiliary Power, 480V three phase power |  |
|  |  |  | 1 Lot | Import Duties and Taxes |  |
| Contractor – Derwick Responsibility |  |  |  |  |  |
| Civil / Structural |  |  |  |  |  |
|  |  |  | 1 Lot | Site Excavation, and final grading |  |
|  |  |  | 1 Lot | Design and Installation of Plant Concrete Foundations |  |
| Buildings |  |  |  |  |  |
|  |  |  | 2 | Truck off-loading facilities including pumps |  |
|  |  |  | 1 | Expansion of DFO Treatment Shed |  |
| Mechanical |  |  |  |  |  |
|  |  |  | 1 | Treated Fuel Storage Tank Expansion of 100,000 gallon Raw DFO Storage Tank Expansion of 250,000 gallon |  |
|  |  |  | 1 | DFO Treatment Skid with two Centriguges |  |
|  |  |  | 1 | LM 6000 Diesel Fuel Oil Injection Pump Skid |  |
|  |  |  | 1 | LM 6000 Diesel Fuel Oil Duplex Filter Skids |  |
|  |  |  | 4 | Liquid Fuel unloading pumps |  |
|  |  |  | 1 | Liquid Fuel Metering |  |
|  |  |  | 1 | Fire Water Foam System Expansion |  |
|  |  |  | 2 | Diesel Fuel Forwarding Pumps |  |
| Electrical |  |  |  |  |  |
|  |  |  | 1 | 13.8KV / 480 V 250 KVA Aux. Transformer DP 100 |  |
|  |  |  | 1 | DFO Treatment 480 V MCC |  |
|  |  |  | 1 Lot | BOP 480 V / 120 V Transformers, Lights, Panels etc. |  |
|  |  |  | 1 Lot | Cathodic Protection for underground steel piping |  |
| I&C |  |  |  |  |  |
|  |  |  | 1 Lot | Plant Instrumentation |  |
|  |  |  | 1 | Interconnect to Plant DCS System |  |
| Construction |  |  |  |  |  |
|  |  |  | 1 Lot | Construction Tools, Rental Equipment & Rental Cranes |  |
|  |  |  | 1 Lot | Temporary Power Distribution |  |
|  |  |  | 1 Lot | Local Subcontractor(s) Electrical & Mechanical Craft Labor |  |
|  |  |  | 1 Lot | Transportation of all Contractor supplied BOP equipment |  |
|  |  |  | 1 Lot | Construction Offices, Storage, Temporary Facilities and Utilities |  |
| Engineering |  |  |  |  |  |
|  |  |  | 1 Lot | Conceptual and Detail Design Engineering (Total Plant) |  |
| Project Management |  |  |  |  |  |
|  |  |  | 1 Lot | Project Management with Scheduling, QA/QC, safety, and training |  |
|  |  |  | 1 Lot | Plant Start-up, Commissioning and Testing |  |
|  |  |  | 1 Lot | Overall Plant Training |  |