PROPOSAL

Presented To:

Derwick Associates Corporation

for

Controls Retrofit

Prepared By

Proposal No. 1310-3882

October 7, 2010

This document is privileged and contains confidential information intended for use only by

Derwick.

Table of Contents

1.0	Introduction	3

2.0	Work Scope	3

3.0	Controls Platform	3

4.0	Pricing	4

5.0	Terms & Conditions	4

6.0	Follow Up	4

Attachment A	Controls System Overview

Attachment B1	Controls Hardware Description

Attachment B2	Allen Bradley Flex I/O Brochure

Attachment C1	HMI System Overview

Attachment C2	Allen Bradley FactoryTalk Brochure

Attachment D	Preliminary Cabinet Layout

- Introduction

ProEnergy Controls Solutions (“ProEnergy”) is pleased to provide this proposal to Derwick Associates Corporation (“Derwick”) for a Control System Upgrade located at Margarita Island.  This system will replace the existing GE Speedtronic Mark V control, and add the capability of dual fuel operation for a General Electric Frame 7EA Gas Turbine Generator.  This control system will execute all governing for the gas turbine and all associated sequencing functions necessary to perform turbine start-up, power generation, shutdown, and protection.

Primary turbine operations include the following:

- Start Sequencing

- Flame Detection

- Speed Governor Control

- Exhaust Gas Temperature Control

- Accel/Decel Fuel Limiting

- Dual Fuel Systems Control

- Fuel Valve Position Controls w/ dual LVDT Position Feedback

- IGV Position Control w/ dual LVDT Position Feedback

- Bleed Valve Control

- Generator Stator Temperature Monitoring

- Bearing Temperature Monitoring

- Wheelspace Temperature Monitoring

- Auto/Manual Synchronization

- Mega Watt/VAR Control

- Alarm/Fault Monitoring

- Ancillary System controls as applicable (i.e. enclosure fans, hydraulic ratchet, motor controls, etc.)

- Work Scope

ProEnergy will provide the new cabinets complete with control hardware, software, and cabinet internal wiring. An HMI computer and software will also be provided.

Installation, commissioning, and site services will be addressed via a separate proposal once we agree upon the system to be supplied.

- Controls Platform

The new controls system proposed will be based upon the AB ControlLogix Platform with Flex I/O.  This ensures that replacement parts are readily available from Rockwell’s worldwide supplier network.  This open expandable platform ensures that EDC will be able to maintain this system either with their own resources, with contracted support, or through the engineering and support services provided by ProEnergy.

ControlLogix with Flex I/O has been used in turbine control applications for years on a variety of turbine types, including:

- Heavy Industrial Gas turbines

- Aero-derivative Gas turbines

- Light Industrial Gas turbines

- Industrial Steam turbines

- Mainline Steam turbines

- Hydro turbines

Specific functions for the Flex I/O modules:

- Speed sensing – interface to existing speed probes

- Vibration – Flex XM vibration modules will interface to existing Vibration sensors

- Fuel valves control – demand and feedback signals to existing stop and control valves

- Guide vane control - demand and feedback signals to existing servos

- Temperature monitoring and control - interface to existing thermocouples and RTDs

- Pricing

ProEnergy will perform the scope of work identified above for a price of $540,000.00 per unit.

- Terms & Conditions

This proposal shall be valid for thirty (30) days; provided, however, the obligation to treat this proposal as confidential, and that it cannot be shared with any third party without the prior written consent of ProEnergy shall survive.

This proposal, and any resulting contract or agreement, shall be subject to the terms and conditions to be mutually agreed upon.

- Follow Up

Please contact the following person at ProEnergy for information regarding this proposal:

Joaquin Mavares, VP International Sales

jmavares@proenergyservices.com

Office:	660.829.5100

Cell:	719-992-1790

Fax:	660.829.1160

- Attachment A

- Controls System Overview

Functional Overview

The Allen Bradley based control system will support all current modes of operation currently being provided by the GE Speedtronic Mark V turbine controls package.  A chain of governor control programs are provided within the control system software, each providing a dedicated fuel demand with selection logic being used to determine which loop should be in control.  If the system approaches any operational limits, a descriptive alarm will be issued.  If a limit is exceeded, or if there is a system failure, the unit will be shutdown. The new systems will provide trending and archiving functionality, the ability to produce operator logs, and to capture high speed data during a trip event to help identify the root cause of the trip.

Dual Fuel Control

The control system will provide outputs for the existing Gas Fuel system and to support the requirements of a Liquid Fuel system.  This includes Gas and Liquid Fuel Valve position controls and controls for the liquid fuel pump, ignitors, and all associated shutoff, vent, dump, manifold purge, fill, and atomizing air valves.

Generator Stator Temperature Monitoring

The control system will provide generator stator temperature monitoring.  This includes high and low RTD identification, display of their associated values, and the temperature spread between them.  Alarms or trips will be generated based on high temperatures or an excessive spread.

Exhaust Temperature Monitoring and Control

The new control system will provide Exhaust temperature monitoring and control.  An average of the valid thermocouples will be used to provide a value for EGT Control.  Individual thermocouples will be rejected if they are sensed as faulty or can manually be rejected by the operator via HMI.  Failure of thermocouples will be alarmed and an excessive number of failures will cause a unit trip.   The temperature spread of valid T/Cs will be monitored and alarmed if the spread is excessive.

Bearing Temperature Monitoring

The control system will monitor bearing temperatures including Lube Oil Bearing Drain Temp 1, 2 and 3, Generator Bearing Drain Temp 1 and 2, Active and Inactive Thrust Bearing Temps, Turbine Bearing Metal Temperatures 1, 2 and 3, and Generator Bearing Metal Temperatures 1 and 2.  High temperature alarms and trips will be provided.

Vibration Monitoring

The control system will monitor all vibration and position signals from both the Turbine and Generator and provide alarms and/or trips.

Hydraulic/Lube Oil Control and Monitoring

The control system will provide outputs for the existing Lube Oil and Hydraulic Oil systems.  This includes managing primary and emergency pumps and valves, as well as monitoring all associate I/O corresponding to pressures, temperatures, tank levels and status for alarm and fault management.

Additional Control Functions

The control system will manage functionality associated with Inlet Guide Vanes, Bleed Valves, Space Heaters, Coolers and Cooling Fans, the Water Wash system, and the Generator Controls Interface.  Smoke, Fire, and Gas Detection are also provided.

Operator Interface

A local HMI running on a desktop PC will provide the primary operator interface for control system operation, monitoring and troubleshooting.  Operators will be able to perform all control selections, monitor the turbines performance and operating parameters, and see the status of auxiliary systems.  All system and operational alarms and unit trip information including first out trip information will be provided on the local HMI.  Real time trending and optional archiving functionality are available.

- Attachment B1

- Controls Hardware Description

ControlLogix System Overview

The ControlLogix system provides discrete, drives, motion, process, and safety control together with communication and state-of-the-art I/O in a small, cost-competitive package. The system is modular, so you can design, build, and modify it efficiently - with significant savings in training and engineering.

System Components

ControlLogix system components include:

| Component | Description |
| --- | --- |
| ControlLogix Controllers | • Options include: ControlLogix standard controllers, GuardLogix safety controllers, and ControlLogix-XT extended temperature controllers. • User memory ranges from 2 to 32 MB, depending on controller type. |
| ControlLogix Communication Modules | • Options include: EtherNet/IP, ControlNet, DeviceNet, Data Highway Plus, Remote I/O, Foundation Fieldbus, Serial, DH-485, and SynchLink. • Install multiple communication-interface modules into the ControlLogix backplane to configure a gateway to bridge or route control and information data between different networks. |
| ControlLogix Power Supplies | • ControlLogix power supplies are used with the 1756 chassis to provide 1.2V, 3.3V, 5V, and 24V DC power directly to the chassis backplane. • Select one power supply for each chassis, if you are using standard power supplies. • Select a power supply bundle if you are planning a redundant power supply system. |
| ControlLogix Chassis | • Choose from ControlLogix standard or ControlLogix-XT chassis. • Standard chassis are available with 4, 7, 10, 13, or 17 slots. • ControlLogix-XT chassis are available with 5 or 7 slots. • Slot fillers are required for empty slots. |

Optional system components include:

| Component | Description |
| --- | --- |
| Visualization Products | • Visualization products, together with Logix for control and NetLinx architecture for communication, make up the Rockwell Automation Integrated Architecture strategy. • Choose from PanelView Plus and PanelView Plus CE electronic operator interface and Rockwell Automation industrialized personal computer hardware. • Available Rockwell Software supervisory control software includes FactoryTalk View software. |
| Software | • Your selection of modules and network configuration determines what software packages you need to configure and program your system. • Program your ControlLogix system with RSLogix 5000 Enterprise Series software. • Additional design and configuration products include RSNetworx and RSLinx software. |

ControlLogix Controllers Overview

The ControlLogix controller provides a scalable controller solution that is capable of addressing a large amount of I/O points.  The controller can be placed into any slot of a ControlLogix I/O chassis and multiple controllers can be installed in the same chassis. Controllers in the same chassis communicate with each other over the backplane (just as controllers can communicate over networks) but operate independently.

ControlLogix controllers can monitor and control I/O across the ControlLogix backplane, as well as over I/O links. ControlLogix controllers can communicate over EtherNet/IP, ControlNet, DeviceNet, DH+, Remote I/O, and RS-232-C (DF1/DH-485 protocol) networks and many third party process and device networks. To provide communication for a ControlLogix controller, the proper communication interface module must be installed into the chassis.

ControlLogix Controllers Environmental Specifications

|  | ControlLogix Controllers |
| --- | --- |
| Operating temperature | 0…60 °C (32…140 °F) |
| Storage temperature | -40…85 °C (-40…185 °F) |
| Relative humidity | 5…95% noncondensing |
| Vibration | 2 g at 10…500 Hz |
| Operating shock | 30 g |
| Nonoperating shock | 50 g |

Standard ControlLogix Controllers

The ControlLogix controller is part of the Logix5000 family of controllers. A ControlLogix system includes:

- The ControlLogix controller, available in different combinations of user memory.

- RSLogix 5000 programming software.

- Separate communication modules for network communications

- A built-in serial port on every 1756-L6x controller.

- A built-in USB port on every 1756-L7x controller.

Features

| Feature | 1756-L61, 1756-L62, 1756-L63, 1756-L64, 1756-L65 | 1756-L73, 1756-L75 |
| --- | --- | --- |
| Controller tasks | • 32 tasks • 100 programs/task • Event tasks: all event triggers |  |
| Communication options | • EtherNet/IP • ControlNet • DeviceNet • Data Highway Plus • Remote I/O • SynchLink • Third-party process and device networks |  |
| Controller connections supported, max | 250 | 500 |
| Network connections, per network module | • 100 ControlNet (1756-CN2/A) • 40 ControlNet (1756-CNB) • 256 EtherNet/IP; 128 TCP (1756-EN2x) • 128 EtherNet/IP; 64 TCP (1756-ENBT) |  |
| Controller redundancy | Full support |  |
| Programming languages | • Relay ladder • Structured text • Function block • SFC |  |

- Attachment B2

- Allen Bradley Flex I/O Brochure

- Attachment C1

- HMI System Overview

Functional Overview

The ProEnergy Controls Solutions HMI application will be created using Rockwell Automation’s Factory Talk View Studio Site Edition software.

Each screen will include a Title bar with date and time and a critical data summary which will display important information for the unit such as operating mode, mode status, speed, load, and governor mode. Each screen will also include navigation buttons with single button selection for common screens such as the Main Overview, Alarm, and Trend screens.  In addition to the standard turbine screens, there will be an alarm summary screen and a screen to access any trends. Alarms can be acknowledged and reset from any screen.

Background Colors used are designed not to interfere with alarming, indicators and other indications that are used in operations.  The base background color will be a light gray.

Typically, ProEnergy Controls Solutions will provide a local HMI which will be placed in close proximity to the turbine. If needed, an additional remote HMI can be provided.

Alarms

Alarms will be displayed on the Alarm Summary screen. The summary shall have a black background with rows and columns for Time, Date, Alarm Name/Number and Description.  Depending on Software functionality, multiple columns may be available and configured to customer specifications.  The Alarm Summary Screen shall contain a First-Out Shutdown Indication for troubleshooting.

Alarm Color coding will be as follows:

  - Red for Shutdowns or Trips.

  - Yellow for Alarms

Trending

Real-Time trending will be provided per customer requirements. Historical trending can also be provided if needed.

Example Screens

Typical screens would include a system overview screen, permissive and sequence screens, fuel system screens, auxiliary system overview screens (as applicable).  Although the following screens are not a representation of a Frame 7EA, these will illustrate examples of some typical screens that would be developed.

Turbine Overview

Turbine Enclosure

Turbine Lube Oil

Start Permissives

Turbine Fuel System

Turbine Vibration

Turbine Alarms

- Attachment C2

- Allen Bradley FactoryTalk Brochure

- Attachment D

- Preliminary Cabinet Layout