Derwick

Electricidad de Caracas

La Raisa Power Plant

Technical Scope Document

Section 3.0 Equipment List - Detailed Division of Responsibility

Material/Responsibility Owner

Contractor – ProEnergy EPC Responsibility Civil / Structural

Buildings

Qty              Description

1 Lot                Removal of all unused building foundations, underground piping, etc. on the

proposed project site 1 Lot                Natural Gas Pipeline and supply metered at inter-tie point within 50’ of Plant

Boundary
3	100 % Fuel Gas Compressors

1 Lot                Fuel Gas and Liquid Fuel for Commissioning / Start-up that meets GE Fuel

Specifications
1 Lot                Liquid Fuel Treatment System
1 Lot                Raw Water Supply to site boundary
1 Lot                Raw Water for Commissioning and Startup
1 Lot                Waste Water and Waste Oil Truck Removal
1	Provision for Telephone communications to site

1 Lot                Permits for Environmental, Importation, Transportation, Building, Operations,

etc. to allow plant construction and commercial operations 1 Lot                Construction Commissioning 480V three phase power 1 Lot                Access Roads to site

1 Lot                Any required Electric Utility 230 KVTie-In interconnect to the Utility
1 Lot                Site Soil Analysis at key foundation locations
1 Lot                Import Duties and Taxes
I Lot	Construction lay down area (up to 3 acres)

1 lot	Right of Way, easements, etc. to allow proposed interconnects as required

Gas Turbine Generator

Pratt & Whitney FT8-3 SwiftPac (60Hz) Dual Fueled Gas Turbine Generator 3

packages

3	Modular Control Buildings with:

-Turbine Control Panel

- Generator Control Panel and Protection Panel

- 24 VDC Batteries and Chargers

- 125 VDC Batteries and Chargers

- 125 VDC Rundown motor starter

- 13.8 KV Generator Breaker Nema 3R

-	GTG 480 V MCC
1                              Water Treatment consisting of Multi-Media filters and R.O. System

6	Exhaust Stacks with Silencers

1 Lot                Site Preparation, Rough Grading, Excavation, final grading and fencing

1 Lot                Temporary Power Distribution

1 Lot                Plant Concrete Foundations

1 Lot                Plant Paving, Gravel and Pads for the GTG Turbines and Generators

1 Lot                Structural piping supports, platforms, ladders, and Misc structural steel

supports. (Note: Platforms and ladders for the exhaust stacks to be furnished

by Owner)

1	Demin Water Treatment & Forwarding Pump & Firewater pump Building

1	Compressor shed

1	Liquid Fuel Treatment, Diesel Pump Shed.

Section 3.0 Balance of Plant Equipment Description

1-3

Revision 2, April 29, 2009

Derwick

Electricidad de Caracas

La Raisa Power Plant

Technical Scope Document

Material/Responsibility Contractor (continued) Buildings

Mechanical

Electrical

Qty              Description

1	Control Room and, Office Building.

1	Warehouse and Maintenance building

1	Gas Compressor Building

1	Water Treatment Building

1	Liquid Fuel Treatment Building/Shed

1	Fuel Gas Plant ESD Valve

1	Fire Water System including pumps, controls, Loop, Monitors and Hydrants

1	650,00 gallon raw water/firewater Carbon Steel storage tank

1	250,000 stainless steel Demin Water Storage Tanks

3	GTG Duplex Demin Water Filter Skids

- Demineralized Water Treatment system (EDI) 3	Demin Water forwarding Pumps

- Oily Water Separator 1	Waste Oil Tank 15,000 Gallon w/waste oil pump to Truck for disposal

1	Waste Water Tank 15,000 Gallon w/waste water pump to Truck for disposal

2	650,000 gallon raw liquid fuel storage tanks
2	250,000 gallon clean liquid fuel storage tanks

2	100% capacity Liquid Fuel forwarding pumps

3	sets	Duplex Liquid Fuel Filters
1 Lot	Pipe, Valves and Fittings with Insulation as required

1	Lot	Mechanical Labor

3	13.8KV / 230KV 42/56/70 MVA GSU’s

2	13.8 KV Fused Disconnects (Station Service) NEMA 3R

1	13.8 KV/4160 V 2000 KVA Fuel Gas Compressor

2	13.8 KV/480 V 750 KVA Transformers
1	4160 V MCC for Fuel Gas Compressor
1	480 V Distribution Switch board
1	480 V BOP MCC

1 Lot	BOP 480/220 V Transformers, Lights, Panels etc

1	120 V UPS System for Control Room

1	Plant Grounding Grid

1 Lot	Lightning Protection

1 Lot	Cathodic Protection for underground steel piping

1 Lot	Area Lighting

1 Lot	480 V Welding Receptacles

1 Lot	Electrical Labor

1 Lot	Local Subcontractor(s) Civil, Electrical & Mechanical Craft Labor

Section 3.0 Balance of Plant Equipment Description

2-3

Revision 2, April 29, 2009

Derwick

Electricidad de Caracas

La Raisa Power Plant

Technical Scope Document

Material/Responsibility Contractor (continued)

Instrumentation & Control

Construction

Engineering

Project Management

Qty              Description

1 Lot	Plant Instrumentation

1	Remote DCS PLC

1 Lot	I&C Installation and Construction

1 Lot	Major Equipment Erection

1 Lot	Mechanical Installation and Construction

1 Lot	Construction Tools, Rental Equipment & Rental Cranes

1 Lot	Lubricants, Chemicals, Filters, etc. for Plant Commissioning

and Start up

1 Lot	Balance of Plant Start up and Commissioning Spare Parts

1 Lot	Transportation of all BOP Equipment to site

1 Lot	Plant Commissioning and Performance Testing

1 Lot	Overall Plant Training

1 Lot	Conceptual and Detail Design engineering (Total Plant)

1 Lot	Project Manuals including O&M, Warranty, and Engineering Calculations

1 Lot	As Built Drawings

1 Lot	Recommended Vendor Spare Parts List

1 Lot	Project Management with QA/QC, Safety, and Training

1 Lot	Project manuals including project procedures, Systems Turnover Manuals,

project implementation, QA/QC, safety and training

1 lot	Training

Section 3.0 Balance of Plant Equipment Description

3-3

Revision 2, April 29, 2009