NOMINAL 170 MW

Información Técnicas para

Las maquinas GE FRAME 7EA

Planta de Ciclo Sencillo

Para

CORPOELEC

(VENEZUELA)

170 MW POWER PLANT

PROPUESTA

Introduccion:

Energy Parts Solutions propone proveer una planta constituida del siguiente grupo de Turbinas a Gas en ciclo sencillo rateado nominalmente a 170 MW este equipo está constituido de (2 Turbinas GE Frame 7EA (“7EA”) con muy pocas horas de uso (Horas para el test de Prueba). Las Turbinas industriales generalmente no son probadas en sitio antes de la instalación; Debido a que estas ya fueron probadas esto elimina muchas preocupaciones y sorpresas en el campo durante la instalación reduciendo el tiempo del mismo. Esta unidades están completamente operacionales y listas para ser desmontadas y enviadas a Venezuela.

Scope of Supply

El siguiente seria el Scope o listado con la descripción del equipo a proveer:

GE Frame 7EA Gas Turbine

Dos Turbinas Gas General Electric (PG7121) 7EA Dry Low NOx (DLNI) rateada a 84.4 MW ISO / 60 Hz (59oF/15oC, sea level, 60% RH).

Brush-Generator

Air cooled Brush BDAX 8.365 ER generator with an EX2000 brushless excitation system that is rated at 101.8 MVA@0.85 PF, 13.8 kV, 3600 rpm, 60 Hz, 3 phases. The generator is a synchronous two-pole cylindrical rotor machine.

- Generator Air Filter

Donaldson self cleaning, single stage, pulse clean filter system.

- Generator-Control-System

The Generator Control System includes a Beckwith, Automatic Voltage Regulator (AVR), Digital Generator Protection (DGP) and Nexus 1250 metering module. The system is located in the PEECC and interfaces directly with the turbine control system. Other components included in the generator control system include the GE Multilin transformer protection relay, the EX2000 Excitation System and the lock out relays

- Generator Auxiliary Compartment

Contains the GE 15 kV vacuum circuit breaker.  The 15 kV class, vacuum, metal clad switchgear is installed in a NEMA 3R enclosure.  Circuit breaker charging and trip/close mechanisms operate from a 125 VDC battery supplied system.

- Generator Lineside and Neutral Grounding Equipment

The generator lineside equipment for the gas turbine generators is contained within the 15 kV switchgear that connects the generators to the generation bus. The neutral grounding equipment for the gas turbine generators, consisting of a typical transformer/resistor combination, is contained on the GTG skid within the generator package. The lineside and neutral grounding equipment is comprised of connections, surge arresters, surge capacitors, CTs, PTs and grounding transformers and resistors.

Accessory Module

Skid containing the 800 hp electric starting motor, auxiliary gearbox, torque converter and the following equipment for the lube oil system: integrated tank, filter, tube and shell heat exchanger, AC and DC pumps.

Exhaust Frame Blowers

Two air blowers provide cooling air to the rear frame of the gas turbine.

Turbine Inlet Filter

Donaldson air inlet filter system for the turbine provides clean filtered air for combustion use. The unit includes the louvered inlet, inlet heating, synthetic canister filter elements, pulse cleaning system, turbine inlet silencer, ductwork, ladders/platforms and support steel.

- Fogging System

A MEE Industries fogging system consisting of one pump skid, one sub-micron water filter and six high pressure pumps provides a cooling effect to the turbine inlet air by injecting demineralized water under high pressure directly into the inlet air stream. By lowering the temperature of the inlet air, the power output and the efficiency of the turbine is increased.

The skid with a design flow rate of 37.6 gpm contains six high pressure Cat Pump pumps. Five 10 hp pumps are rated for 7 gpm each and one 5 hp pump is rated for 3.5 gpm each for a total water pumping capacity of 38.5 gpm.

- Air Processing Unit

Air processing unit provides pressurized air for pulse cleaning of the turbine and generator inlet air filters and instrument actuation.  The system dries and cools compressor bleed air.

Fuel Gas Module

Fuel gas is controlled with the fuel gas stop/ratio valve, gas control valve (GCV), gas splitter valve (GSV) and the gas transfer valve (GTV) assemblies.  The stop/ratio valve and the GCV work in conjunction to regulate the total flow to the gas turbine and the GSV and GTV are used to control the distribution of the fuel flow delivered to the GE DLNI combustion chambers.  Servo valves, controlled from the TCS control panel, actuate the gas system valves.  The system is designed to deliver natural gas fuel at the correct pressure and flow rates to meet all starting, acceleration and loading requirements of gas turbine operation.

The following major components comprise the off-base fuel gas system:

- Gas Strainer

- Gas flow meter (corrected)

- Block valves

- Electronic flow control valves

- Electronic and local instrumentation

Fuel gas from the off-base supply system passes through the strainer. The fuel gas flow is controlled by the block valves and electronic control valves before passing to the distribution manifolds and combustion systems.  The position of these valves is servo-controlled by electrical signals from the TCS position feedback signals

A flow meter measures fuel gas consumed by the gas turbine. The control valves are activated by the turbine control system to provide the amount and distribution of fuel required by the turbine for a given load or speed.  The block valves shut off fuel flow to the turbine when necessary.

Packaged Electrical and Electronic Control Center (PEECC)

A control module for each gas turbine is provided to minimize field installation. The control module is designed to accommodate the turbine control system, motor control center (MCC), lighting/distribution transformer, misc. electrical panels, battery system, and the Generator Control System.  The module is supplied with an HVAC system and ceiling mounted fluorescent lighting fixtures.

- 480V MCC

A 480V motor control center, located in the PEECC, serves the gas turbine generator. This includes the 480 VAC and 120 VAC and 120 VDC distribution panels.

- Batteries System and UPS

125 VDC batteries with two chargers, for reliability.  The battery charger maintains the station batteries in a fully charged condition. The uninterruptable Power Supply (UPS) provides power for plant control system backup and protection.

- Turbine Control System

GE Mark VI Speedtronic Turbine Control System (TCS) that provides operating and controls sequencing for the safe operation and control of the package.  The TCS is located inside the Packaged Electrical and Electronic Control Center (PEECC) and is rated for an indoor, non-hazardous environment.

Starting of the gas turbine is accomplished using a closed loop process of temperature and/or speed control for an electrically driven torque converter system for consistent and reliable starts.  Bumpless transitions between start, temperature, and speed PID’s minimizes wear and reduces maintenance requirements of the package. Temperature and speed rate control during startup allow the turbine to warm up to the manufacturer’s specifications.

Generator output controls offer multiple modes of operation. User selectable modes allow for operation of gas turbine on isolated grids.  Manual, Megawatt, and Frequency modes are easily selected through the appropriate screens of the Human Machine Interface (HMI).

Balance of Plant Equipment

Mechanical Systems

- Lube Oil Demister

R.K. Chase mist eliminator system.

- Fuel Gas Heater

500 kW Watlow heater capable of increasing the temperature of the gas 50°F to meet the superheat requirement.

- Fuel Gas Scrubber

National Filtration System vertical dry scrubber knock out drum that utilizes centrifugal action to achieve last stage removal of solids and entrained liquids. The capacity of the scrubber is 22,100 scfm.

- Cooling Water Module

The cooling water system provides the cooling requirements for the lubricating oil, turbine support legs and flame detectors.  The major equipment includes an expansion tank, an air cooled heat exchanger and two circulating pumps.  The system utilizes a coolant consisting of a solution of 50% ethylene glycol in demineralized water.

Bailiff Enterprise’s 178 gallon expansion tank is open to the atmosphere to           allow for coolant expansion due to increases in ambient temperature.

Ecodyne forced draft air heat exchanger designed to supply coolant at a temperature not to exceed 125°F.

Two 75 hp Goulds Pumps, 100% capacity, rated at 967 gpm.

- Carbon Dioxide Fire Extinguishing System

The carbon dioxide (CO2) fire protection system supplied by Chemtron for GT fire protection is designed to reduce to an acceptable level the risk of a fire developing within the gas turbine that could result in damage to the plant and/or possible loss of life.

The system consists of the following major components that are located both on base and off base:

- CO2 tank system

- Discharge pipes and nozzles

- Pilot cylinder and solenoid valve

- Isolating valves and limit switches

- Fire (heat) detectors

- Pressure switches

CO2 is supplied to a distribution system that conducts the extinguishant through pipes to discharge nozzles located in the various compartments of the gas turbine.

The solenoid valve that opens the CO2 tank and initiates the discharge is located on the skid. This solenoid valve is automatically actuated by the fire panel when it receives an electrical signal from the heat-sensitive fire detectors that are strategically located in the various compartments of the unit. The system may also be actuated manually in the event of an electrical power failure by means of a lever at the top of each CO2 tank. Actuation of the system, either electrically or manually, will trip the gas turbine.

Within a few seconds after actuation, sufficient CO2 flows from the initial is charge system into the compartment of the machine to rapidly build up to an extinguishing concentration. This concentration is maintained for a prolonged period of time by the gradual addition of more CO2.

Exhaust Stack

92’ Braden simple cycle exhaust stack with silencer panels and emissions monitoring

Electrical Systems

The electrical system is comprised of the generator and associated equipment necessary to supply power to its auxiliary electrical equipment and systems as well as deliver power to the switchyard.

Each of the generators generates electrical power at 13.8 kV, 3-phase, 60 Hz. The output of each will be connected to a generation bus via its own 15 kV class, vacuum type, metal clad, and circuit breaker

Station service power will be supplied from the utility system by backfeed during plant start-up, shutdown, and maintenance periods.  The station service power will be supplied at 480V, 3-phase from one of the two station service busses which are fed by the two station service transformers.

Each generator will be synchronized to the utility system by closing its respective 15 kV circuit breaker.

- Power Distribution Center (PDC)

The PDC contains various breakers and control equipment including: motor starters, motor management relay, and switchgear breakers for the MCCs located in the PEECC. Other equipment such as the heat trace panel, BOP MCC, UPS, and 125VDC power distribution are also in the PDC.

- Auxiliary Transformers

Two low voltage distribution transformers manufactured by Virginia Transformer, suitable for outdoor service, one rated at 13.8 kV/480V, 2341 KVA and one rated at 13.8 kV/4160V, 3000 KVA.

- Generator Step-Up Transformer

230 kV Ferranti Packard generator step-up (GSU) transformer, including appropriate protection. The GSU is rated at approximately 200 MVA. Its purpose is to step the 13.8 kV outputs of the generators up to 230 kV as required for interconnection to the utility transmission system.

- Switchyard Equipment

Includes an SF-6 breaker and air switch, CTs, PTs and appropriate relaying, protection and control equipment.

Precio:

- US $60,000,000 (precio total) Unidades En Sitio

- US $  6,000,000 (Precio por desarme, empaquetamiento  y transporte hasta el Puerto de Houston

Pago

El Pago será aceptado en la forma de una Carta de Crédito:  irrevocable, confirmado contra documentos y transferible  carta de crédito por la cantidad de US $ 66,000,000.00 proporcionada por un Banco Internacional y confirmado por un Banco en US con condiciones aceptados por Energy Parts Solutions.

Wire information for

Energy Parts Solutions

US Bank 3615 W Broadway Blvd Sedalia, MO  65301

Routing Number:  081000210

Account Number:  152307883347

SWIFT Code is:  USBKUS44IMT   (that is an 'i' and not a '1')

Validez

Esta propuesta es válida por un periodo de 30 días.

Entrega:

Estas unidades están disponibles para la venta inmediatamente.

Desmantelamiento y transporte al puerto de Puerto Cabello (Como Opción que podemos hacer, no incluido en este precio) pueden ser terminados en menos de 90 días a partir de la fecha de contracto y recibo de pago.

Nuestro precio excluye impuestos y  derecho de importación o de aduana.

Impuestos:

El contratante deberá estar exento de cualquier impuesto estatal, local, tasa aduanal, impuesto al valor agregado y cualquier otro impuesto derivados de la venta de esta planta eléctrica y de cualquier otro material o partes requeridas para el arranque y comisionamiento de esta  planta

Garantías

Energy Parts Solutions proporcionara garantía de maquinaria nueva, basada en los mismos términos de estándar del fabricante GE por todas las maquinas incluyendo el BOP proporcionado por Energy Parts Solutions.

Typical Performance Characteristic Curves