Scope of Supply

(2 ea)  RB 211’s and (2 ea) LM 2500

Simple Cycle

Venezuela

Sept 26, 2008

T 8020

The proposed “EPC” Balance of Plant Scope of Work for the installation of the above project in Venezuela is outlined.

- Civil, Foundations, and Structural

.. Site preparation assuming the site is level and soil of 1500 to 2000 psf

.. Concrete foundations for 3 ea Gas Turbines with associated ancillaries

.. Gravel on site

.. Paved roads and removal areas for turbine and generators

.. Plant fencing with gates

- Buildings

.. Gas turbine generator to house the 2ea RB 211’s with associated ancillaries

.. Modular Control Building to house the LM 2500 TCP, GCP, MCC, Batteries, and chargers.

.. Plant office/control and maintenance prefabricated insulated building

.. Liquid fuel unloading shed

.. Liquid fuel centrifuge shed

- Mechanical

.. Dual fuel provisions for gas turbines

.. Fuel gas regulators – adequate gas pressure to be furnished

.. Liquid with raw fuel storage, pumping, cleaning, with day tank

.. Instrument air compressor

.. Oily water separator

.. Small water treatment system with storage tanks for fuel treatment

.. Plant instrumentation

.. Plant pipe, valves, fittings etc.

- Electrical

.. 13.8 KV Generator Breakers, aux breaker to feed aux transformers

.. Aux transformers

.. BOP electrical including DP Panels, MCC’s, Lighting Panels, etc.

.. Area Lighting

.. Plant ground grid, cathodic protection, and lightning protection

.. Cable, conduit, and cable tray

- Instrumentation

.. Plant instrumentation to augment the GTG’s instrumentation and Plant DCS

6.0 Plant DCS System

.. PLC based DCS system to interface with the GTG’s and BOP

- Engineering

- Plant Project Management

.. Scheduling, Construction Management, O&M Manuals, Commissioning and Start UP, As Built drawings, etc.

- Transportation to Venezuelan Port

Items not included in Proposal:

- Main Generator Step Up Transformers or substation

- Fire Water System Fuel Gas Compressors

- Water Injection into gas turbines

- Import duties, taxes, VAT, etc.

- In-country Transportation to Site

- Permits including Building, Environmental, Sound, etc.