PROPOSAL

Presented To:

Derwick Associates, S.A.

for

Relay Protection

Jusepin I Substation

Prepared By

Proposal No. 710-3648

July 23, 2010

This document is privileged and contains confidential information intended for use only by

Derwick Associates.

- Introduction

Energy Parts Solutions (“EPS”) is pleased to offer this proposal to Derwick Associates, S.A. (“Derwick”) for Relay Protection Upgrades at the Jusepin I - Petroleos de Venezuela (PDVSA) substation.

- Work Scope

PDVSA has indicated that there are no existing drawings or as-built drawings for the station; therefore, a working set of drawings will be developed prior to engaging the design work. The engineering and design development phase of the project will entail the inspection and wire tracing of each of the cabinets requiring upgrades and modifications. The working drawings will then be updated to reflect the final design scheme and be updated to As-Built once the new relaying, SCADA and communications systems are installed and functionally tested.

The scope of work will include new primary and backup protection and controls for the three 115kV panels H105, H120 and H405, six TX panels, nine 13.8kV panels, bus protection, and SCADA and control systems.  All relays will be IEC 61850 compliant and manufactured by Schweitzer. The existing relay cabinets along with the wiring and terminal blocks are in good shape and are suitable for re-use. We propose to replace the swing doors of the cabinets with new custom doors housing the new protection systems. We have used this technique in the past for similar upgrades and its success has proven to be effective in keeping the cost low and achieving the necessary protection upgrades at the same time.

The metering and controls will be mapped to a new remote terminal unit utilizing DNP3.0 protocol, which can then be connected to the existing fiber ring and SCADA.  Revenue metering will be replaced with ION 8800 meters.  All control switches will be ElectroSwitch, SCADA ready with lighted nameplate for indication and breaker coil monitoring.  Protective relays will be programmed, tested and functionally tested along with the control system following installation.

Primary transformer protection will be provided by a SEL-787 Transformer Protection Relay.  The transformer backup protection will be provided by a SEL-587 differential protection.  The overcurrent, distance and breaker failure protection for the 115kV breakers will be provided by SEL-311L and SEL-421 relays. All required yard excavation, foundations and installation of conduits or trench ways will be performed by PDVSA.

  - Jusepin I

Develop a working set of drawings for the substation.

Electrical Design

- Revisions to One Line diagram

- Revisions to Three line diagram

- Schematic diagrams

- New panel layout/arrangement drawing

- Revisions to panel wiring diagrams and interconnect drawings

115kV Line Protection Relay Panels (H105) for Juespin II.

- Remove swing panel door and existing relays.

- Install new prewired swing panel door complete with relays.

- SEL 421 distance, over current, directional over current relays.

- SEL 311L line differential relays.

- ElectroSwitch 52CSR control switch

- FT-1 or FT-19R test switches for each relay.

115kV Line Protection Relay Panels (H405) for Juespin II/Wilpro Media/Wilpro Alta.

- Remove swing panel door and existing relays.

- Install new prewired swing panel door complete with relays.

- SEL 421 distance, over current, directional over current relays.

- SEL 311L line differential relays.

- ElectroSwitch 52CSR control switch

- FT-1 or FT-19R test switches for each relay.

115kV Line Protection Cabinet 101

- Remove existing outdoor cabinet

- Provide and install new free standing control cabinet to be mounted under switchgear structure

- SEL-351 115kV Overcurrent Protection

- ElectroSwitch 52CSR control relay

- ElectroSwitch LOR lockout relays

- FT-1 or FT-19R test switches for each relay and LOR

CB H120 Bus Tie Protection Relay panel

- Remove existing swing door and relays

- Install new prewired swing panel door complete with relays.

- SEL 351S over current relay

- ElectroSwitch 52CSR control switch

- FT-1 or FT-19R test switches for each relay.

115kV Bus Protection Panel Bus 1

- Remove swing panel door and GE IBC/IBCG relaying.

- New custom swing panel door.

- SEL-487B 115kV Bus Different relay

- ElectroSwitch LOR lockout relays

- FT-1 or FT-19R test switches for each relay and LOR

115kV Bus Protection Panel Bus 2

- Remove swing panel door and GE SR760 relaying.

- New custom swing panel door.

- SEL-487B 115kV Bus Different relay

- ElectroSwitch LOR lockout relays

- FT-1 or FT-19R test switches for each relay and LOR

115kV Line Protection Communication

- Remove existing multiplexer

- Install new Iniven multiplexing for SCADA and fiber systems.

115kV Revenue Metering

- Remove swing panel door and relaying.

- New custom swing panel door.

- ION 8800 meters.

- FT-1 or FT-19R test switches for each meter.

Transformer Protection Panels for TX3 & TX4 w/ LTC

- Remove swing panel door.

- Remove existing paralleling transformer control relays.

- New custom swing panel door for protection.

- SEL 787 over current and differential relays.

- SEL 587 differential backup relay

- Two 86 L/O relays

- FT-1 or FT-19R test switches for each relay.

Transformer Paralleling Panel for TX3 & TX4 LTC

- Remove swing panel door.

- Remove existing paralleling transformer control relays.

- New custom swing panel door for paralleling system

- Two Beckwith M2270 and M2001 LTC voltage controllers.

- Two LTC position indicators

- FT-1 or FT-19R test switches for each relay.

Transformer Protection Panels for TX1, TX2, TX5 and TX6

- Remove swing panel door.

- Remove existing paralleling transformer control relays.

- New custom swing panel door for protection.

- SEL 787 over current and differential relays.

- SEL 587 differential backup relay

- FT-1 or FT-19R test switches for each relay.

15kV Switchgear Protection and Control (9 circuit breakers)

- Retrofit switchgear door panels and Merlin Gerin relays and controls.

- New swing door with SEL 751A over current, synchronism check, and breaker control relay.

- FT-1 or FT-19R test switches for each relay.

- ElectroSwitch LOR lockout relays

- ElectroSwitch 52CSR control relay

- ElectroSwitch  L/R switch

- Digital Ammeter

SCADA

- Develop I/O points list.

- SEL 3354 Automated Computing Platform with Human Machine Interface.

- Interface to upgraded 115kV and 13.8kV relays and meters.

Transformer On-Line DGA for LTC TX3 and TX4

- Provide and install (2) Kelman TapTrans transformer monitor.

- Develop program and I/O points list.

Transformer On-Line DGA for TX1, TX2, TX5 and TX6

- Provide and install (4) Kelman TransFIX transformer monitor.

- Develop program and I/O points list.

Relay Settings/Programs

- Develop Relay Protective Settings.

- Develop Programs for SEL and associated relays.

- Develop Programs for Kelman units

- Develop testing and commissioning requirements for new relay protection schemes.

Testing/Commissioning

- Relay

  - Check tightness of connections

  - Functional test of each elements used in the protection scheme.

  - Verify operation of light-emitting diodes, display, and targets.

  - Check all internal logic functions used in the protection scheme.

  - Check all output contacts

  - Check operation of all active digital inputs

- Current Transformers

  - Check tightness of connections

  - Perform Insulation-resistance test of each current transformer and its secondary wiring with respect to ground at 1000 volts dc for one minute.

  - Perform a polarity test of each current transformer.

  - Perform a ratio-verification test

- Voltage Transformers

  - Check tightness of connections

  - Perform insulation-resistance tests winding-to-winding and each winding-to-ground.

  - Perform a turns-ratio test on all tap positions

- Wire check of all protection and control circuits to ensure wiring is installed in accordance with design drawings

- Functional testing of all Protection and control circuits.

- Assumptions and Clarifications

- The following assumptions and clarifications have been made in the preparation of this proposal:

- The above references proposal represents our understanding of your requirements based on our engineering review conducted during the week of July 5th, 2010. If for any reason you do not agree with the above proposed protection scheme please let us know immediately so that we can modify your proposal accordingly.

- A schedule is in production for this project and will be forwarded as soon as it is completed.

- In order to minimize impact on the schedule, evaluation of the proposal is needed as soon as possible with proper instruction given to in regards to moving forward with the engineering of this project.

- Pricing

The scope of work outlined above will be performed for the following pricing:

| Description | Price |
| --- | --- |
| Engineering and Expenses | $1,245,700 |
| Installation Labor | $571,200 |
| Testing/Commissioning | $378,900 |
| Material | $3,100,500 |
| TOTAL | $5,296,300 |

  - Payment Information

Wire information for Energy Parts Solutions:

US Bank

3615 W Broadway Blvd

Sedalia, MO 65301

Routing Number: 081000210

Account Number: 152307883347

SWIFT Code is: USBKUS44IMT (that is an 'i' and not a '1')

- Terms & Conditions

This proposal shall be valid for thirty (30) days; provided, however, the obligation to treat this proposal as confidential, and that it cannot be shared with any third party without the prior written consent of ProEnergy, shall survive.

This proposal, and any resulting contract or agreement, shall be subject to the terms and conditions set forth in the attached Supplemental Terms.

- Follow Up

Please contact the following person at ProEnergy for information regarding this proposal:

Joaquin Mavares, Director of International Sales 	or 	Omar Petit, Regional Sales Manager

jmavares@proenergyservices.com  				opetit@proenergyservices.com

Office:  660-829-5100						Office:	660-829-5100

Cell: 	 713-992-1790						Cell:	660-281-8588

- Conclusion

Why select ProEnergy Services?

ProEnergy Services is the right teaming partner for Derwick! ProEnergy has talent, depth of experience and resources unparalleled in the power generation industry.  When you succeed, we succeed.  ProEnergy Services will win your confidence and your business one job at a time, starting now!

Attachment A

Supplemental Terms

These Supplemental Terms complement and are included as part of Energy Parts Solution’s Proposal No. 710-3648 dated July 23, 2010 to Derwick for the Jusepin I Substation Relay Protection  and would be included in any resulting Contract:

1.	Terms obligating ProEnergy to accept pre-existing site conditions and drawing specifications shall only apply in the event ProEnergy has actually been to the sight or inspected the drawings prior to commencement of the work.

2.	For invoice payments not received by ProEnergy within 30 days from the date of receipt, a late fee of the lesser of 1 ½ % per month or the highest rate allow by applicable law may be assessed.  If Client fails to timely make payment ProEnergy may also suspend or terminate performance of any and all of its work.

3.	No retainage will apply in the event ProEnergy is required to post a performance bond.  In no event shall retainage exceed 10% of each invoiced amount.

4. 	Any prohibition on placing a lien on the project by ProEnergy shall be subject to Client fulfilling its payment obligations under the Contract.

5. 	The parties shall indemnify, defend and hold one other harmless from and against any and all liabilities, claims, demands, suits, losses, damages, costs and expenses (including reasonable attorney fees and court costs) for bodily injury to or death of any third person, or damage to or destruction of any property of third party, caused by any negligent act or omission on the part of the indemnifying party its officers, employees, contractors or agents, except to the extent such liabilities, claims, suits, losses, damages, costs and expenses result from any negligent or willful act or omission on the part of the indemnified party, its officers, employees, contractors or agents.

6. 	ProEnergy’s obligation to indemnify and protect Client against infringement of third party intellectual property rights is subject to: (i) ProEnergy’s right to settle or defend such claim or seek the right of continued use or modify or replace the infringing work, (ii) only work which is otherwise not provided according to Client’s design or instructions, (iii) the work being used by Client for its intended use under the Contract, and (iv) any work not manufactured or developed directly by ProEnergy will be limited only to the indemnity, if any, of the manufacturer or vendor of said work.

7.	ProEnergy shall not be responsible or liable for delays in performance of its obligations under the Contract due to any event of force majeure or any other cause beyond its reasonable control.

8.	ProEnergy warrants that its work shall be performed in a competent, diligent and workmanlike manner, of good quality and material, and in compliance with any mutually agreed written instructions or specifications.  ProEnergy’s work shall be warranted for a period of one (1) year from the date of completing the work.  Any repairs or replacements made to ProEnergy’s work during the warranty period shall be warranted for the remainder of the original warranty term or 90 days, whichever is longer.  This provision sets forth the exclusive remedies for all claims based on failure of or defect in the ProEnergy’s work provided under the Contract whether the failure arises before, during or after the warranty period and whether said claim is based on contract, indemnity, warranty, tort (including negligence), strict liability or otherwise.  NO IMPLIED, STATUTORY, OR COMMON LAW WARRANTY OF ANY KIND, INCLUDING MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE, SHALL APPLY.    The duties, liabilities and obligations of ProEnergy do not extend to any repairs, adjustments, alterations, replacements or maintenance that may be required as a result of normal wear and tear, normal degradation in the performance of equipment, or as a result of (a) improper repair or alteration by Client or other persons, and (b) misuse, negligence or damage by Client or other persons (c) excessive operation at peak capacity, frequent starting, type of fuel, detrimental air inlet conditions, or erosion, corrosion or material deposit of fluids.  The warranty and remedies are further conditioned upon (i) the proper storage, installation, operation and maintenance of the equipment and conformance with the operation and instruction manuals provided by the suppliers and manufacturers and (ii) repair or modification pursuant to the instructions of the suppliers and manufacturers and as otherwise directed by ProEnergy.

9.	Care, custody, control and risk of loss for the work of ProEnergy shall pass to Client upon the earlier of when the work is completed or when it is taken over and used by Client.

10.	The total liability of ProEnergy for all claims of any kind, whether based on contract, warranty, tort (including negligence), indemnity, strict liability or otherwise, for any loss or damage arising out of, connected with, or resulting from the Contract or its work shall in no case exceed the total contract price for the work giving rise to such claim plus any insurance proceeds recovered under the coverages furnished by ProEnergy under the Contract. Notwithstanding anything in the Contract or at law to the contrary, ProEnergy shall in no event be liable for exemplary, special, incidental, indirect or consequential damages of any kind including, but not limited to, loss of use, profits or revenue.  ProEnergy shall have no liability for its competent performance of instructions given by Client or its personnel or representatives in the event such instructions prove to be defective.

11.	ProEnergy will be given at  least 10 days advance written notice and an opportunity to cure before Client may terminate the Contract for a breach of any material term of the Contract by ProEnergy.

12.	In the event ProEnergy agrees to the payment of liquidated damages (LDs) for unexcused shortfalls in any guaranteed performance or delays in any guaranteed completion date(s) then (i) the payment of LDs shall be Client’s exclusive remedy (ii) the total amount of LDs shall not exceed 10% of the total contract price unless otherwise agreed, and (iii) a corresponding bonus shall be paid by Client to ProEnergy in the event of better than guaranteed performance or early completion by ProEnergy.

13. 	Any dispute which cannot be settled amicably between the parties under the Contract will be submitted to binding and final arbitration under the Rules of the American Arbitration Association and such proceeding will be held in a mutually agreeable location.

14.	ProEnergy is not responsible for furnishing any performance bonds and builder’s risk or professional liability insurance unless specifically included in its proposal and proposal price.