Gas Turbine (GG)

Manufacturer: Rolls Royce

Model: RB211-24C

N1 Speed: 6555 RPM

Fuel: Duel Fuel currently operating on natural gas

Power Turbine (PT)

Manufacturer: Cooper Bessemer

Model: RT62 PT two stage impulse reaction

Rated HP: 27,749 (ISO)

Rated Speed: 4800

Maximum Continuous Speed: 5050 RPM

Serial Numbers: 602RT and 654RT

Over speeds Trip: 5450

Turbine Inlet Pressure: 43.2 psia @ 40F ambient

Turbine Inlet Temp: 1320F @ 40F ambient

Turbine Exhaust Temp: 861F @40F ambient

Lubrication: Mineral Oil 150 SSU

Gear Reducer

Manufacturer: Lufkin

Model Number: F33070

Rating: 35,960 Hp

Service Factor: 1.35

Input Speed: 4800 Rpm

Output Speed: 1800 Rpm

Gear Ratio: 2.660:1

AC Generator

Manufacturer: Brush Electrical Machines, LTD

Model: BSGMD 131-290-4

Type: 4 Pole

RPM: 1800

Volts: 12500

Amps: 1454

Phase: 3

HZ: 60

Power Factor: 0.85

Air Filter System

GT Intake: American Air Filter

AC Gen Air Vent Filter: American Air Filter

Main and Auxiliary Lube Oil Pumps and AC Motors

Pump Manufacturer: Transamerica DeLaval

Pump Rating: 310 GPM@ 1750 RPM and 77 PSI

Motor Manufacturer: Westinghouse

Motor Rating: 30 HP, 1750 RPM, 3 Phases, 60 HZ, 460 V

Emergency Lube Oil Pump and DC Motor

Pump Manufacturer: Transamerica DeLaval

Pump Rating: 257 GPM @1750 RPM and 77 PSI

Motor Manufacturer: Reliance

Motor Rating: 20HP, 1800 RPM, 125VDC

Water Forwarding Pumps

Pump Manufacturer: Transamerica DeLaval

Pump Rating: 5GPM @3500 RPM and 400 PSI

Motor Manufacturer: Westinghouse

Motor Rating: 3 HP, 3500 RPM, 3 Phase 60 HZ , 460V

Lube Oil Cooler

Cooler Manufacturer: Air-X-Changer

Cooler Rating: RT Sec 1,850,000 BTU/HR @234 GPM

Motors Manufacturer: Westinghouse

Motors Rating: 10HP, 1800 RPM, 3 Phase, 60 HZ, 460 V

Type: Fin Fan Radiator

Lube Oil Filter

Manufacturer: Hilco

Model: 52718-0150-2006

Type: Duplex

Capacities in Approximate U.S. Gallons

PT Lube Oil Reservoir: 2500

GG Lube Oil Reservoir: 88

GG Lube Oil Accumulator: 1.5

GG Cleaner Tank: 45

Coupling-Power Turbine to Gear

Manufacturer: Zurn Industries

Model: Ameriflex Series HPS

Type: Flex

480 Volt MCC

Manufacturer: Westinghouse Five Star MCC

Battery Chargers

Manufacturer: Sab Nife Corporation